3GPP TSG-WG SA2 Meeting #160AHE	S2-2400407r08

Online, Jan 22 – 29, 2024

Source:	Huawei, HiSilicon, LG Electronics

Title:	Terms, Architecture Requirements and Assumptions of Multi-hop U2N/U2U Relay for 5G_ProSe_Ph3

Document for:	Approval

Agenda Item:	19.7

Work Item / Release:	FS_5G_ProSe_Ph3 / Rel-19

Abstract: Terms, architecture requirements and assumptions of multi-hop U2N/U2U Relay support is proposed in this contribution.

# 1. Introduction

In the agreed WT#1 of SP-231798, as shown below, the objective is to study how to support multi-hop U2N/U2U Relay.

WT#1: Enhance ProSe to support multi-hop over NR PC5 reference point

WT-1.1: for UE-to-Network Relay

WT-1.2: for UE-UE Relay

NOTE 1:	The UE-to-Network Relay include both Layer-3 and Layer-2 Relays. UE-to-UE Relay include L3 Relay only.

NOTE 2:  This study requires coordination with RAN WGs.

This paper intends to provide architecture requirements and assumptions of multi-hop U2N/U2U Relay support.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-03.

# * * * * First change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 23.304 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and TS 23.304 [4].

5G ProSe Intermediate Relay: A 5G ProSe-enabled UE that provides functionality to support connectivity to the network for 5G ProSe Remote UE(s) by using the PC5 reference point with other 5G ProSe-enabled UEs,. The 5G ProSe Intermediate  Relay is located on the path between 5G ProSe Remote UE and 5G ProSe UE-to-Network Relay.

NOTE 1:	This term is defined for use during the study phase, so whether to define this term for normative work will be decided when concluding the study. There is no restriction that such 5G ProSe Intermediate Relay entity/function must be used for solutions.

Editor’s note: The definition of 5G ProSe Intermediate Relay may need to be further revised if this term is deemed necessary according to solution discussion. Whether and how to define intermediate relay for multi-hop UE-to-UE relay is FFS.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], TS 23.304 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and TS 23.304 [4].

# * * * * End of changes * * * *