SA WG2 Meeting #S2-160AHE	S2-2400455r03

22 - 29 January, 2024, Electronic

Title:	[Draft] Reply LS on RedCap UE MBS Broadcast reception

Response to:	Reply LS (S2-2400054/ R2-2313689 & S2-2400061/ R3-237959) on RedCap UE MBS Broadcast reception

Release:	Rel-18

Work Item:	TEI18, 5MBS_Ph2

Source:	SA2

To:	RAN2, RAN3

Cc:	CT3, CT4

Contact Person:

Name:                Thomas Belling

E-mail Address:   Thomas (dot) belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.247 CR 0341

1. Overall Description:

SA2 thanks RAN2 for the feedback. RAN2 has replied to SA2´s question 1:

An indication that an MBS broadcast session is intended to be received by both non-RedCap UE and RedCap UE may assist the gNB to decide when to transmit the session on both default and RedCap CFR and avoid waste of resources when this is not needed.

SA2 would like to inform that it agreed the attached CR against TS 23.247 to reflect that reply.

SA2 understands that for an MBS session intended for both RedCap and non-RedCap UEs separate radio resources may be allocated by RAN nodes for RedCap and non-RedCap UEs.

It is, however, unclear whether a single MBS Frequency Selection Area (FSA) ID can apply to RedCap UEs and non-RedCap UEs in the same MBS session. If it is not the case, whether it is feasible to adopt different MBS FSA IDs for RedCap and non-RedCap UEs.

SA2 would thus like to ask RAN2 to answer the following related questions:

Q1: SA2 would like to ask RAN2 to confirm the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session .

Q2: If the answer to Q1 is no, could RedCap UEs and non-RedCap UEs in the same MBS session use separate MBS FSA ID(s)?

SA2 would thus like to ask RAN3 to answer the following related question:

Q3: If the answer to Q1 is no, and RedCap UEs and non-RedCap UEs in the same MBS session use separate MBS FSA ID(s), is there a need for CN to indicate to NG-RAN which FSA ID is aimed for RedCap UEs and which for non-RedCap UEs?

2. Actions:

To RAN2, RAN3:

ACTION: SA2 asks RAN2 and RAN3 to kindly answer the above questions.

3. Date of Next SA2 Meetings:

TSG-SA2 Meeting #161			26 Feb - 01 Mar 2024				Athens, GR

TSG-SA2 Meeting #162			15 - 19 Apr 2024					China (TBC), CN