3GPP TSG-SA2 Meeting #160AHE 	S2-2400457r02

# Electronic Meeting

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1029 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Federated Learning between NWDAFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 12-01-2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Delete Nnwdaf_MLModelTraining_Subscribe Response service operation in step 9 since such response service operation does not exist. - Other minor changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete Nnwdaf_MLModelTraining_Subscribe Response service operation in step 9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Correction for alignment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 6.2C.2.1	Registration and Discovery procedure for Federated Learning

Figure 6.2C.2.1-1: Registration and Discovery procedure for Federated Learning

Steps 1 to 3 are the NWDAF registration procedure.

1-3.	NWDAF containing MTLF as FL Server NWDAF or FL Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability type information (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

Steps 4 to 6 are the NWDAF Discovery procedure.

4-6.	NWDAF containing MTLF determines ML model requires FL based on operator policy (e.g. pre-configured list of ML models), Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as FL Server NWDAF, the MTLF first discovers and selects FL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in clause 6.2A.2, FL capability Type (i.e. FL server), Time Period of Interest, Service Area.

Once the FL Server NWDAF (the requested or the selected one) is determined, the FL Server NWDAF discovers and selects other NWDAF(s) containing MTLF as FL Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, NF type(s) of data sources from which the FL Client NWDAF is able to collect data for local ML model training, Time Period of Interest, ML Model Interoperability Indicator.

7.	FL Server NWDAF sends Federated Learning preparation request to the FL Client NWDAF(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, to check if the FL Client NWDAF(s) can meet the ML model training requirement (e.g. Analytics ID, ML Model Interoperability information, Available data requirement, Availability time requirement (time span needed for the FL process), etc.). Available data requirement includes a list of Event IDs of the local data for training, and may also include the dataset statistical properties, the time window of the data samples and the minimum number of data samples.

NOTE:	Federated Learning preparation procedure (i.e. steps 7-9) can be skipped if the FL Server NWDAF can decide that the FL Client NWDAF(s) supports the FL procedure to be performed, e.g. based on information acquired from previous FL procedures or from the NRF, or based on local configuration.

8.	FL Client NWDAF(s) checks if it can meet the ML model training requirement and/or can successfully download the model if the model information is provided in the request and decides whether to join the Federated Learning process based on operator policy (e.g. pre-configured list of ML models) and/or implementation. Example criteria used by FL Client NWDAF(s) may be based on its data availability and time availability, computation and communication capability and ML Model Interoperability information.

9.	FL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the FL Server NWDAF whether it will join the FL procedure and may include the reason in the response message if it cannot join the FL process.

10.	FL Server NWDAF determines the FL Client NWDAF(s) to be involved in the FL procedures based on the information received in step 6 and other information received in step 9 (if available).

* * * * End of Change * * * *