3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400543r05

Electronic Meeting, 22-29 January 2024

# Title:	[Draft] Reply LS on network initiated IMS Data Channel

# Response to:	LS on Network Initiated IMS Data Channel (UPG08_126/ S2-2400034)

# Release:	Release 18

# Work Item:	NG_RTC

Source:	SA2

To:	GSMA NG UPG

Cc:	CT1

Contact Person:

Name:	Kefeng Zhang

Tel. Number:

E-mail Address:	kefezhan AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank GSMA NG UPG for the LS on Network Initiated IMS Data Channel (UPG08_126).

SA2 would like to provide the following feedback:

The scenario were user A makes an IMS call to user B (e.g., in an enterprise) and user B wants to add a data channel to this call can be supported in 3GPP Rel-18.

The scenario were the data channel is added through on request of a data channel server is not supported in Rel-18 and SA2 is not considering to introduce such a feature in Rel-18 since the release is frozen.

SA2 is currently working on FS_NG_RTC_Ph2 in Release 19. This study item contains the Key Issue #2 on “The impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session” (see TR 23.700-77). Key Issue #2 studies following aspects which are related to the Scenario 1:

-	Study enhancements to IMS architecture, interfaces and procedures to expose IMS services in the following IMS data channel related scenarios:

-	IMS data channel initiation, update and release. This includes the initiation, update and release of application and bootstrap data channels.

-	Study whether and how an application server can request to use the existing bootstrap data channel for downloading a specific application.

2. Actions:

To GSMA NG UPG:

ACTION: 	SA2 kindly asks GSMA NG UPG to take the above feedback into account and appreciates the feedback from GSMA NG UPG regarding the above mentioned three scenarios..

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #161		26 February-1 March 2024	Athens, Greece

TSG-SA2 Meeting #162		15-19 April 2024	China (TBD)