3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400600

Online, Jan 22 – 29, 2024	(revision of S2-231xxxx)

Source:	China Mobile

Title:	New Sol for WT2: Solution for PDU Set information Identification for end-to-end encrypted traffic

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM Ph2 /Rel-19

Abstract of the contribution: This paper proposes a solution for PDU Set information Identification for end-to-end encrypted traffic in FS_XRM_Ph_2.

1	Discussion

This solution addresses the Key issue #2 to Support PDU Set information identification for end-to-end encrypted XRM traffic.

- If and how the 5GS performs PDU Set information Identification in an end-to-end encryption scenario

QUIC is a UDP-based, stream-multiplexing, encrypted transport protocol that is widely accepted. However, QUIC natively integrates TLS encryption technology, making it difficult for the network intermediate node to identify the application layer information such as PDU Set information.

Media over QUIC (MoQ) develops a simple low-latency media delivery solution for ingest and distribution of media. MoQ focuses on building protocol mechanisms for publication of media and means to identify and receive the media, which can be used over raw QUIC or WebTransport. MoQ specifies a simple method for clients to authenticate to the relay or server to transmit or receive media. The metadata is never encrypted and is always visible to relays. So, the PDU-set related information could be identified in the relay (i.e., the UPF).

This paper proposes a solution for identifying PDU Set information from encrypted traffic based on the Media over QUIC (MoQ). In this solution, UPF acting as a relay node could identify the PDU Set information and place it in MoQ metadata/GTP header.

The solution is based on the existing PDU Session Establishment/ modification with the following enhancement:

- AF: Provide encrypted traffic handling assistance information along with the Protocol description.

- PCF: Determining PCC Rules based on encrypted traffic handling assistance information and the Protocol description.

- SMF: Provide N4 rule based on received encrypted traffic handling assistance information the Protocol description.

- UPF: Identifier the PDU set information based on encrypted traffic handling assistance information and Protocol description.

2	Proposal

It is proposed to adopt the following changes into TR 23.700-70.

* * * * 1st Change (revision marked) * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #2 |  |  |
| #X PDU Set information identification based on MoQ |  | X |

* * * *2st Change (all the text new) * * * *

## 6.X	Solution #X: < PDU Set information identification based on MoQ>

### 6.X.1	Key Issue mapping

This solution addresses the Key issue #2 to Support PDU Set information identification for end-to-end encrypted XRM traffic.

### 6.X.2	Description

QUIC is a UDP-based, stream-multiplexing, encrypted transport protocol that is widely accepted. However, QUIC natively integrates TLS encryption technology, making it difficult for the network intermediate node to identify the application layer information such as PDU Set information.

Media over QUIC (MoQ) develops a simple low-latency media delivery solution for ingest and distribution of media. MoQ focuses on building protocol mechanisms for publication of media and means to identify and receive the media, which can be used over raw QUIC or WebTransport. MoQ specifies a simple method for clients to authenticate to the relay or server to transmit or receive media.

The basic data element of MOQT is an object. An object is an addressable unit whose payload is a sequence of bytes. Objects are comprised of two parts: metadata and a payload. The format of the OBJECT message is as follows:

-	Track ID: The track identifier obtained as part of subscription and/or publish control message exchanges.

-	Group Sequence : The object is a member of the indicated group within the track.

-	Object Sequence: The order of the object within the group. The sequence starts at 0, increasing sequentially for each object within the group.

-	Object Send Order: An integer indicating the object send order or priority value.

-	Object Payload Length: The length of the following Object Payload. If this field is absent, the object payload continues to the end of the stream.

-	Object Payload: An opaque payload intended for the consumer and SHOULD NOT be processed by a relay.

The metadata is never encrypted and is always visible to relays. So, the PDU-set related information could be identified in the relay (i.e., the UPF).

This paper proposes a solution for identifying PDU Set information from encrypted traffic based on the MoQ. In this solution, UPF acting as a relay node could identify the PDU Set information and place it in MoQ metadata/GTP header.

The solution is based on the existing PDU Session Establishment/ modification with the following enhancement:

AF: Provide Encrypted Traffic Handling Assistance Information along with the Protocol description.

PCF: Determine PCC Rules based on Encrypted Traffic Handling Assistance Information and the Protocol description.

SMF: Provide N4 rule to activate the MoQ relay functionality of UPF.

UPF: Identify the PDU set information based on MoQ metadata. Maintain two QUIC connections(i.e., one between the UE and UPF, another one between the UPF and the AS) for one QoS flow..

UE : Support the QUIC/MoQ connection establishment with UPF.

Editor’s NOTE: it is FFS how the PDU set information(e.g. PDU Sequence Number within a PDU Set) can be supported by MoQ .

### 6.X.3	Procedures

The procedures is used for AF requested encrypted traffic handling, and the QUIC connection may or may not be established between the UE and the AF. The network could help to identify the PDU set information based on the AF provided assistance information.

Figure 6.X.3-1: PDU Set information identification based on MoQ

- A PDU session has been established between the UE and UPF. A QUIC connection has been established            between the UE and AS.

- The AF/AS may provide Encrypted Traffic Handling Assistance Information (ETHAI) along with the Protocol Description to the NEF/PCF, the ETHAI includes the MoQ relay support indication along with one or more URI of AS, UE GPSI. Those information can be provided via AF triggered traffic influence procedure.

- The PCF generates PCC rules based on the AF provided ETHAI and Protocol Description, and sends the PCC rules to the SMF.

- If the PCC rules indicates the MoQ relay support, the SMF generates the N4 rules containing the Protocol Description information and ETHAI. The SMF sends the N4 rules to the UPF and requests the UPF to activate the MoQ relay functionality.

- Then the UPF triggers the connection(s) establishment with the UE identified by the UE IP address. And the UPF triggers the connection establishment with the AS identified by the URI. Alternatively, the UPF may trigger the UE and AF to initiate the MoQ connection establishment, by sending the UPF’s URI with a "moq" scheme.

- Upon successfully established the connections, the UPF sends the N4 modification response to SMF.

- The SMF sends the response of the MoQ relay support to AF via PCF/NEF.

- For the downlink direction, the PSA UPF identifies PDU Set information from MoQ metadata, and sends the PDU Set information in the GTP-U header to RAN.The PDU Set Information can be extracted from the metadata as following,

-	The PDU Set Sequence Number could be extracted from Object Sequence.

-	Indication of End PDU of the PDU Set could be identified based on the Payload Length .

-	PDU Set Size in bytes could be identified based on Object Payload Length .

-	PDU Set Importance could be identified based on Object Object Send Order .

### 6.X.4	Impacts on services, entities and interfaces

AF

- 	Provide Encrypted Traffic Handling Assistance Information along with the Protocol description.

PCF

- 	Determine PCC Rules based on Encrypted Traffic Handling Assistance Information and the Protocol Description.

SMF

- 	Provide N4 rule to activate the MoQ relay functionality of UPF.

UPF

- 	Identify the PDU set information based on MoQ metadata.

-	Maintain two QUIC connections(i.e., one between the UE and UPF, another one between the UPF and the AS) for one QoS flow.

UE

- 	Support the QUIC/MoQ connection establishment with UPF.

* * * * End of Changes * * * *