3GPP SA WG2 Meeting #160-Ad Hoc-e		S2-24100733

E-meeting, January 22 – 29, 2024	(revision of S2-241xxxx)

# Source:	LG Electronics

# Title:	New WID: URSP rules with extended time window (TEI19_URSP_eTW)

# Document for:	Endorsement

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	URSP rules with extended time window

Acronym:	TEI19_URSP_eTW

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 740061 | Stage 2 of 5G System - Phase 1 | Rel-15 SA2 5GS_Ph1 |

3	Justification

CT WG1 had sent the LS(S2-2311956/C1-237982) to check with SA2 whether there is any issue regarding the improvement on URSP signalling for recurrent events and extend a route selection descriptor introducing multiple time windows and optionally a recurrence for each (hereafter called extended time window).

The URSP rule may include one or more RSD(s) each with a Time Window defining the validity period for the corresponding RSD. Each Time Window is comprised with a sequence of start and end time value to indicate when the matching traffic is allowed for the UE.

With the current design, the PCF may provision and update URSP rule to the UE in order to support periodic services. However, there may be the case for periodic services having certain recurrence events that the PCF might need frequent update on the URSP rule which would lead to inefficient signalling and UE power consumption.

Therefore, it is proposed to extend the RSD for multiple time windows and each with the recurrence pattern to be included in the URSP rule for the URSP signalling improvement.

As the URSP continues to be enhanced and extended over multiple releases, the UE indicates support for the feature in the UE Policy Container (e.g. Indication of URSP Provisioning Support in EPS, UE capability of supporting VPLMN-specific URSP rules, etc).

The possibility of UE indicating the support for the extended time window in the UE Policy Container and the inclusion of multiple time windows and for each with a recurrence pattern in the URSP rule to support periodic services with recurrent events would improve URSP signalling.

Therefore, it is proposed to specify URSP rules with extended time window in order to improve URSP signalling for recurrent events.

4	Objective

The objective of this work item is to:

-	Add description for the UE capability report for URSP rules with extended time window.

-	Add description for the URSP extension with RSD to include multiple time windows and each with optional recurrence information.

This work item will require 0.5 TU(s) to discuss and agree on the corresponding CRs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 | Add description for the UE capability report for URSP rules with extended time window | SA#106 (Dec. 2024) |  |
| TS 23.503 | Add description for URSP rules with extended time window and the Indication of UE support for URSP rules with extended time window | SA#106 (Dec. 2024) |  |

6	Work item Rapporteur(s)

Hongsuk Kim, LG Electronics, hongsuk.kim@lge.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

None.

9	Supporting Individual Members

Supporting IM name

LG Electronics

LG Uplus

ETRI

DanKook University

SyncTechno Inc.

Orange

KT Corporation

Ericsson