3GPP TSG-SA WG2 Meeting #160AHE	S2-2400825r01

# 22 - 29 January, 2024, Electronic							 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0420 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to default L2 ID usage for discovery message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-1-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Destination Layer-2 ID(s) used for sending/receiving Relay Discovery Solicitation messages are missing in clause 5.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the usage of Destination Layer-2 ID(s) for sending/receiving Relay Discovery Solicitation messages are missing in clause 5.1 from U2U relay UE and remote UE respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE can not determine Destination Layer-2 ID(s) for sending/receiving Relay Discovery Solicitation messages if missing in Authorization and Provisioning part. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.1.5.1	Policy/Parameter provisioning for 5G ProSe UE-to-UE Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-UE Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay:

-	when the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 and/or Layer-2 End UEs accessing 5G ProSe UE-to-UE Relays over PC5 reference point.

-	when the UE is not "served by NG-RAN":

-	Indicates whether the UE is authorized to relay traffic for 5G ProSe Layer-3 and/or Layer-2 End UEs accessing 5G ProSe UE-to-UE Relays over PC5 reference point.

-	The authorisation for a UE to act as a 5G ProSe UE-to-UE Relay also authorizes the use of 5G ProSe UE-to-UE Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe UE-to-UE Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe UE-to-UE Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator). The UE-to-UE Relay Layer indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement message and sending/receiving Relay Discovery Solicitation messages;

-	Default Destination Layer-2 ID(s) for sending/receiving Direct Communication Request message for ProSe UE-to-UE Relay Communication with integrated Discovery;

-	For 5G ProSe Layer-3 UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

-	Includes security related content for Relay Discovery for each Relay Service Code.

NOTE 2:	SA WG3 will determine the security related content for 5G ProSe UE-to-UE Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe End UE and thereby enabling the use of a 5G ProSe UE-to-UE Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to use a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay.

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorised to use a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay.

The authorisation for a UE to act as a 5G ProSe End UE also authorizes the use of 5G ProSe UE-to-UE Relay discovery with Model A and Model B.

NOTE 3:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe UE-to-UE Relay Discovery:

-	Includes the parameters for 5G ProSe UE-to-UE Relay Discovery as a 5G ProSe End UE and for enabling the UE to connect to the 5G ProSe UE-to-UE Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator). The UE-to-UE Relay Layer indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

-	Default Destination Layer-2 ID(s) for sending/receiving Relay Discovery Solicitation messages and receiving Relay Discovery Announcement message;

-	Default Destination Layer-2 ID(s) for sending/receiving Direct Communication Request message for ProSe UE-to-UE Relay Communication with integrated Discovery;

-	For 5G ProSe Layer-3 UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

-	Includes security related content for Relay Discovery for each Relay Service Code.

NOTE 4:	SA WG3 will determine the security related content for 5G ProSe UE-to-UE Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-UE Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe End UE and thereby enabling the use of a 5G ProSe UE-to-UE Relay:

1)	Radio parameters for 5G ProSe UE-to-UE Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

NOTE 5:	RAN WG2 will determine the default PC5 DRX configuration for 5G ProSe UE-to-UE relay scenario.

NOTE 6:	Radio parameters for 5G ProSe UE-to-UE Relay Discovery when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Discovery in clause 5.1.2.1 are expected to be aligned for direct and relayed discovery for UE to UE communication.

2)	Radio parameters for 5G ProSe UE-to-UE Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 7:	Radio parameters for 5G ProSe UE-to-UE Relay communication when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Communication in clause 5.1.3.1 are expected to be aligned for direct and relayed UE to UE communication.

* * * * End of Change * * * *