3GPP SA WG2 Meeting #160-Ad Hoc-e	 S2-2400969r05

22 - 29 January, 2024, Electronic

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1257 | Rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF related corrections to Network Slice Replacement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is not clear that the PCF uses also the S-NSSAI for session binding. The SMF action when the replaced S-NSSAI is available again is not stated. The PCF actions in relation to what S-NSSAI to request for subscription information in all cases and what to do if the Alternative S-NSSAI is not a subscribed S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that S-NSSAI can be used for session binding and that the AF is not aware of the Alternative S-NSSAI in such case. Clarified the SMF action when the replaced S-NSSAI is available again. Clarified the PCF actions in relation to Network Slice replacement for a PDU Session. Add dynamic support for PCF to update PDU Session Inactivity Timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There Network Slice replacement for a PDU Session will be work differently dependent on PCF implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2.2, 6.1.3.5, 6.1.3.29, 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.1.3.2.2	Session binding

Session binding is the association of the AF session information to one and only one PDU Session. The PCF shall perform the session binding, which may take the following PDU Session parameters into account:

a)	For an IP type PDU Session, the UE IPv4 address and/or IPv6 prefix as specified in TS 23.501 [2], in addition when using W-5GAN the specification in TS 23.316 [27] applies. For IPv6 prefix delegation, the assigned IPv6 network prefix is shorter than /64;

For an Ethernet type PDU Session, the UE MAC address(es);

b)	The UE identity (e.g. SUPI), if present;

c)	The information about the Data Network (DN) the user is accessing, i.e. DNN, if present or DNN and S-NSSAI, if present.

NOTE:	The AF is not aware of the Network Slice Replacement functionality (as described in clause 5.15.19 in TS 23.501 [2]), therefore the (replaced) S-NSSAI stored as part of the Session Management context is used for session binding.

Once it has determined the impacted PDU Session, the PCF shall identify the PCC rules affected by the AF session information, including new PCC rules to be installed and existing PCC rules to be modified or removed.

Session Binding applies for PDU Sessions of IP type. It may also apply to Ethernet PDU Session type but only when especially allowed by PCC related Policy Control Request triggers.

NEXT CHANGE

6.1.3.5	Policy Control Request Triggers relevant for SMF

The Policy Control Request Triggers relevant for SMF define the conditions when the SMF shall interact again with PCF after a PDU Session establishment as defined in the Session Management Policy Establishment and Session Management Policy Modification procedure as defined in TS 23.502 [3].

The PCR triggers are not applicable any longer at termination of the SM Policy Association.

The access independent Policy Control Request Triggers relevant for SMF are listed in table 6.1.3.5-1.

The differences with table 6.2 and table A.4.3-2 in TS 23.203 [4] are shown, either "none" means that the parameter applies in 5GS or "removed" meaning that the parameter does not apply in 5GS, this is due to the lack of support in the 5GS for this feature or "modified" meaning that the parameter applies with some modifications defined in the parameter.

Table 6.1.3.5-1: Access independent Policy Control Request Triggers relevant for SMF

| Policy Control Request Trigger | Description | Difference compared with table 6.2 and table A.4.3-2 in TS 23.203 [4] | Conditions for reporting | Motivation |
| --- | --- | --- | --- | --- |
| PLMN change | The UE has moved to another operators' domain. | None | PCF |  |
| QoS change | The QoS parameters of the QoS Flow has changed. | Removed |  | Only applicable when binding of bearers was done in PCRF. |
| QoS change exceeding authorization | The QoS parameters of the QoS Flow has changed and exceeds the authorized QoS. | Removed |  | Only applicable when binding of bearers was done in PCRF. |
| Traffic mapping information change | The traffic mapping information of the QoS profile has changed. | Removed |  | Only applicable when binding of bearers was done in PCRF. |
| Resource modification request | A request for resource modification has been received by the SMF. | None | SMF always reports to PCF |  |
| Routing information change | The IP flow mobility routing information has changed (when IP flow mobility as specified in TS 23.261 [11] applies) or the PCEF has received Routing Rules from the UE (when NBIFOM as specified in TS 23.161 [10] applies). | Removed |  | Not in 5GS yet. |
| Change in Access Type (NOTE 8) | The Access Type and, if applicable, the RAT Type of the PDU Session has changed. | None | PCF |  |
| EPS Fallback | EPS fallback is initiated | Added | PCF |  |
| Loss/recovery of transmission resources | The Access type transmission resources are no longer usable/again usable. | Removed |  | Not in 5GS yet. |
| Location change (serving cell) (NOTE 6) | The serving cell of the UE has changed. | None | PCF |  |
| Location change (serving area) (NOTE 2) | The serving area of the UE has changed. | None | PCF |  |
| Location change (serving CN node) (NOTE 3) | The serving core network node of the UE has changed. | None | PCF |  |
| Change of UE presence in Presence Reporting Area (see NOTE 1) | The UE is entering/leaving a Presence Reporting Area. | None | PCF | Only applicable to PCF |
| Out of credit | Credit is no longer available. | None | PCF |  |
| Reallocation of credit | Credit has been reallocated after the former Out of credit indication. | Added | PCF |  |
| Enforced PCC rule request | SMF is performing a PCC rules request as instructed by the PCF. | None | PCF |  |
| Enforced ADC rule request | TDF is performing an ADC rules request as instructed by the PCRF. | Removed |  | ADC Rules are not applicable. |
| UE IP address change | A UE IP address has been allocated/released. | None | SMF always reports allocated or released UE IP addresses |  |
| UE MAC address change | A new UE MAC address is detected or a used UE MAC address is inactive for a specific period. | New | PCF |  |
| Access Network Charging Correlation Information | Access Network Charging Correlation Information has been assigned. | None | PCF |  |
| Usage report (NOTE 4) | The PDU Session or the Monitoring key specific resources consumed by a UE either reached the threshold or needs to be reported for other reasons. | None | PCF |  |
| Start of application traffic detection and Stop of application traffic detection (NOTE 5) | The start or the stop of application traffic has been detected. | None | PCF |  |
| SRVCC CS to PS handover | A CS to PS handover has been detected. | Removed |  | No support in 5GS yet |
| Access Network Information report | Access information as specified in the Access Network Information Reporting part of a PCC rule. | None | PCF |  |
| Credit management session failure | Transient/Permanent failure as specified by the CHF. | None | PCF |  |
| Addition / removal of an access to an IP-CAN session | The PCEF reports when an access is added or removed. | Removed |  | No support in 5GS yet |
| Change of usability of an access | The PCEF reports that an access becomes unusable or usable again. | Removed |  | No support in 5GS yet |
| 3GPP PS Data Off status change | The SMF reports when the 3GPP PS Data Off status changes. | None | SMF always reports to PCF |  |
| Session AMBR change | The Session-AMBR has changed. | Added | SMF always reports to PCF |  |
| Default QoS change | The subscribed QoS has changed. | Added | SMF always reports to PCF |  |
| Removal of PCC rule | The SMF reports when the PCC rule is removed. | Added | SMF always reports to PCF |  |
| Successful resource allocation | The SMF reports to the PCF that the resources for a PCC rule have been successfully allocated. | Added | PCF |  |
| GFBR of the QoS Flow can no longer (or can again) be guaranteed | The SMF notifies the PCF when receiving notifications from RAN that GFBR of the QoS Flow can no longer (or can again) be guaranteed. | Added |  |  |
| UE resumed from suspend state | The SMF reports to the PCF when it detects that the UE is resumed from suspend state. | None | PCF | Only applicable to EPC IWK |
| Change of DN Authorization Profile Index | The DN Authorization Profile Index received from DN-AAA has changed. | Added | SMF always reports to PCF |  |
| 5GS Bridge/Router information available | SMF has detected new 5GS Bridge/Router information, which may contain, user-plane Node ID, UE-DS-TT residence time and Ethernet port (port number and MAC address) or IP address for the PDU Session, MTU size for IPv4 or MTU size for IPv6 and/or PMIC and/or UMIC. | Added | PCF |  |
| QoS Monitoring | The SMF notifies the PCF of the QoS Monitoring reports (as defined in clause 5.45 of TS 23.501 [2]). | Added | PCF |  |
| DDN Failure event Subscription with Traffic Descriptor | The SMF requests PCF to provide or remove policies if it received an event subscription or cancellation for DDN Failure event including traffic descriptors. The SMF provides the traffic descriptors to the PCF for policy evaluation. | Added | PCF |  |
| DDD Status event Subscription with Traffic Descriptor | The SMF requests PCF to provide or remove policies if it received an event subscription or cancellation for DDD Status event including traffic descriptors. The SMF provides the traffic descriptors and the requested type(s) of notifications (notifications about downlink packets being buffered, and/or discarded) to the PCF for policy evaluation. | Added | PCF |  |
| QoS constraints change | The QoS constraints in the VPLMN have been provided or changed. | Added | SMF always reports to PCF |  |
| Satellite backhaul category change | The backhaul is changed between different satellite backhaul categories, or between satellite backhaul and non-satellite backhaul. | Added | PCF |  |
| NWDAF info change | The NWDAF instance IDs used for the PDU session or associated Analytics IDs used for the PDU session and available in the SMF have changed. | Added | PCF |  |
| Request for notification on SM Policy Association establishment or termination (NOTE 9) | The SMF reports to the PCF the request to notify on the established or terminated SM Policy Association. | Added | PCF |  |
| Notification on BAT offset | The SMF reports the BAT offset and optionally the adjusted periodicity provided by the RAN. | Added | PCF |  |
| UE reporting Connection Capabilities from associated URSP rule (NOTE 10) | The SMF has received from the UE reporting from an associated URSP rule via a PDU session establishment or PDU session modification request. | None | PCF |  |
| UE Policy Container received | The SMF reports that a UE Policy Container has been received from the UE. | Added | SMF always reports to PCF |  |
| Change of HR-SBO support indication | The HR-SBO support indication has changed. | Added |  |  |
| Network Slice Replacement | The SMF reports the event of change between S-NSSAI and Alternative S-NSSAI to PCF when the SMF determines that the PDU Session and SM Policy Association can be retained. The SMF provides Alternative S-NSSAI when the PDU Session is transferred from S-NSSAI to Alternative S-NSSAI. | Added | PCF |  |
| ECN marking for L4S can no longer (or can again) be performed | The SMF notifies the PCF when ECN marking for L4S can no longer (or can again) be performed. | Added | PCF |  |
| Result of UE Policy Container delivery via EPS | The SMF reports the result of UE policies delivery via EPS | Added | SMF always reports to PCF |  |
| NOTE 1: The maximum number of PRA(s) per UE per PDU Session is configured in the PCF. The PCF may have independent configuration of the maximum number for Core Network pre-configured PRAs and UE-dedicated PRAs. The exact number(s) should be determined by operator in deployment. NOTE 2: This trigger reports change of Tracking Area in both 5GS and EPC interworking, or reports change of Routing Area for GERAN/UTRAN access (see Annex G of TS 23.502 [3]). NOTE 3: This trigger reports change of AMF in 5GC, change between ePDG and Serving GW in EPC, change between Serving GWs in EPC, change between EPC and 5GC, change between Serving Gateway and SGSN in GERAN/UTRAN from/to E-UTRAN mobility, or change between SGSNs in the case of GERAN/UTRAN access. In HR roaming case, if the AMF change is unknown by the H-SMF, then the AMF change is not reported. NOTE 4: Usage is defined as either volume or time of user plane traffic. NOTE 5: The start and stop of application traffic detection are separate event triggers, but received under the same subscription from the PCF. NOTE 6: Location change of serving cell can increase signalling load on multiple interfaces. Hence it is recommended that any such serving cell changes only applied for a limited number of subscribers avoiding extra signalling load. It also is applicable for GERAN/UTRAN access. NOTE 7: Void. NOTE 8: For 3GPP access the RAT type may refer to NR, E-UTRAN, and, when the SMF+PGW-C enhancements to support GERAN/UTRAN access via Gn/Gp interface as specified in Annex L of TS 23.501 [2] apply, to UTRAN or GERAN. For MA PDU Session this trigger reports the current used Access Type(s) and RAT type(s) upon any change of Access Type and RAT type. NOTE 9: The PCF for the PDU Session knows the change of the PCF for the UE by this Policy Control Request Trigger based on the associated binding information of and notifies the PCF for the UE as described in clause 6.1.3.18. NOTE 10: See clause 6.6.2.4. |  |  |  |  |

NOTE 1:	In the following description of the access independent Policy Control Request Triggers relevant for SMF, the term trigger is used instead of Policy Control Request Trigger where appropriate.

When the EPS Fallback trigger is armed by the PCF, the SMF shall report the event to the PCF when a QoS Flow with 5QI=1 is rejected due to EPS Fallback.

When the Location change trigger is armed, the SMF shall subscribe to the AMF for reports on changes in location to the level indicated by the trigger. If credit-authorization triggers and Policy Control Request Triggers require different levels of reporting of location change for a single UE, the location to be reported should be changed to the highest level of detail required. However, there should be no request being triggered for PCC rules update to the PCF if the report received is more detailed than requested by the PCF.

NOTE 2:	The access network may be configured to report location changes only when transmission resources are established in the radio access network.

The Resource modification request trigger shall trigger the PCF interaction for all resource modification requests not tied to a specific QoS Flow received by SMF. The resource modification request received by SMF may include request for guaranteed bit rate changes for a traffic aggregate and/or the association/disassociation of the traffic aggregate with a 5QI and/or a modification of the traffic aggregate.

The enforced PCC rule request trigger shall trigger a SMF interaction to request PCC rules from the PCF for an established PDU Session. This SMF interaction shall take place within the Revalidation time limit set by the PCF in the PDU Session related policy information. The SMF reports that the enforced PCC rule request trigger was met and the enforced PCC Rules.

NOTE 3:	The enforced PCC rule request trigger can be used to avoid signalling overload situations e.g. due to time of day based PCC rule changes.

The UE IP address change trigger shall trigger a SMF interaction with the PCF if a UE IP address is allocated or released during the lifetime of the PDU Session. The SMF reports that the UE IP address change trigger was met and the new or released UE IP address.

The UE MAC address change trigger shall trigger a SMF interaction with the PCF if a new UE MAC address is detected or a used UE MAC address is inactive for a specific period during the lifetime of the Ethernet type PDU Session. The SMF reports that the UE MAC address change trigger was met and the new or released UE MAC address.

NOTE 4:	The SMF instructs the UPF to detect new UE MAC addresses or used UE MAC address is inactive for a specific period as described in TS 23.501 [2].

The Access Network Charging Correlation Information trigger shall trigger the SMF to report the assigned access network charging identifier for the PCC rules that are accompanied with a request for this trigger at activation. The SMF reports that the Access Network Charging Correlation Information trigger was met and the Access Network Charging Correlation Information.

If the Usage report trigger is set and the volume or the time thresholds, earlier provided by the PCF, are reached, the SMF shall report this situation to the PCF. If both volume and time thresholds were provided and the thresholds, for one of the measurements, are reached, the SMF shall report this situation to the PCF and the accumulated usage since last report shall be reported for both measurements.

The management of the Presence Reporting Area (PRA) functionality enables the PCF to subscribe to reporting change of UE presence in a particular Presence Reporting Area.

NOTE 5:	PCF decides whether to subscribe to AMF or to SMF for those triggers that are present in both tables 6.1.2.5-2 and 6.1.3.5-1. If the Change of UE presence in Presence Reporting Area trigger is available on both AMF and SMF, PCF should not subscribe to both AMF and SMF simultaneously.

Upon every interaction with the SMF, the PCF may activate / deactivate reporting changes of UE presence in Presence Reporting Area by setting / unsetting the corresponding trigger by providing the PRA Identifier(s) and additionally the list(s) of elements comprising the Presence Reporting Area for UE-dedicated Presence Reporting Area(s).

The SMF shall subscribe to the UE Location Change notification from the AMF by providing an area of interest containing the PRA Identifier(s) and additionally the list(s) of elements provided by the PCF as specified in clause 5.6.11 of TS 23.501 [2] and in clause 5.2.2.3.1 of TS 23.502 [3].

When the Change of UE presence in Presence Reporting Area trigger is armed, i.e. when the PCF subscribes to reporting change of UE presence in a particular Presence Reporting Area and the reporting change of UE presence in this Presence Reporting Area was not activated before, the SMF subscribes to the UE mobility event notification service provided by the AMF for reporting of UE presence in Area of Interest which reports when the UE enters or leaves a Presence Reporting Area (an initial report is received when the PDU Session specific procedure is activated). The SMF reports the PRA Identifier(s) and indication(s) whether the UE is inside or outside the Presence Reporting Area(s), and indication(s) if the corresponding Presence Reporting Area(s) is set to inactive by the AMF to the PCF.

NOTE 6:	The serving node (i.e. AMF in 5GC or MME in EPC/EUTRAN) can activate the reporting for the PRAs which are inactive as described in the TS 23.501 [2].

When PCF modifies the list of PRA id(s) to change of UE presence in Presence Reporting Area for a particular Presence Reporting Area(s), the SMF removes or adds the PRA id(s) provided in the UE mobility event notification service provided by AMF for reporting of UE presence in Area Of Interest. When the PCF unsubscribes to reporting change of UE presence in Presence reporting Area, the SMF unsubscribes to the UE mobility event notification service provided by AMF for reporting of UE presence in Area Of Interest, unless subscriptions to AMF remains due to other triggers.

The SMF stores PCF subscription to reporting for changes of UE presence in Presence Reporting Area and notifies the PCF with the PRA Identifier(s) and indication(s) whether the UE is inside or outside the Presence Reporting Area(s) based on UE location change notification in area of interest received from the serving node according to the corresponding subscription.

NOTE 7:	The SMF can also be triggered by the CHF to subscribe to notification of UE presence in PRA from the AMF, and notifies the CHF when receiving reporting of UE presence in PRA from the AMF, referring to TS 32.291 [20].

If PCF is configured with a PRA identifier referring to the list of PRA Identifier(s) within a Set of Core Network predefined Presence Reporting Areas as defined in TS 23.501 [2], it activates the reporting of UE entering/leaving each individual PRA in the Set of Core Network predefined Presence Reporting Areas, without providing the complete set of individual PRAs.

When a PRA set identified by a PRA Identifier was subscribed to report changes of UE presence in Presence Reporting Area by the PCF, the SMF additionally receives the PRA Identifier of the PRA set from the AMF, along with the individual PRA Identifier(s) belonging to the PRA set and indication(s) of whether the UE is inside or outside the individual Presence Reporting Area(s), as described in TS 23.501 [2].

When the Out of credit detection trigger is set, the SMF shall inform the PCF about the PCC rules for which credit is no longer available together with the applied termination action.

When the Reallocation of credit detection trigger is set, the SMF shall inform the PCF about the PCC rules for which credit has been reallocated after credit was no longer available and the termination action was applied.

The Start of application traffic detection and Stop of application traffic detection triggers shall trigger an interaction with PCF once the requested application traffic is detected (i.e. Start of application traffic detection) or the end of the requested application traffic is detected (i.e. Stop of application traffic detection) unless it is requested within a specific PCC Rule to mute such interaction for solicited application reporting or unconditionally in the case of unsolicited application reporting. The application identifier and service data flow descriptions, if deducible, shall also be included in the report. An application instance identifier shall be included in the report both for Start and for Stop of application traffic detection when service data flow descriptions are deducible. This is done to unambiguously match the Start and the Stop events.

At PCC rule activation, modification and deactivation the SMF shall send, as specified in the PCC rule, the User Location Report and/or UE Timezone Report to the PCF.

NOTE 8:	At PCC rule deactivation the User Location Report includes information on when the UE was last known to be in that location.

If the trigger for Access Network Information reporting is set, the SMF shall check the need for access network information reporting after successful installation/modification or removal of a PCC rule or upon termination of the PDU Session. The SMF shall check the Access Network Information report parameters (User Location Report, UE Timezone Report) of the PCC rules and report the access network information to the PCF. The SMF shall not report any subsequent access network information updates received from the PDU Session without any previous updates of related PCC rule unless the associated QoS Flow or PDU Session has been released.

If the SMF receives a request to install/modify or remove a PCC rule with Access Network Information report parameters (User Location Report, UE Timezone Report) set the SMF shall initiate a PDU Session modification to retrieve the current access network information of the UE and forward it to the PCF afterwards.

If the Access Network Information report parameter for the User Location Report is set and the user location (e.g. cell) is not available to the SMF, the SMF shall provide the serving PLMN identifier to the PCF.

The Credit management session failure trigger shall trigger a SMF interaction with the PCF to inform about a credit management session failure and to indicate the failure reason, and the affected PCC rules.

NOTE 9:	As a result, the PCF may decide about e.g. PDU Session termination, perform gating of services, switch to offline charging, change rating group, etc.

NOTE 10:	The Credit management session failure trigger applies to situations wherein the PDU Session is not terminated by the SMF due to the credit management session failure.

The default QoS change triggers shall trigger the PCF interaction for all changes in the default QoS data received in SMF from the UDM.

The Session AMBR change trigger shall trigger the SMF to provide the Session-AMBR to the PCF containing the DN authorised Session AMBR if received from the DN-AAA, or the Subscribed Session-AMBR received from the UDM as described in clause 5.6.6 of TS 23.501 [2].

The default QoS change trigger reports a change in the default 5QI/ARP retrieved by SMF from UDM, as explained in clause 5.7.2.7 of TS 23.501 [2].

If the PCC Rules bound to a QoS Flow are removed when the corresponding QoS Flow is removed or the PCC rules are failed to be enforced, the SMF shall report this situation to the PCF. The PCF may then provide the same or updated PCC rules for the established PDU Session.

If the trigger for successful resource allocation is set and the PCF has also provided an indication that a specific PCC rule is subject to this trigger, the SMF shall report to the PCF when the resources associated to this PCC rule have been successfully allocated. The SMF shall report resource allocation failure always to the PCF, independently of this trigger. If the SMF reports resource allocation failure for a PCC rule containing MA PDU Session Control information with Redundant as Steering Mode (see clause 5.32.4 of TS 23.501 [2]), the SMF shall also indicate the respective Access Type.

If the GFBR of the QoS Flow can no longer (or can again) be guaranteed trigger is armed, the SMF shall check the need for reporting to the PCF when the SMF receives an explicit notification from (R)AN indicating that GFBR of the QoS Flow can no longer (or can again) be guaranteed or when the condition described in clause 5.7.2.4 of TS 23.501 [2] is met during the handover. The SMF shall report that GFBR of the QoS Flow can no longer (or can again) be guaranteed accordingly to the PCF for those PCC rules which are bound to the affected QoS Flow and have the QoS Notification Control (QNC) parameter set. If additional information is received with the notification from NG-RAN (see clause 5.7.2.4 of TS 23.501 [2]), the SMF shall also provide to the PCF the reference to the Alternative QoS parameter set corresponding to the Alternative QoS Profile referenced by NG-RAN. If NG-RAN has indicated that the lowest priority Alternative QoS Profile cannot be fulfilled, the SMF shall indicate to the PCF that the lowest priority Alternative QoS parameter set cannot be fulfilled.

In an interworking scenario between 5GS and EPC/E-UTRAN, as explained in clause 4.3 of TS 23.501 [2], the PCF may subscribe via the SMF also to the Policy Control Request Triggers described in clause 6.1.2.5 when the UE is served by the EPC/E-UTRAN.

The change of DN Authorization Profile Index shall trigger a SMF interaction to send DN Authorization Profile Index to retrieve a list of PCC Rules (as defined in clause 6.3) and/or PDU Session related policy (as defined in clause 6.4) for an established PDU Session.

If the trigger for 5GS Bridge/Router information available is armed, the SMF shall report the 5GS Bridge/Router information when the SMF has determined or updated the 5GS Bridge/Router information, e.g. when SMF has detected an Ethernet port which supports exchange of Ethernet Port Management Information Containers or received User plane node Management Information Container or Port Management Information Container. If a new manageable Ethernet DS-TT port is detected, the SMF provides User plane node ID, the port number and optionally MAC address of the related port of the related PDU Session to the PCF. If the SMF has received UE-DS-TT Residence Time then the SMF also provides UE-DS-TT Residence Time to the PCF. If the SMF has received the User plane node Management Information Container from NW-TT or Port Management Information Container from NW-TT or DS-TT, the SMF also provides User plane node Management Information Container or Port Management Information Container and related port number to the PCF. In the case of Deterministic Networking, the SMF may also provide the MTU size for IPv4 or the MTU size for IPv6.

When the QoS Monitoring trigger is set, the SMF shall, upon receiving the QoS Monitoring report from the UPF, send the measurement report to the PCF.

If the Policy Control Request Trigger "DDN Failure event subscription with Traffic Descriptor" or "DDD Status event subscription with Traffic Descriptor" is set, the SMF shall request policies if it received a subscription or cancellation of notifications for availability after DDN Failure event with traffic descriptor or DDD Status event with traffic descriptor, respectively. The SMF indicates whether it is a subscription or cancellation event and provides the received Traffic Descriptor as well as the requested type(s) of notifications (notifications about downlink packets being buffered, and/or discarded) to the PCF. When the SMF indicates a subscription event, the PCF checks whether an installed PCC rule exists for the received Traffic Descriptor and if so, the PCF sets the Downlink Data Notification Control information of that PCC rule according to the requested type(s) of notifications. Otherwise, the PCF provides a new PCC Rule with the received Traffic Descriptor in the SDF Template, the Downlink Data Notification Control information set according to the requested type(s) of notifications and other PCC Rule information set to the same values as in the existing PCC rule that previously matched the traffic. When the new PCC has to be bound to the QoS Flow associated with the default QoS rules, the PCF sets the "Bind to QoS Flow associated with the default QoS rule" parameter. From now on, the PCF needs to keep the PCC rule for the DDD event detection fully synchronized with the existing PCC rule that previously matched the traffic for all other policy and charging control settings to ensure the same user experience and traffic treatment according to the operator policy. When the SMF indicates a cancellation event, the PCF removes the Downlink Data Notification Control information in the installed PCC Rule or removes the PCC Rule if a new PCC rule has been provided during the subscription event and this PCC rule is no longer necessary for any other policy enforcement.

NOTE 11:	Downlink Data Delivery (DDD) status event and DDN Failure event are specified in clause 4.15.3 of TS 23.502 [3].

The QoS constraints change trigger shall trigger a SMF interaction with the PCF if QoS constraints are received by the SMF during the lifetime of the PDU Session. The SMF reports that the QoS constraints change trigger was met and the new QoS constraints.

When the Satellite backhaul category change trigger is armed, the SMF reports to the PCF that the Satellite backhaul category change was met and the new satellite backhaul category (or that satellite backhaul is no longer used) when it becomes aware that there is a change of the backhaul which is used for the PDU Session between satellite backhaul categories, or between satellite backhaul and a non-satellite backhaul. The SMF determines whether or not a satellite backhaul is used and whether there is a change of backhaul based on signalling from the AMF as specified in TS 23.501 [2].

NOTE 12:	As specified in clause 5.43.4 of TS 23.501 [2], satellite backhaul category refers to the type of the satellite used in the backhaul. Only a single backhaul category can be indicated.

The NWDAF info change trigger shall trigger the SMF to interact with the PCF when the list of NWDAF Instance IDs used for the PDU Session or associated Analytics IDs used for the PDU Session are changed in the SMF.

The Request for notification on SM Policy Association establishment or termination indicates to the SMF that the request from the AMF to notify on the established or terminated SM Policy Association should be sent to the PCF together with the received PCF binding information.

When the Notification on BAT offset trigger is set, the SMF shall, upon receiving a BAT offset and optionally an adjusted periodicity from the RAN (in a notification that GFBR of the QoS Flow can no longer be guaranteed as defined in clause 5.27.2.5.3 of TS 23.501 [2]), report the BAT offset and optionally the adjusted periodicity to the PCF for the PCC rule which is bound to the QoS Flow for which the notification from RAN was received.

The UE reporting Connection Capabilities from associated URSP rule trigger indicates to the SMF that when a UE includes Connection Capabilities in the PDU Session Establishment Request or PDU Modification Request, the SMF shall forward this information to the PCF as described in clause 6.6.2.4, if the PCRT is set in the SMF.

Editor's note:	Whether UE reporting Connection Capabilities from associated URSP rule trigger needs to be a new trigger or whether "start of application traffic detection and stop of application traffic detection" trigger can be reused is FFS.

The UE Policy Container received trigger shall trigger a SMF interaction with the PCF, if a UE Policy Container is received from the UE in EPC over a PDN connection.

When the Change of HR-SBO support indication trigger is armed, the H-SMF reports to the H-PCF that the HR-SBO support indication change was met. The H-SMF determines whether there is a change of HR-SBO support indication based on HR-SBO Request Indication from the V-SMF and/or the SM subscription data from UDM as described in clause 6.7.2.2 of TS 23.548 [33].

The Network Slice Replacement trigger shall trigger a SMF interaction with the PCF to notify change between S-NSSAI and Alternative S-NSSAI when the SMF determines that the existing PDU Session and existing SM Policy Association can be retained as described in clause 4.15.19 of TS 23.501 [2]. The SMF provides Alternative S-NSSAI if the PDU Session is transferred from a S-NSSAI to its Alternative S-NSSAI. The SMF indicates to the PCF that the PDU Session is transferred from the Alternative S-NSSAI to the replaced S-NSSAI, when the replaced S-NSSAI is available again and the PDU Session is transferred to the replaced S-NSSAI.

NOTE:	The SMF reports to the PCF a PDU session transfer anytime when the PDU Session is transferred from one S-NSSAI to another S-NSSA.

If the 'ECN marking for L4S can no longer (or can again) be performed trigger' is armed, the SMF shall report to the PCF for those PCC rules which have enabled ECN marking for L4S (explicitly or implicitly as described in clause 6.1.3.22) if neither RAN nor UPF PSA ECN marking for L4S can be enabled on the affected QoS Flows, and when ECN marking for L4S can be enabled on the affected QoS Flows (again).

The Result of UE Policy Container delivery via EPS trigger shall trigger a SMF interaction with the PCF, if the SMF received the UE response on the result of URSP delivery via EPS in ePCO in Update Bearer Response, or rejection for Update Bearer Request with appropriate reason (e.g. due to the UE is not reachable), as described in clause 4.11.0a.2a.10 of TS 23.502 [3].

NEXT CHANGE

6.1.3.29	Network Slice replacement for a PDU Session

The Network Slice Replacement is specified in clause 5.15. 19 of TS 23.501 [2].

The PCF may set the Network Slice Replacement trigger event in the SMF as defined in clause 6.1.3.5.

When a new PDU Session is established and the SMF receives both an S-NSSAI and an Alternative S-NSSAI, the SMF includes both the replaced S-NSSAI and the Alternative S-NSSAI in SM Policy Association establishment request to PCF. The PCF maintains both the S-NSSAI and the Alternative S-NSSAI in the Session Management context. The PCF may make a policy decision based on the Alternative S-NSSAI when this does not conflict with the policy decisions based on the replaced S-NSSAI, e.g if there is no PDU Session policy control subscription information associated with the Alternative S-NSSAI in the UDR the PCF uses the policies associated with the replaced S-NSSAI. The PCF registers the replaced S-NSSAI in the BSF.

When the existing PDU Session is transferred from S-NSSAI to Alternative S-NSSAI, if the SMF determines that the existing PDU Session and existing SM Policy Association can be retained (e.g. if the SMF determines that same PCF can be used for the Alternative S-NSSAI), then the SMF includes the Alternative S-NSSAI in SM Policy Association modification request to PCF to update the S-NSSAI of the PDU Session. The PCF may request subscription information associated with the Alternative S-NSSAI from the UDR when these do not conflict with the policy decisions based on the replaced S-NSSAI and if there is  no PDU Session policy control subscription information associated with the Alternative S-NSSAI in the UDR the PCF uses he policies associated with the replaced S-NSSAI. Then the PCF makes policy decision based on the Alternative S-NSSAI and may provide updated PCC rules to SMF. The PCF does not update the binding information in the BSF.

When existing PDU Session is transferred back to the replaced S-NSSAI, the SMF sends SM Policy Association modification request to PCF to update the S-NSSAI of the PDU Session. Then the PCF makes policy decision based on the replaced S-NSSAI and may provide updated PCC rules to the SMF.

The PCF checks if the PDU Session of a SUPI for a DNN and a S-NSSAI that has been replaced by an Alternative S-NSSAI is subject of usage monitoring control, as defined in clause 6.2.1.7, if so, the PCF retrieves usage monitoring related information per DNN and Alternative S-NSSAI combination for a SUPI from the UDR if available, then performs usage monitoring control as described in clause 6.2.1.6 based on the SUPI for a DNN and replaced S-NSSAI combination and additionally the SUPI for a DNN and replaced S-NSSAI combination, until the Alternative S-NSSAI is not applicable any longer. The PCF shall stop monitoring control for both the DNN and Alternative S-NSSAI combination and start doing usage monitoring control only for the DNN and replaced S-NSSAI combination when the replaced S-NSSAI is available again.

The PCRTs provided to the SMF are still reported to the PCF, when the PDU Session is established or has been replaced by an Alternative S-NSSAI.

NEXT CHANGE

6.4	PDU Session related policy information

The purpose of the PDU Session related policy information is to provide policy and charging control related information that is applicable to a single Monitoring key or the whole PDU Session respectively. The PCF may provide PDU Session related policy information to the SMF together with PCC rules or separately.

Table 6.4-1 includes the PDU Session related policy information.

The differences with table 6.4 and table 6.6 in TS 23.203 [4] are shown, either "none" means that the IE applies in 5GS or "removed" meaning that the IE does not apply in 5GS, this is due to the lack of support in the 5GS for this feature or "modified" meaning that the IE applies with some modifications defined in the IE.

Table 6.4-1: PDU Session related policy information

| Attribute | Description | PCF permitted to modify for dynamically provided information | Scope | Differences compared with table 6.4. and 6.6 in TS 23.203 [4] |
| --- | --- | --- | --- | --- |
| Charging information | Defines the containing CHF address and optionally the associated CHF instance ID and CHF set ID. | No | PDU Session | None |
| Default charging method | Defines the default charging method for the PDU Session. | No | PDU Session | None |
| PDU Session with offline charging only | Indicates that the "online" charging method is never used for PCC rules in the PDU Session. | No | PDU Session | Added |
| Policy control request trigger | Defines the event(s) that shall cause a re-request of PCC rules for the PDU Session. | Yes | PDU Session | Explicitly subscribed by invoking Npcf_SMPolicyControl service operation |
| Authorized QoS per bearer (UE-initiated IP-CAN bearer activation/modification) | Defines the authorised QoS for the IP-CAN bearer (QCI, GBR, MBR). | Yes | IP-CAN bearer | Removed |
| Authorized MBR per QCI (network initiated IP-CAN bearer activation/modification) | Defines the authorised MBR per QCI. | Yes | IP-CAN session | Removed |
| Revalidation time limit | Defines the time period within which the SMF shall perform a PCC rules request. | Yes | PDU Session | None |
| PRA Identifier(s) | Defines the Presence Reporting Area(s) to monitor for the UE with respect to entering/leaving | Yes | PDU Session | None but only applicable to PCF |
| List(s) of Presence Reporting Area elements (NOTE 14) | Defines the elements of the Presence Reporting Area(s) | Yes | PDU Session | None but only applicable to PCF |
| Default NBIFOM access | The access to be used for all traffic that does not match any existing Routing Rule | Yes (only at the addition of an access to the IP-CAN session) | IP-CAN session | Removed |
| IP Index (NOTE 11) | Provided to SMF to assist in determining the IP Address allocation method (e.g. which IP pool to assign from) when a PDU Session requires an IP address – as defined in clause 5.8.2.2.1 of TS 23.501 [2]. | No | PDU Session | Added |
| Redundant PDU Session | Indicates whether the PDU Session is a redundant PDU Session | No | PDU Session | New |
| Explicitly signalled QoS Characteristics (NOTE 1) | Defines a dynamically assigned 5QI value (from the non-standardized value range) and the associated 5G QoS characteristics as defined in clause 5.7.3 of TS 23.501 [2]. | No | PDU Session | Added |
| Reflective QoS Timer | Defines the lifetime of a UE derived QoS rule belonging to the PDU Session. | No | PDU Session | Added |
| Authorized Session-AMBR (NOTE 2) (NOTE 3) | Defines the Aggregate Maximum Bit Rate for the Non-GBR QoS Flows of the PDU Session. | Yes | PDU Session | Modified |
| Authorized default 5QI/ARP (NOTE 3) (NOTE 10) | Defines the default 5QI and ARP of the QoS Flow associated with the default QoS rule. | Yes | PDU Session | Modified |
| Time Condition (NOTE 4) | Defines the time at which the corresponding Subsequent Authorized Session-AMBR or Subsequent Authorized default 5QI/ARP shall be applied. | No (NOTE 5) | PDU Session | Modified |
| Subsequent Authorized Session-AMBR (NOTE 4) (NOTE 2) | Defines the Aggregate Maximum Bit Rate for the Non-GBR QoS Flows of the PDU Session when the Time Condition is reached. | No (NOTE 5) | PDU Session | Modified |
| Subsequent Authorized default 5QI/ARP (NOTE 4) (NOTE 10) | Defines the default 5QI and ARP when the Time Condition is reached. | No (NOTE 5) | PDU Session | Modified |
| Usage Monitoring Control related information (NOTE 12) (NOTE 13) | Defines the information that is required to enable user plane monitoring of resources for individual applications/services, groups of applications/services, for a PDU Session. |  |  |  |
| Monitoring key | The PCF uses the monitoring key to group services that share a common allowed usage. | No | PDU Session (NOTE 12) | None |
| Volume threshold (NOTE 7) | Defines the traffic volume value after which the SMF shall report usage to the PCF for this monitoring key. | Yes | Monitoring key | None |
| Time threshold (NOTE 7) | Defines the resource time usage after which the SMF shall report usage to the PCF. | Yes | Monitoring key | None |
| Monitoring time | Defines the time at which the SMF shall reapply the Volume and/or Time Threshold. | No (NOTE 6) | Monitoring Key | None |
| Subsequent Volume threshold (NOTE 9) | Defines the traffic volume value after which the SMF shall report usage to the PCF for this Monitoring key for the period after the Monitoring time. | No (NOTE 6) | Monitoring Key | None |
| Subsequent Time threshold (NOTE 9) | Defines resource time usage after which the SMF shall report usage to the PCF for this Monitoring key for the period after the Monitoring time. | No (NOTE 6) | Monitoring Key | None |
| Inactivity Detection Time (NOTE 8) | Defines the period of time after which the time measurement shall stop, if no packets are received. | Yes | Monitoring Key | None |
| Ethernet or IP port management related |  |  |  |  |
| Port number | Port number for which Port Management Information Container is provided. | Yes | PDU Session | New |
| Port Management Information Container | Includes Ethernet/IP port management information. | Yes | PDU Session | New |
| User plane node Management Information Container | Includes User plane node management information. | Yes |  | New |
| Target of reporting | Target of reporting (indicated as Notification Target Address + Notification Correlation ID). | Yes | PDU Session | New |
| PDU Session slice Inactivity Timer value (NOTE 16) | Defines the inactivity timer value for a PDU Session before releasing the PDU session due to slice usage control. | Yes | PDU Session | New |
| VPLMN Specific Offloading Policy | HR-SBO policy for the local part of DN in VPLMN. |  |  |  |
| IP range(s) | IP address range(s) allowed to be routed to the local part of DN in VPLMN. | Yes | PDU Session (NOTE 15) | New |
| FQDN(s) | FQDN(s) allowed to be routed to the local part of DN in VPLMN. | Yes | PDU Session (NOTE 15) | New |
| Authorized DL Session AMBR for Offloading | Defines the DL Aggregate Maximum Bit Rate for the Non-GBR QoS Flows of the PDU Session authorized for offloading to the local part of DN in VPLMN. | Yes | PDU Session (NOTE 15) | New |
| NOTE 1: Multiple Non-standardized QoS Characteristics can be provided by the PCF. Operator configuration is assumed to ensure that the non-standardized 5QI to QoS characteristic relation is unique within the PLMN. NOTE 2: The Authorized Session-AMBR and the Subsequent Authorized Session-AMBR may be provided together with a list of Access Types possibly complemented by RAT types. NOTE 3: There is always an unconditional value for the Authorized Session-AMBR and Authorized default 5QI/ARP available at the SMF. The initial value is received as Subscribed Session-AMBR/Subscribed default 5QI/ARP, and the PCF can overwrite it with these parameters. NOTE 4: The Time Condition and Subsequent Authorized Session-AMBR/ Subsequent Authorized default 5QI/ARP are used together. The PCF may provide up to four instances of them. When multiple instances are provided, the values of the associated Time Condition have to be different. NOTE 5: The PCF may replace all instances that have been provided previously with a new instruction. A previously provided Time Condition and Subsequent Authorized Session-AMBR/ Subsequent Authorized default 5QI/ARP pair cannot be individually modified. NOTE 6: The PCF may replace all instances that have been provided previously with a new instruction. A previously provided Volume threshold/Time threshold and Monitoring Time pair cannot be individually modified. NOTE 7: This attribute is also used by the SMF, e.g. during PDU Session termination, to inform the PCF about the resources that have been consumed by the UE. NOTE 8: This attribute is applicable in presence of Time threshold only. NOTE 9: This attribute is applicable in presence of Monitoring Time only. NOTE 10: The Authorized default 5QI and the Subsequent Authorized default 5QI shall be of Non-GBR Resource Type. NOTE 11: This attribute is applicable only when no IP address/Prefix for the PDU Session is received from the SMF. NOTE 12: A Monitoring Key can either be used to monitor the traffic of a PDU Session, the traffic of a PDU Session per access (for a MA PDU Session) or the traffic of specific SDF(s) in the PCC Rule(s) that share the same Monitoring Key. NOTE 13: For a MA PDU Session, the PDU Session level Usage Monitoring shall be possible per access (i.e. 3GPP and/or Non-3GPP) and irrespective of the access. NOTE 14: The list of PRA elements shall be a short list of elements. NOTE 15: This attribute is applicable only for the PDU Session supporting HR-SBO. This attribute is applied only to the current serving VPLMN. This attributed shall not be forwarded to RAN or UE. NOTE 16: Included if the PDU Session Slice Inactivity Timer value is set by PCF. |  |  |  |  |

Upon the initial interaction with the SMF, the PCF may provide the following attributes to the SMF:

The Charging information contains addresses of the CHF that manages charging for the PDU Session and optionally the associated CHF instance ID and CHF set ID (see clause 6.3.1.0 of TS 23.501 [2]). If received, the SMF shall apply it as defined in clause 6.3.11 of TS 23.501 [2].

The Default charging method indicates what charging method shall be used in the PDU Session for every PCC rule where the charging method identifier is omitted, including predefined PCC rules that are activated by the SMF. If received by the SMF, it supersedes the Default charging method in the charging characteristics profile.

The PDU Session with offline charging only can be assigned to a PDU Session by the PCF to indicate that the online charging method is never set for any of the PCC Rules activated during the lifetime of the PDU Session nor provided as Default charging method.

NOTE 1:	If this parameter is provided by the PCF or configured in the SMF charging characteristics the SMF can use the Nchf_OfflineOnlyCharging service instead of the Nchf_ConvergedCharging service for a PDU Session as defined in TS 32.255 [21].

NOTE 2:	When the "PDU Session with offline charging only" indication is provisioned by the PCF, the PCF can set the charging method for a PCC rule within this PDU Session to either "offline" or "neither".

The IP Index indicates the IP Address/Prefix allocation method which is used by the SMF for IP Address/Prefix allocation during PDU Session Establishment procedure as defined in clause 5.8.2.2.1 of TS 23.501 [2].

Upon every interaction with the SMF, the PCF may provide the following attributes to the SMF:

The Revalidation time limit defines the time period within which the SMF shall trigger a request for PCC rules for an established PDU Session.

The Reflective QoS Timer defines the lifetime of a UE derived QoS rule belonging to the PDU Session. It is used in the UE as defined in clause 5.7.5.3 of TS 23.501 [2].

NOTE 3:	The Reflective QoS Timer that is sent to the UE has to be in alignment with the corresponding timer configured in the UPF (defined in clause 5.7.5.3 of TS 23.501 [2]).

The Authorized Session-AMBR defines the UL/DL Aggregate Maximum Bit Rate for the Non-GBR QoS Flows of the PDU Session, which is enforced in the UPF as defined in clause 5.7.1 of TS 23.501 [2]. The PCF may provide the Authorized Session-AMBR in every interaction with the SMF. When the SMF receives it from the PDU Session policy, it is provided to the UPF over N4 interface for the enforcement.

The Authorized default 5QI/ARP defines the 5QI and ARP values of the QoS Flow associated with the default QoS rule as described in clause 6.2.2.4. The PCF may provide a 5QI Priority Level according to clause 5.7.3.3 of TS 23.501 [2] together with the Authorized default 5QI, when a 5QI Priority Level value different from the standardized Default Priority Level value in the QoS characteristics Table 5.7.4-1 of TS 23.501 [2] is required. The SMF applies the Authorized default 5QI/ARP also for the QoS Flow binding as described in clause 6.1.3.2.4.

The Time Condition and Subsequent Authorized Session-AMBR / Subsequent Authorized default 5QI/ARP are used together and up to four instances with different values of the Time Condition parameter may be provided by the PCF. Time Condition indicates that the associated Subsequent Authorized Session-AMBR/ Subsequent Authorized default 5QI/ARP is only applied when the time defined by this attribute is met. The PCF may provide a 5QI Priority Level according to clause 5.7.3.3 of TS 23.501 [2] together with the Subsequent Authorized default 5QI, when a 5QI Priority Level value different from the standardized Default Priority Level value in the QoS characteristics Table 5.7.4-1 of TS 23.501 [2] is required. When the SMF receives a Time Condition and Subsequent Authorized Session-AMBR/ Subsequent Authorized default 5QI/ARP pair, it stores it locally. The SMF shall discard any previously received Subsequent Authorized Session-AMBR / Subsequent Authorized default 5QI/ARP instances on explicit instruction as well as whenever the PCF provides a new instruction for one or more Subsequent Authorized Session-AMBR / Subsequent Authorized default 5QI/ARP. When the time defined by the Time Condition parameter is reached, the SMF shall apply (or instruct the UPF to apply) Subsequent Authorized Session-AMBR/ Subsequent Authorized default 5QI/ARP.

NOTE 4:	In order to reduce the risk for signalling overload, the PCF should avoid simultaneous provisioning of the Subsequent Authorized Session-AMBR/ Subsequent Authorized default 5QI/ARP for many UEs (e.g. by spreading over time).

NOTE 5:	In order to provide further Subsequent Authorized Session-AMBR/ Subsequent Authorized default 5QI/ARP in a timely fashion the PCF can use its own clock to issue the desired changes or use the Revalidation time limit parameter to trigger an SMF request for a policy decision.

NOTE 6:	For services that depend on specific Session-AMBR and/or default 5QI/ARP (e.g. MPS session) the PCF is responsible to ensure that no Subsequent Authorized Session-AMBR or Subsequent Authorized default 5QI/ARP interfere with the service, e.g. by removing the Subsequent Authorized Session-AMBR or Subsequent Authorized default 5QI/ARP before the respective change time is reached.

The Monitoring Key is the reference to a resource threshold. Any number of PCC Rules may share the same monitoring key value. The monitoring key values for each service shall be operator configurable.

It shall also be possible for an operator to use the Monitoring Key parameter to indicate usage monitoring on an PDU Session level or, in the case of an MA PDU Session, to indicate usage monitoring on PDU Session level for the 3GPP access and/or the Non-3GPP access.

Usage monitoring on PDU Session level is active when a PDU Session is active when a Monitoring Key for the PDU Session and a corresponding volume and/or time threshold value have been provided to the SMF. Usage monitoring on Monitoring key level is active when a volume and/or time threshold has been provided for a Monitoring Key to the SMF and there is at least one PCC rule active for the PDU Session that is associated with that Monitoring Key.

The Volume threshold indicates the overall user traffic volume value after which the SMF shall report the Usage threshold reached trigger to the PCF.

The Time threshold indicates the overall resource time usage after which the SMF shall report the Usage threshold reached trigger to the PCF.

The Monitoring time indicates the time at which the SMF shall store the accumulated usage information.

The Subsequent Volume threshold indicates the overall user traffic volume value measured after Monitoring time, after which the SMF shall report the Usage threshold reached trigger to the PCF.

The Subsequent Time threshold indicates the overall resource time usage measured after Monitoring time, after which the SMF shall report the Usage threshold reached trigger to the PCF.

The Inactivity Detection Time indicates the period of time after which the time measurement shall stop, if no packets are received during that time period.

The Port Management Information Container carries Ethernet or IP port management information for an Ethernet/IP port located in DS-TT or NW-TT. The port for which the container is provided is identified by the port number.

The User plane node Management Information Container carries User plane node management information for a 5GS Bridge or Router.

The VPLMN specific roaming offloading policy carries the attributes for the traffic to be offloaded to the local part of DN in VPLMN. The following attributes under this policy is applicable only for the serving VPLMN. When the H-SMF receives it from PCF and HR-SBO is authorized, H-SMF shall send this policy to the V-SMF. The V-SMF may use this information to configure V-EASDF, ULCL/BP UPF and Local UPF.

The IP range(s) indicates one or more IPv4/IPv6 address range(s) that are allowed to be offloaded to the local part of the DN in VPLMN when the PDU Session is authorized for HR-SBO.

The FQDN(s) indicates one or more FQDN or FQDN range expressed by regular expression that are allowed to be offloaded to the local part of the DN in VPLMN when the PDU Session is authorized for HR-SBO.

The Authorized Session-AMBR for Offloading defines the DL Aggregate Maximum Bit Rate for the Non-GBR QoS Flows applicable for the local traffic offloaded to the local part of DN in VPLMN of the PDU Session for HR-SBO.

END CHANGE