3GPP TSG-SA WG2 Meeting #160 Ad hoc-e	S2-2401020r03

22–29 Jan. 2024, Emeeting

Source:	Xiaomi

Title:	KI #1, New Sol: Support of Regenerative-based satellite access.

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: The contribution proposes a solution to Key Issue #1: support of regenerative-based satellite access.

1. Introduction

This contribution proposes a solution to Key Issue #1: Support of regenerative based satellite access.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-29-020.

* * * Start of change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 |  |
| 6.x | X |  |  |  |

* * * Next change * * * *

## 6.x	Solution X:	Support of Regenerative-based satellite access with deploying RAN-agent and Proxy RAN node

### 6.x.1	Description

This solution resolves KI#1 about any impact of RAN nodes changing for any given 5GC/EPC and for a given area in the case of RAN nodes on-board moving.

Embarking gNB on-board LEO/MEO implies the on-board gNB serving for the UE access will be change as the satellite moving, which may cause the following network impacts assuming the UE is not moving:

-	N2/N3 connection frequent change (disconnection of N2/N3 between old serving gNB and AMF/UPF, reconnection of N2/N3 between new serving gNB and AMF/UPF) will cause lots of signalling on N2 interface.

-	Frequent change of the gNB serving for the TA the UE located which may cause AMF unware of which gNB should paging the UE.

The solution proposes to enhance 5G network architecture to reduce the impact of 5GC. Solution#8 in TR 23.737-h20 is proposed to reuse to address the above mentioned issues.

In this solution:

- Deploying RAN agent close to gateways on earth as shown in Figure 6.x.1-1. The RAN agent is used to process control plane signalling, i.e. handle N2 signalling and forward the N2 messages between gNB and AMF.

- Deploying Proxy RAN nodes which is collocated with RAN agent. The Proxy RAN node is used to process user plane data, i.e. terminate the N3 connection with UPF and establish N3’ connection with serving gNB.

NOTE 1:	This is to study the architecuture enhancement. The feasibility of the solution with the new defined NFs and relevant interfaces are in the scope of RAN and needs to coordinate with RAN WGs.

Editor’s Note:	Each ground gateways may deployed different RAN agents and Proxy RAN node. How to support scenarios 3 and 4 mentioned in Annex A of 23.700-29-020 is FFS.

Figure 6.x.1-1: Support of LEO/MEO satellite based gNB using RAN agent and Proxy RAN node

NOTE 2:	A similar mechanism can also be applied for EPS where the gNB uses eNB instead.

The solution assumes that Earth-fixed Tracking Areas are used. The cells can be Earth-moving or Earth-fixed cells.

The solution assumes that in a given geographical area the “setting” satellite passes the control on per-cell basis to the “rising” satellite.

The funcations of RAN agent include:

-	Establishing TNL Associations with the gNB, and establishing TNL Associations with the AMF;

-	Terminating the NG AP message(s) from on-board gNB for NG Setup or RAN Configuration Update;

-	Sending a NG AP message to the AMF to establish or update NG connection by including a proxy RAN node ID and/or proxy RAN node name which are statically configured with same format as RAN node ID and RAN node name;

-	Storing the mapping between TA(s) that each connected on-board gNB serves and the gNB ID;

-	Storing the mapping between the serving gNB ID and the proxy RAN node ID. Updating the mapping information if the serving gNB is changed.

-	Provide N3 and N3’ tunnel information through Nx interface to indicate proxy RAN node to establish N3 and N3’ connection

-	Receiving Paging message from AMF, and forwarding the Paging message to the gNB(s) which is serving the Registration Area allocated to the UE;

-	Allocating a RAN UE NGAP proxy ID which has same format as RAN UE NGAP ID allocated by the gNB. The RAN UE NGAP proxy ID doesn't change as long as the UE is within the serving area of the RAN agent;

-	Storing 1:1 mapping between RAN UE NGAP ID and RAN UE NGAP proxy ID;

-	Updating the mapping between RAN UE NGAP ID and RAN UE NGAP proxy ID if RAN UE NGAP ID allocated by the gNB is changed;

-	Replacing the RAN UE NGAP ID included in any UE associated NG AP message from the gNB with the corresponding RAN UE NGAP proxy ID;

-	Replacing the RAN UE NGAP proxy ID included in any UE associated NG AP message from the AMF with the corresponding RAN UE NGAP ID.

NOTE 2:	Security related functions need to be addressed by SA WG3 during the normalisation phase.

Existing Registration management and connection management can be reused in the 5G network with support of gNBs on NGSO satellites with following changes:

-	The RAN agent needs to identify which on-board gNB(s) is serving the Registration Area allocated to a UE before paging the UE.

The funcations of Proxy RAN node include:

-	Receiving N3 and N3’ tunnel information to establish N3 and N3’ connection.

-	Receiving data traffic from serving gNB using N3’ tunnel and sending the data traffic to the UPF using N3 tunnel.

-	Receiving data traffic from UPF using N3 tunnel and sending the data traffic to the serving gNB using N3’ tunnel.

### 6.x.2	Procedures

The following call flow should be applied to RAN agent.

Figure 6.X.2: Call Flow for RAN agent

1.	The address of an RAN agent is configured to the earth station, so that the NG setup messages sent by satellite RAN nodes can be sent to the RAN agent.

2.	The RAN agent stores the TA list supported by the satellite RAN node, and replaces the RAN node ID with a Proxy RAN node ID which is allocated by the RAN agent. The RAN agent stores the mapping between the RAN node ID and the Proxy RAN node ID. TheRAN agent may provide its serving area (TA list) to the AMF.

3-4.The AMF responds the request, the RAN agent replaces the Proxy RAN node ID with the RAN node ID, and forwards the response message to the NG RAN node.

5.	If satellite RAN node changes the supported TA list due to mobility, then the satellite RAN node sends RAN Configuration Update message including the updated TA list the RAN node supports. The message is not required to be sent to the AMF.

6.	The RAN agent sends an acknowledge message.

7.	When a UE registers via satellite RAN. The NAS transport message carrying Registration Request will be sent to the RAN agent first.

8.	The RAN agent needs to allocate a RAN UE NGAP proxy ID which is similar as RAN UE NGAP ID, and replaces the RAN UE NGAP ID in the NAS transport message with the allocated RAN UE NGAP proxy ID (so that the downlink message can be routed to the AMF agent). The RAN agent stores the mapping between RAN UE NGAP ID and RAN UE NGAP proxy ID.

9-10.	The AMF responds the registration request, the RAN agent replaces the RAN UE NGAP proxy ID with the RAN UE NGAP ID, and forwards the response message.

### 6.x.3	Impacts to Services, Entities and Interfaces

There is no impact to the existing NFs or Entities (e.g. gNB, AMF, SMF). However, the proposed solution needs to deploy two new NFs, as the following:

RAN agent:

- New NF, which is located near the satellite ground station. The functionalities can be referred to clause 6.x.1.

Proxy RAN node:

- New NF, which is co-located with RAN agent. The functionalities can be referred to clause 6.x.1.

Nx interface:

- New interface, which is used to provide N3 and N3’ tunnel information to the proxy RAN node by RAN agent.

N2’/N3’ interface:

- Can reuse the funcationality of N2/N3 interface

* * * End of change * * * *