3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401059r01

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Background Information for Ambient IoT Service

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_Ambient IoT / Rel-19

Abstract: The contribution would like to provide background information for better understanding of the Ambient IoT service.

# 1. Introduction/Discussion

This contribution would like to propose an overview of Ambient IoT service in order to provide relative background information for better understanding of Ambient IoT service business model, which can be used as guidance for thinking detailed solutions targeting to different Key Issues or different aspects to be considered.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

Annex <X> (informative):
Service enablement and value chain for Ambient IoT service

# X.1	General

Figure X.1-1: High level flow chart showing the steps to enable an Ambient IoT service

The following events are expected to take place for the enablement of Ambient IoT services:

Service agreement

For a mobile network to provide Ambient IoT services to a third party (e.g. enterprise), the third party and the operator need to signa service agreement. The service agreement can include the authorized service area, service operation type (e.g. inventory, read, write, permanently disabling of devices), number of Ambient IoT devices, Ambient IoT device Identifiers, Security requirements etc. The service agreement may involve the aspects of the device identification and subscription, which will be studied in SA WG2.

Configuration

The configuration can include both the configuration in the operator network and the configuration of the Ambient IoT devices used to provide the service. How and when the information is provided to the network is not described here, but examples what the information is used for include:

to enable following example aspects:

-	Ambient IoT device management

-	Authorisation of Ambient IoT service request from a third party (AF)

-	Reader management

-	Charging for the use of Ambient IoT service

The Ambient IoT device is assumed to be configured with information for accessing to the network and for service transmission. The information can include, for example, device identifier, credentials (if needed). This event mainly concerns the identification aspects.

NOTE 1:	How to configure and provision such information is not defined by 3GPP.

NOTE 2:	The information needed to configure a secure communication with the Ambient IoT device is determined by SA WG3.

Using Ambient IoT Services

The Ambient IoT Services can be initiated to support e.g. inventory services, traffic to and from the AIoT Device etc.. This procedure may also include the authentication and/or authorization of Ambient IoT Device, which depends on solution. This event mainly involves the aspects of identification, subscription, service operation and security.

Information collection for Charging

Network collects and reports information to support service charging, and the third party can be charged accordingly.

# * * * * End of changes * * * *