3GPP TSG-WG SA2 Meeting #160E e-meeting	S2-2401068

Elbonia, January 22th – 29th, 2024

Source:	China Mobile, KDDI?, Lenovo?, vivo?, Huawei?

Title:	Use case for user experience analytics based on VFL

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN/R19

1. Proposal

It is proposed to agree the following use case into TR 23.700-84.

* * * First change * * * *

X	Use Cases

X.X	Use Case #X: NWDAF supported user experience analytics based on VFL

x.x.1	 Description

In many cases, raw data can not be exchanged directly between 5GC/NWDAF and AF domains because the user data has high level privacy protection needs. VFL can be very helpful to enable data sharing and jointly training between 5GC/NWDAF and AF.

One of the important use cases for network is user experience analytics. It provides the real user feedback to the network so that the network could self-optimize and offer customized services according to the true user needs. In this area, 5GC/NWDAF could help. Due to the issue of data privacy, the AF and 5GC may not be able to exchange the data directly. By leveraging VFL technology with the situation that the datasets of distributed nodes, i.e., between NWDAF and AF, have different features, NWDAF jointly train a model for user experience analytics together with AF, which predicts the user experience accurately. Then the network may adjust its policy to achieve better performance.

NWDAF and AF would collect their local training data, respectively (e.g., access speed, network access delay for NWDAF, stall time, frame rate for AF). The data has different features where VFL could be applied to. NWDAF and AF perform VFL to get the prediction results.

We identify two scenarios in this case:

Scenario 1: NWDAF takes control of the training process, and provides the labels of the training data;

Scenario 2: AF takes control of the training process, and provides the labels of the training data

The results can be beneficial to the followings:

- 5GC NFs could adjust their policy for better user experience;

- RAN could reserve resources for higher QoS;

- OAM could optimize the network behaviour for leveraging resources;

And more

* * * End of change * * * *