3GPP TSG-WG SA2 Meeting #160E e-meeting	S2-2401068r03

Elbonia, January 22th – 29th, 2024

Source:	China Mobile, KDDI?, Lenovo, vivo, Huawei?

Title:	Use case for observed service experience analytics based on VFL

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN/R19

1. Proposal

It is proposed to agree the following use case into TR 23.700-84.

* * * First change * * * *

5.x.2.x	Use Case #X: NWDAF supported observed service experience analytics based on VFL

5.x.2.x.1 	 Description

When NWDAF provides observed service experience analytics, as in other analytics that require input data from the AF, raw data cannot be exchanged directly between NWDAF and an external AF, as NWDAF is in the PLMN and the AF is outside the PLMN because the user data has high level privacy protection needs. VFL can be very helpful to enable data sharing and jointly training between NWDAF and AF.

The use case for observed service experience analytics is illustrated as follows. It provides the real user feedback to the network so that the network could self-optimize and offer customized services according to the true user needs. Due to the issue of data privacy, the AF and NWDAF may not be able to exchange the data directly. Horizontal Federated Learning between AF and NWDAF does not apply for this use case as the data available (Features) between the AF and NWDAF will be different. By leveraging VFL technology with the situation that the datasets of distributed nodes, i.e., between NWDAF and AF, have different features, NWDAF and AF can jointly participate to train an ML model for observed service experience...

NWDAF and AF would collect their local training data, respectively (e.g., access speed, network access delay for NWDAF, stall time, frame rate for AF).

We identify two scenarios in this case:

Scenario 1: NWDAF takes control of the training process, and provides the labels of the training data;

Scenario 2: AF takes control of the training process, and provides the labels of the training data.

NOTE X: This use case is applicable only if the AF is capable of participating in VFL procedure as a training entity. AF ML model training specification is out-of-scope.

NOTE Y: This use case is also applicable for other analytics where AF and NWDAF can collaborate, e.g., DN Performance Analytics.

* End of change * * * *