3GPP TSG-SA2 Meeting #160-AH-e	S2-2401088

# 22 - 29 January, 2024, Electronic meeting									(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4724 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MT-SDT term alignment and support of DL data size reporting configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCAP_Ph2 |  |  |  |  |  | Date: |  |  | 2024-1-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 4.8.2.2b, it says in the end of step3 using MT-SDT flag: the NG-RAN determines whether to set the MT-SDT flag as defined in TS 38.300 [9]). While in running CR e.g., R2-2312091 in RAN2, it is called MT-SDT indication instead of MT-SDT flag, the sentence in 6.2.2 is adopted as: The network includes mt-SDT indication in paging message only if the UE’s I-RNTI is included in the paging message. In addition, how the SMF decides to configure proper rules to trigger data size reporting is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Replace MT-SDT flag with MT-SDT indication and editorial change. 2. Add missing data size reporting configuration from SMF to UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Term misalignment between SA2 and RAN2. Missing data size reporting configuration from SMF to UPF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.1.1a, 4.8.2.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.8.1.1a	Connection Inactive procedure with CN based MT communication handling

This procedure may be initiated by the serving NG-RAN node when CN based mobile terminating (MT) communication handling is requested for a UE that is configured with eDRX cycle value longer than 10.24 seconds for RRC_INACTIVE state and has at least one PDU session with active user plane as defined in clause 5.31.7 of TS 23.501 [2].

Figure 4.8.1.1a-1: NG-RAN initiated Connection Inactive procedure with CN based MT communication handling

0.	The UE is registered in the network with negotiated eDRX parameters for CM-IDLE state and is in CM-CONNECTED with RRC_CONNECTED state. The AMF provides the eDRX values for CM-IDLE state to NG-RAN as part of the RRC Inactive Assistance Information as defined in clause 5.3.3.2.5 of TS 23.501 [2].

1.	NG-RAN determines eDRX values for UE in RRC_INACTIVE state and decides to initiate Connection Inactive with CN based MT communication handling as specified in clause 5.31.7.2.4 of TS 23.501 [2].

1a.	Optionally the NG-RAN may initiate state transition from RRC_CONNECTED to RRC_INACTIVE with RRC configuring eDRX value as specified in TS 38.300 [9]. The NG-RAN may send the request in step 2 towards CN immediately following step 1a or the NG-RAN may delay this request towards CN as specified in clause 5.31.7.2.1 of TS 23.501 [2].

2.	Either immediately following step 1a or after having delayed the request for NG-RAN based on implementation, the NG-RAN sends N2 MT Communication Handling request message to AMF indicating the UE is transitioning to RRC_INACTIVE state. The CN takes the NG-RAN request into consideration and handles MT communication. The NG-RAN also provides the determined eDRX values for RRC_INACTIVE to AMF.

If the NG-RAN receives DL NAS message and the UE is in RRC_INACTIVE with RRC configured eDRX and is considered not reachable, NG-RAN indicates to the AMF a NAS non-delivery and then initiates for the CN to handle mobile terminated (MT) communication.

3.	For each of the PDU sessions with user plane resources have been activated, the AMF invokes Nsmf_PDUSession_UpdateSMContext Request (PDU Session ID, Cause, Operation type, User Location Information, Age of Location Information, N2 SM Information (Secondary RAT usage data), CN based MT handling indication) towards SMF. The Operation Type is set to a value that indicates to stop user plane DL data transmissions towards the UE and enable data buffering. The SMF starts data buffering for MT data if the data buffering is handled in SMF.

4.	If data buffering is handled in the UPF, the SMF updates the UPF with proper rules for MT data handling and may also instruct UPF to report the DL data size in case of DL data notification.

5.	The SMF sends the Nsmf_PDUSession_UpdateSMContext response.

6.	The AMF sends N2 MT Communication Handling response message to NG-RAN acknowledging the NG-RAN request and indicating the AMF has taken the NG-RAN request into account. The AMF considers the UE is in CM-CONNECTED with RRC_INACTIVE state.

7.	If the UE connection is not released as specified in step 1a, the NG-RAN initiates state transition from RRC_CONNECTED to RRC_INACTIVE with RRC configuring the eDRX values as specified in TS 38.300 [9].

#### 4.8.2.2b	Network Triggered Connection Resume in RRC_INACTIVE with CN based MT communication handling

When the UE is in CM-CONNECTED with RRC_INACTIVE state with CN based mobile terminating (MT) communication handling, high latency communication as described in clause 5.31.8 of TS 23.501 [2] is applied.

This procedure may be triggered by MT data, or a N1 procedure from SMF and UPF as shown in Figure 4.8.2.2b-1. When the procedure is triggered by other NFs (e.g. SMSF, LMF, GMLC), the UPF (or SMF) in the following figure should be replaced by the respective NF (the corresponding service operations used by other NFs when they communicate with AMF may also be different from the service operations used by SMF/UPF). For MT-SMS delivery request from SMSF, see also procedures defined in clause 4.13.3.6, clause 4.13.3.7 and clause 4.13.3.8.

During the procedure, the NG-RAN (i.e. gNB) performs RAN paging towards the UE based on the N2 message from the AMF in order to trigger the UE triggered Connection Resume procedure in clause 4.8.2.2.

Figure 4.8.2.2b-1: Network Triggered Connection Resume for UE in RRC_INACTIVE with CN based MT communication handling

1a.	When downlink data is received and the SMF/UPF is requested to perform buffering as specified in clause 4.8.1.1a, the UPF/SMF checks with AMF for the possibility of data delivery, similar to step 2 of clause 4.24.2 with the following differences:

-	The UPF provides the DL data size information of the QoS Flow when sending Data Notification to SMF if the UPF has received instruction from SMF.

-	In the Namf_MT_EnableUEReachability the SMF may also send the following parameters the PPI, the ARP and the 5QI, DL data size and/or QFI for the QoS Flow of the PDU Session which triggered the request for paging policy differentiation as defined in clause 5.4.3.2 of TS 23.501 [2].

If the SMF, while waiting for UE triggered Connection Resume indication or a reject response (with Estimated Maximum Wait time) from the AMF, receives any additional Data Notification message due to additional data packets for another QoS Flow associated with a higher priority (i.e. ARP priority level) than the priority indicated to the AMF in the previous Namf_MT_EnableUEReachability, or the SMF derive a different Paging Policy Indicator according to the additional Data Notification, the SMF invokes a new Namf_MT_EnableUEReachability indicating the higher priority or different Paging Policy Indicator to the AMF. The information contained in the new Namf_MT_EnableUEReachability request overrides the information from the previous Namf_MT_EnableUEReachability request that is stored in the AMF. If the SMF receives any additional Data Notification messages due to additional data packets for another QoS Flow associated with same or lower priority than the priority indicated to the AMF in the previous Namf_MT_EnableUEReachability or if the SMF has sent the second Namf_MT_EnableUEReachability message indicating the higher priority and receives additional downlink data packets for this UE, based on local configuration, the SMF either buffers these Data Notification messages and does not send a new Namf_MT_EnableUEReachability message, or the SMF sends a new Namf_MT_EnableUEReachability message to AMF.

-	The AMF determines if the UE is reachable based on the stored eDRX values for RRC_INACTIVE state provided by NG-RAN in clause 4.8.1.1a. If the UE is unreachable, the AMF stores the information received in the Namf_MT_EnableUEReachability request and provides the Estimated Maximum Wait time in the response message based on the eDRX values for RRC_INACTIVE in AMF (steps 2-5 are postponed until the UE becomes reachable). If the UE is considered reachable, step 2 is executed immediately.

NOTE 1:	This handling is similar to CM-IDLE with eDRX. When the AMF provides the Estimated Maximum Wait time, it can consider the time needed for RRC level procedures (e.g. RRC RNA update procedure) when UE wakes up from the eDRX cycle.

NOTE 2:	The other NFs can use the Namf_Communication_N1N2MessageTransfer service operation to deliver the MT signalling.

2.	When the AMF determines that the UE is reachable, the AMF sends a RAN Paging Request message to NG-RAN with the request for the UE's RRC connection to be resumed. The AMF may include the following per QoS Flow parameter(s) the PPI, the ARP and the 5QI, DL data size and/or QFI for the QoS Flow(s) of the PDU Session in the RAN Paging Request message to trigger and enable RAN paging.

If the AMF receives MT signalling (i.e. via Namf_Communication_N1N2MessageTransfer) in step 1a, AMF includes DL Signalling indication in the N2 RAN Paging Request message.

3.	NG-RAN performs RAN paging towards the UE considering the parameters provided by the AMF. Based on the DL data size for QoS Flow(s), if it's provided, the NG-RAN determines whether to set the MT-SDT indication as defined in TS 38.300 [9] in paging message.

4.	When the UE receives RAN paging, it initiates the UE triggered Connection Resume procedure and NG-RAN notifies CN as specified in clause 4.8.2.2 including the N2 Notification in step 3b.

5.	The UPF triggers downlink data delivery if there is any. The AMF sends downlink NAS messages if there is any.

* * * End of Changes * * *