3GPP TSG-SA2 Meeting #160AH-e	S2-2401119r01

# 22 – 29 January, 2024, Electronic meeting									 (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0496 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update LMF service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following description is agreed in SA2#160 meeting to align with the RAN1 agreement: NOTE 2: Steps 3, 8 and 9 can be performed in any order, including simultaneously. If the LMF determines that simultaneous measurements for UE and PRU(s) are needed, the LMF determines time window(s) in a positioning method dependent manner for simultaneous measurements and sends the time window(s) to UE, PRU(s) and NG-RAN. Definition of the time window and the associated configuration parameters, and the corresponding positioning methods are specified in TS 37.355 [20] and TS 38.455 [15]. If the target UE serving LMF and PRU serving LMF are different, the time window(s) is decided by target UE serving LMF which is responsible to provide the time window(s) to the PRU serving LMF. The PRU serving LMF is responsible to send the time window(s) to PRU. This CR is proposed to update the Nlmf_Location_MeasurementData service operation to add the time window(s) as an optional input parameter. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Nlmf_Location_MeasurementData service operation to add time window(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

8.3.2.6	Nlmf_Location_MeasurementData service operation

Service operation name: Nlmf_Location_MeasurementData

Description: Provides PRU location measurements to the consumer NF.

Input, Required: Target UE cell ID.

Input, Optional: Pre-calculated location of target UE, time window(s).

Output, Required: PRU location measurement(s) and associated PRU known location.

Output, Optional: None.

See clause 6.17 for an example of usage of this service operation.

* * * End of Change * * *