SA WG2 Meeting #160-Ad Hoc-e	S2-2401137r07

# E-meeting, January 22 – 29, 2024			         (revision of S2-234xxxx)

Source: 	Nokia, Nokia Shanghai Bell, Ericsson, CATT, ETRI

Title: 	FS_MASSS_Key Issue for WT#1.2, Registration and mobility management for SUPI/UE associated with a DualSteer device

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes a KI for WT#1.2 of FS_MASSS.

1	Discussion

FS_MASSS SID objectives have 2 sets of work tasks. WT#1 is for supporting DualSteer device. WT#1 has been further divided into sub-tasks as below:

WT#1.1: Study whether and how to enhance subscription aspects for each of the above scenarios.

WT#1.2: Study whether there are any impacts to the registration procedure for DualSteer Devices.

WT#1.3: Session management enhancements and policies:

-	Study whether and how to enhance network policies provided by the HPLMN to the DualSteer Device and within the network to support DualSteer Devices.

-	Study whether and how to enhance session management procedures for initial steering and potential subsequent switching. Traffic policies are in full control of the HPLMN. For sessions subject to potential switching, assuming data anchoring in the HPLMN or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN, study how to select the PSA UPF(s) in the HPLMN to allow routing the traffic across 3GPP accesses towards the same PSA UPF during the switching.

NOTE 9:	Impact to existing functionality related to the change of a service-related data between a 3GPP access network and a non-3GPP access network will be considered as part of WT-D-1.4.

This CR focuses on WT#1.1 and WT#1.2 above.

As per the SA1 definition, DualSteer device is a device supporting traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be a single UE, in case of non-simultaneous data transmission over the two networks, or two separate UEs in case of simultaneous data transmission over the two networks. Further as per SA1 requirements, a DualSteer device has two subscriptions/SUPIs, sharing one subscription profile from the same operator;

The above means that a DualSteer device is different than a Rel-18 UE as the DualSteer device can be a single UE or two separate UEs, having two subscriptions/SUPIs that share one subscription profile.

Observation 1: DualSteer device is a new type of device that will register onto the 5G system.

From the network pov, the network shall be to identify the two SUPIs/UEs that are associated with a DualSteer device during registration, so as to be able to manage the PDU Session requests from those SUPIs of the DualSteer device.

Observation 2: There are possible impacts to Registration procedure/information exchange for supporting registrations from SUPI/UE associated with a DualSteer device.

For a DualSteer device, as there are two subscriptions/SUPIs, sharing one subscription profile from the same operator, there is also subscription data impacts. There could be more than one areas of subscription data that may be impacted based on different solutions and scenarios. For example: there may be enhancements needed to Access and Mobility subscription data for supporting registrations from SUPI/UE associated with a DualSteer device, and/or there may be enhancements needed in Session Management subscription data to support PDU Session requests from SUPI/UE associated with a DualSteer device and/or there may be policy subscription data impacts to support providing appropriate steering/switching policies/rules for DualSteer device. As a result, it may not be feasible to comprehensively study all the subscription aspects in an independent Key Issue. Rather it would be beneficial to study the related subscription data impacts along with the individual Key Issues for studying registration impacts, session management impacts and policy impacts.

Observation 3: Subscription data enhancements are required but it is appropriate to study the subscription data impacts along with the individual Key Issues for studying registration impacts, session management impacts and policy impacts.

Based on the above observations, it is proposed that:

-	There is no need for an independent Key Issue to discuss the Subscription data impacts.

-	Agree on a Key Issue to study the enhancements needed to support registration and mobility management for SUPI/UE associated with a DualSteer device. The Key Issue will also cover the associated subscription data impacts.

2 Proposal

It is proposed to update TR 23.700-54 v0.0.0 on FS_MASSS as follows.

* * * * First Change * * * *

5.1.x	Key Issue #X: Registration management for DualSteer

5.1.x.1 	Description

This Key Issue will study the overall registration management impacts to support DualSteer including the following:

-	Whether and what enhancements are needed in registration and deregistration procedures for supporting DualSteer.

* * * * End of Changes * * * *