3GPP TSG-SA2 Meeting #160-Ad Hoc-e	S2-2401158

# Online, , 22nd Jan 2024 - 29th Jan 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0423 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update procedure for 5G ProSe UE-to-UE Relay Discovery with Model A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | U2U relay discovery with model A is intended to provide information of other end UEs in proximity via the U2U relay to the monitoring End UE. It is based on the information “user info ID of other UEs in proximity” in U2U relay discovery announcement message and it is assumed that the information are acquired during U2U relay discovery with model B procedure or during U2U relay communication procedure. The assumption is not compliant to security requirement from SA3 which says direct discovery set in U2U relay discovery message is protected end-to-end between End UEs and it shall be transparent to U2U relay. In TS 33.503 subclause 6.1.3.3.3, security procedure for 5G ProSE UE-to-UE Relay Discovery with Model A provides mechanism to provide the user info of other UEs via U2U relay in compliance with above security requirement. Therefore, as alignment with security requirements from SA3, it is proposed to update U2U relay discovery with model A based on TS33.503. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update U2U relay discovery with model A procedure to be aligned with SA3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | U2U Relay discovery procedure does not satisfy the securtiy requirement from SA3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.3.2.4.2	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model A

Depicted in Figure 6.3.2.4.2-1 is the procedure for 5G ProSe UE-to-UE Discovery with Model A.

Figure 6.3.2.4.2-1: 5G ProSe UE-to-UE Relay Discovery with Model A

- The 5G ProSe End UE may send a 5G ProSe UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message from the announcing 5G ProSe End UE include RSC and Direct discovery set. The UE-to-UE Relay Discovery Announcement message from 5G ProSe End UE is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.1.

2.	The 5G ProSe UE-to-UE Relay has discovered other UEs in proximity (e.g. via a previous 5G ProSe UE-to-UE Relay Discovery with model A (as shown in step 1), via a previous 5G ProSe UE-to-UE Relay Discovery with model B (as shown in clause 6.3.2.4.3) or via 5G ProSe U2U Relay Communication). The 5G ProSe UE-to-UE Relay obtains the Direct discovery set from other UEs in proximity per RSC.

NOTE:	For connected 5G ProSe End UEs, 5G ProSe UE-to-UE Relay obtains the Direct discovery set via the secure PC5 unicast link established with the 5G ProSe End UE.

3.	The 5G ProSe UE-to-UE Relay sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains the Type of Discovery Message, User Info ID of the 5G ProSe UE-to-UE Relay, RSC and list of Direct discovery set received from the 5G ProSe End UEs supporting the RSC. The UE-to-UE Relay Discovery Announcement message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

The 5G ProSe UE-to-UE Relay shall only announce User Info IDs of other UEs in proximity that did not include an Announce Prohibited Indication when they were previously discovered.

NOTE:	5G ProSe UE-to-UE Relay announces Direct discovery set from other UEs in proximity only if their PC5 signal strength measured by the 5G ProSe UE-to-UE Relay is above configured signal strength threshold as specified in TS 38.331 [16].

A 5G ProSe End UE monitors announcement messages from a 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

# * * * * End of change * * * *