3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401219r07

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon, OPPO, China Mobile, Vivo?, LG Electronics?, TOYOTA MOTOR CORPORATION?, KDDI?, NEC, ZTE?, Lenovo?, Nokia?, Nokia Shanghai Bell?, Samsung?

Title:	KI#3: New solution: supporting energy aware NF selection

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This document proposes enhancement to support energy aware NF selection addressing KI#3 5GS enhancements for network energy saving and efficiency.

1. Introduction

1.1	Background

KI#3 proposes to study:

“‐	Whether and how to enhance the existing operations and procedures (e.g. to degrade the QoS profile within an SLA agreement, or to transmit the data flows in lower bandwidth), including

-	Whether and how to enhance the NF selection/re-selection related functionalities considering energy saving and energy efficiency based on e.g., NF energy states (e.g., coordination with the OAM in control plane procedures), analytics, UE subscription, capability of NFs, energy related information;”

Current NF selection defined in clause 6.3 of TS 23.501 mainly considers criteria such as Location information, Data Network Name, slicing information, capability, local policy, load condition etc. There is no considerations on energy related criteria, such as maximize the energy efficiency of the network, trade the performance with energy consumption following the SLA agreement, etc., and may lead to a sub-optimal result from the energy perspective.

1.2	Enhancements regarding energy aware NF selection

It is proposed that NRF maintains also energy related information (e.g., configured by OAM in NRF or registered by each individual NF) as part of NF profile for registered NF instance. Such energy related information includes:

-	Renewable Energy Ratio/Carbon emission factor: Amount of carbon emissions relative to an amount of resource consumption (e.g., carbon emissions / GB transmitted, carbon emissions / FLOPS, carbon emissions / GB stored).

-	Normalized Total energy: Amount of energy consumption relative to an amount of resource consumption (e.g., energy consumption / GB transmitted, energy consumption / FLOPS, energy consumption / GB stored).

NRF may returns energy related information as part of the NF profile in response to a NF discovery request. For the above additional information, NRF discovers a NF service considering also energy aware NF discovery policy. The energy aware NF discovery policy includes a score Metric for different information elements in the NF profile for the NF selection; or a network mode (e.g., low energy mode, high performance mode, green energy mode, etc.) which is mapped to a certain energy aware NF discovery policy. NRF evaluates the discovered NF instances based on energy aware NF discovery policy (i.e., sum up the scores of each evaluated information element of a candidate NF instance, prioritizes the NF instance(s) with higher final score, and feedback to the related service consumer).

In addition, NF profile can also be enhanced to guarantee a flexible NF selection regarding energy. The PDU Session Anchor is not expected to be changed during the lifetime of the PDU Session for SSC mode 1. However, energy related executions may require the NF to be switched to "inactive"/"energy saving" or other state to save energy or improve energy efficiency, which could result in the unavailability of the UPF. Therefore, it may negatively affect the flexibility of switching off the UPFs for energy saving, if more UPFs are managing PDU session with SSC mode1. Those UPFs should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1. Therefore, the energy related information also includes:

-	Indication of potential Energy execution: Indication that the whether the NF is subject to energy related execution present or future (e.g., deactivated/turned off because of the energy consumption in the off-peak hours), and it may also include the possibility (or the priority) to be subject to energy related execution, and the interval of the energy related execution.

This solution proposes to enhance the NF discovery and selection related functionalities for energy aware NF selection.

This paper also merges the proposal from S2-2400080, S2-2400085, S2-2400234, S2-2400541, S2-2400728, S2-2400757, S2-2400839, S2-2400931, S2-2401174, and S2-2401280.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| X |  |  | X |  |
| … |  |  |  |  |

# * * * * Second change * * * * (all new)

6.x	Solution #X: Energy aware NF selection

6.x.1	Key Issue mapping

This solution addresses Key Issue #3.

6.x.2	Functional Description

This solution proposes to enhance the NF selection related functionalities for energy aware NF selection.

NRF maintains also energy related information (e.g., configured by OAM) as part of NF profile for registered NF instance. Such energy related information includes:

Renewable Energy Ratio: Percentage of Energy Consumption using renewable sources in the total Energy Consumption using all sources.

Carbon emission factor: Amount of carbon emissions relative to an amount of resource consumption or useful output(e.g., carbon emissions / GB transmitted, carbon emissions / FLOPS, carbon emissions / GB stored).

Normalized Total energy: Amount of energy consumption relative to an amount of resource consumption or useful output (e.g., energy consumption / GB transmitted, energy consumption / FLOPS, energy consumption / GB stored).

Table 6.x.2-1: Additional Energy Information included in the NF profiles.

| Information element name |  | Description |  | Category |  |
| --- | --- | --- | --- | --- | --- |
| Renewable Energy Ratio |  | Percentage of Energy Consumption using renewable sources in the total Energy Consumption using all sources |  | Optional |  |
| Energy Type information |  | Indicating energy type of the NF, e.g. renewable energy, non-renewable energy. |  | Optional |  |
| Carbon emission factor |  | Indicating amount of carbon emissions relative to an amount of resource consumption or useful output (NOTE 1, 2) |  | Optional |  |
| Carbon Efficiency information |  | Indicate relative carbon efficiency level of the NF, e.g. an indication with Low, Medium, High, or a level value with 1, 2, 3, 4, 5. |  | Optional |  |
| Normalized Total energy |  | Amount of energy consumption relative to an amount of resource consumption (NOTE 1). |  | Optional |  |
| Energy States |  | Energy state as specified in TS 28.310 [7], e.g., notEnergySaving state, energySaving state. |  | Optional |  |
| Energy execution indication |  | Indicating whether the NF is subject to energy related execution. |  | Optional |  |
| -Possibility |  | The possibility to be subject to energy related execution. |  | Optional |  |
| -Priority |  | The priority to be subject to energy related execution. |  | Optional |  |
| -Time interval |  | The interval of the energy related execution. |  | Optional |  |
| NOTE 1: amount of resource consumption can be GB transmitted, GB stored, or FLOPs or summary of them per NF instance NOTE 2: Renewable Energy Ratio/Carbon emission factor KPI can be further adjusted/defined based on related study in SA5 |  |  |  |  |  |

Editor's note: It is FFS that whether the EE information will be included in the Additional Energy Information.

NRF may returns energy related information as part of the NF profile in response to a NF discovery request. When receiving a request to discover a NF service, NRF considers also local policy for energy aware NF discovery, and provides the NF information accordingly. .

NOTE:	The local policy in the NRF for energy aware NF discovery can include a score metric for different information elements in the NF profile, or an energy related network operation mode (e.g., low energy mode, high performance mode, green energy mode, etc.). NRF can evaluate the discovered NF instances based on the local policy (e.g., sum up the scores of each evaluated information element of a candidate NF instance), prioritize the NF instance(s) accordingly, and feed back the NF information to the service consumer.

Table 6.x.2-2: Example NF discovery policy (score per information/Metric) in low energy mode

| Information/Metric | Score |
| --- | --- |
| Service Capacity [0..100] | 0 |
| Renewable Energy Ratio/Carbon emission factor [0..100] | 0 |
| Normalized Total Energy [0..100] | 1 |

NF service consumer may take into account energy related information (e.g. Energy States) from the NF profiles discovered from NRF using discovery procedure (and subsequent updates if subscribed to), for selection of a 5GC NF.

Editor's note: It is FFS whether additional means is needed for provisioning the above-mentioned information to the NF service consumer.

The NF service consumer may further check with NWDAF for better NF selection choice, and the details will be addressed in other solutions.

And when SMF selects the UPF, the SMF can also make use of the NF Profiles by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

6.x.3	Procedures

6.x.3.1	General

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The NRF service procedure as defined in clause 4.17 of TS 23.502 [3] are reused for energy related NF management and discovery.

The registered NF profile includes additional NF energy related information as described in clause 6.X.2. A service provider NF or OAM registers the enhanced NF profile into the NRF.

An NF service consumer may include an NF discovery policy, such as score scheme, network mode in the NFDiscovery_Request to the NRF. Alternatively, OAM may configure the NRF with different NF discovery policy, e.g., per subscription, per network mode.

For UPF, the NF energy related information included in the NF profile can also be provisioned to SMF via N4 related procedures as defined in clause 4.4 of TS 23.502 [3].

6.x.3.2	Service discovery considering NF energy related information

Figure 6.x.3.2-1: Service discovery considering NF energy related information

0a. Service provider NF or OAM registers the NF profile with additional energy related information into the NRF.

0b. NRF may be configured by OAM with a set of NF discovery policy (e.g. per subscription/network mode). Each NF discovery policy includes a score Metric for the to be evaluated information (defined in clause 6.x.2) in the NF profile. The following additions apply compared to clause 4.17.4 of TS 23.502 [3]:

1.	(Optional) The NF service consumer optionally indicates energy related criteria (e.g., network operation mode and/or energy efficiency mode) when invoking Nnrf_NFDiscovery_Request.

2.	The NRF authorizes the Nnrf_NFDiscovery_Request as in clause 4.17.4 of TS 23.502 [3].

3.	If allowed, the NRF determines a set of NF instance(s) matching the Nnrf_NFDiscovery_Request and local policies of the NRF and sends the NF profile(s) of the determined NF instances.

If Nnrf_NFDiscovery_Request includes energy related criteria, or based on local policy for energy aware NF discovery, NRF prioritizes the determined NF instances accordingly.

If allowed, the NRF selects the NF instance(s) from the prioritized NF instances. The NRF indicates the priority of the determined NF instances, and/or the selected NF instance(s) to the consumer NF in the Nnrf_NFDiscovery_Response.

The NF profile(s) of the determined NF instances may include additional energy related information in the NF profile (e.g., Renewable Energy Ratio/Carbon emission factor, Energy Type information, Carbon Efficiency information, Normalized Total Energy, Energy states (e.g. per S-NSSAI), or scores of the NF based on the energy related criteria or local policy). If the Energy Information of the alternative NF instance(s) is not available, the NRF obtains the related Energy Information from OAM or EECF.

4.	The NF service consumer selects the NF instance based on the NF profile information received from the NRF. If the NRF has selected the NF instance(s), the NF service consumer may skip this step.

6.x.3.3	Selection of the UPF with considering energy related information

The information to be considered for UPF selection are defined in clause 6.3.3 of 3GPP TS 23.501 [2]. When SMF selects the UPF, the SMF can also make use of the NF Profiles by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

Figure 6.x.3.3-1: Selection of the UPF with considering energy related information

0.	SMF provisioning of UPF instances using NRF, as described in clause 6.x.3.2.

1.	The UE registers to the PLMN, as described in clause 4.2.2.2 of TS 23.502 [3].

2.	The PCF makes use of UE Configuration Update procedure to provide UE Policy as defined in clause 4.2.4.3 of TS 23.502 [3].

3.	UE initiates the UE Requested PDU Session Establishment procedure with including the necessary information, including the requested SSC mode.

4.	Upon UPF selection, the SMF selects the UPF based on its Additional Energy Information  in the NF profile and the additional factors related to Additional Energy Information (Energy Efficiency and Energy saving parameters e.g. NF energy states, analytics, Energy Cost etc.). If the selected SSC mode of the PDU session is SSC mode 1, the SMF selects the UPF that is not subject to energy related execution, or selects the UPF with lower possibility/priority subject to energy related execution.

NOTE Z: These additional factors and NWDAF based related analytics will be determined within the scope of Key Issue 1 (e.g. types of energy related information, how such an information can be exposed, granularity etc.) and Key issue 3 related analytics solutions and referenced from this solution.

6.x.3.4	Selection of the AMF considering energy related information

For AMF selection. the influencing factors are defined in the 3GPP TS 23.501 [2], cause 6.3.5.

On top of this, the additional factors related to Additional Energy Information (Energy Efficiency and Energy saving parameters e.g. NF energy states, analytics, Energy Cost etc.), may be taken into account while making AMF selection decision.

NOTE X: These additional factors and NWDAF based related analytics will be determined within the scope of Key Issue 1 (e.g. types of energy related information, how such an information can be exposed, granularity etc.) and Key issue 3 related analytics solutions and referenced from this solution.

6.x.3.5	Selection of the SMF considering energy related information

It is assumed that the SMF had previously registered its NF profile including NF Energy Consumption into NRF, before SMF selection procedure.

Figure 6.X.3.5-1: SMF selection procedure

NOTE 1:	This procedure is based on Figure 4.3.2.2.3.2-1of TS 23.502 [3].

If the S-NSSAI is subject to energy control, AMF discovers SMF via NRF with the S-NSSAI.

Step 4: The NRF in serving PLMN provides to the AMF, e.g. FQDN or IP address, of a set of the discovered SMF instance(s) or Endpoint Address(es) of SMF service instance(s) in Nnrf_NFDiscovery_Request response message and possibly an NSI ID for the selected Network Slice instance corresponding to the S-NSSAI for subsequent NRF queries.

During this step, Energy Information (Energy Efficiency and Energy saving parameters e.g. NF energy states (e.g. per S-NSSAI), analytics, Energy Cost etc.) may be considered for the SMF selection and in addition, the other related analytics information provided by the NWDAF may be considered together.

Finally, NF profiles returned to AMF may include NF Energy Consumption as well as NF load information, NF capacity information, etc. as defined in TS 23.502 [3].

NOTE Y: These additional factors and NWDAF based related analytics will be determined within the scope of Key Issue 1 (e.g. types of energy related information, how such an information can be exposed, granularity etc.) and Key issue 3 related analytics solutions and referenced from this solution.

6.x.4	Impacts on existing services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

NRF:

-	Enhanced service discovery considering also energy related information and related NF discovery policy.

-	(optional) Feedback service consumer with the priority of the discovered NF instance.

-	(optional) Feedback service consumer with energy related information in the NF profile.

-	(In case of SMF selecting UPF) The NRF provides SMF the NF profiles of the UPFs including additional Energy Information.

Service consumer NF:

-	Taking consideration of energy related information for 5GC NF selection.

-	(optional) includes energy related criteria (e.g., network operation mode and/or energy efficiency mode) in the service discovery request.

SMF:

-	SMF selects the UPF by considering the energy related information, e.g., SMF selects the UPF that is not subject to energy related execution, or selects the UPF with lower possibility/priority subject to energy related execution if the selected SSC mode is SSC mode 1 of the PDU session.

OAM:

-	Configure NRF with energy related information in the NF profile.

-  Configure NRF with energy related NF discovery policy (e.g., score Metric)

Editor's note:	It is FFS whether the OAM can provision energy related information in the NF profile and energy related NF discovery policy to the NRF.

# * * * * End of changes * * * *