SA WG2 Meeting #160-AH-e	S2-2401319r07

January 22 - 29, 2024, Electronic Meeting

Source:	Apple, ETRI

Title:	FS_MASSS Architectural Terms

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes Terms for DualSteer for the FS_MASSS TR 23.700-54.

# 1	Discussion

This paper proposes text for the terms section of the FS_MASSS TR 23.700-54.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

DualSteer UE: A UE supporting DualSteer traffic steering and switching of user data (for different services) across two 3GPP access networks; A DualSteer device can be a single UE stack in case of non-simultaneous data transmission over the two 3GPP access networks or can be two separate UE stacks in case of simultaneous data transmission over the two 3GPP access networks. Each UE stack registers with the corresponding 3GPP access network as a separate UE.

DualSteer traffic steering: the procedure that a DualSteer UE selects a 3GPP access network for a new service and transfers traffic of that service over the selected access network. This may include scenarios where all services use the same access network connection (no simultaneous transmission over the two 3GPP access networks) or different services are steered across different 3GPP access networks (with simultaneous transmission over the two 3GPP access networks).

DualSteer traffic switching: the procedure that a DualSteer UE moves all traffic of a service from one 3GPP access network to another 3GPP access network in a way that minimizes service interruption. This may include scenarios where traffic of all services are switched from one 3GPP access network to another 3GPP access network (no simultaneous transmission over the two 3GPP access networks) or traffic subject to traffic switching are switched from one 3GPP access network to another 3GPP access network (with simultaneous transmission over the two 3GPP access networks).

* * * * End of Changes * * * *