SA WG2 Meeting #160-AH-e	S2-2401319r11

January 22 - 29, 2024, Electronic Meeting

Source:	Apple, ETRI

Title:	FS_MASSS Architectural Terms

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes Terms for DualSteer for the FS_MASSS TR 23.700-54.

# 1	Discussion

This paper proposes text for the terms section of the FS_MASSS TR 23.700-54.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

DualSteer Device: the term DualSteer Device is introduced in TS 22.261[X] for Stage 1.

NOTE: The Stage 2 implications to UE-side architecture will be studied during Solutions discussions.

DualSteer traffic steering: the procedure for selecting a 3GPP access network for a new service and transfers traffic of that service over the selected access network. This may include scenarios where all services use the same access network connection (no simultaneous transmission over the two 3GPP access networks) or different services are steered across different 3GPP access networks (with simultaneous transmission over the two 3GPP access networks).

DualSteer traffic switching: the procedure for moving all traffic of a service from one 3GPP access network to another 3GPP access network in a way that minimizes service interruption. This may include scenarios where all traffic of all services are switched from one 3GPP access network to another 3GPP access network (no simultaneous transmission over the two 3GPP access networks) or all traffic for a service subject to traffic switching are switched from one 3GPP access network to another 3GPP access network (with simultaneous transmission over the two 3GPP access networks).

* * * * End of Changes * * * *