SA WG2 Meeting #S2-160AH-e	S2-2400008

22 - 29 January, 2024, Electronic meeting

SA WG2 Meeting #S2-160	S2-2311950

13 - 17 November, 2023, Chicago, USA

3GPP TSG-CT WG1 Meeting #144	C1-237896

# Xiamen, China, 09– 13 October 2023

# Title:	Reply LS on provisioning list of tuples to UE

# Response to:	LS on provisioning list of tuples to UE (S2-2308086)

# Release:	Rel-18

# Work Item:	eUEPO

Source:	CT1

To:	SA2

Contact Person:

Name:	Ivo Sedlacek

E-mail Address:	ivo dot sedlacek at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

CT1 would like to thank SA2 for LS S2-2308086 on provisioning list of tuples to UE.

LS S2-2308086 states:

-----------

ACTION: 	SA2 kindly asks 3GPP CT WG1 to notify SA2 if the procedure to send the list of tuples to the UE can be further optimised.

-----------

CT1 would like to inform SA2 that the procedure to send the list of tuples to the UE can be further optimised.

CT1 would like to inform SA2 that apart from providing a full list of tuples, TS 24.501 also enables the PCF to provide a partial list of tuples. For partial list of tuples, the UE:

-	adds, updates or removes the tuples of the stored VPS URSP configuration, tuple IDs of which are indicated in the VPS URSP configuration UE; and

-	keeps the tuples of the stored VPS URSP configuration, tuple IDs of which are not indicated in the VPS URSP configuration UE.

2. Actions:

To SA2 group.

ACTION: 	CT1 would like to ask SA2 to take the information above into consideration.

3. Date of Next CT1 Meetings:

CT1#145	13th   – 17th November 2023 		Chicago