3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2300088

Online, Jan 22 – 29, 2024	(revision of S2-231xxxx)

# Title:	5GC Enhancements to Support of Store and Forward Satellite operation

# Source:	China Mobile

# Document for:	Approval

Agenda Item:	19.1

Abstract: Propose a new solution to study the enhancements to support store and forward satellite operation for KI #2.

1 Discussion

The Store and Forward (S&F) Satellite operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service.

To support the S&F operation, there could be two deployment options.

Option 1: User plane enabled S&F. Only user plane function is on-board, i.e., the UPF and optionally the EAS could be onboard. User plane on board enabled S&F can support the rapid implementation of delay-insensitive services (CIoT, etc.), and has low requirements for onboard resources. In addition, with the on-board EAS, some service could be provided on the satellite, which could save the feeder link resource, and reduce the transmission delay. However, as discussed below, this option may have dependence on (R)AN capabilities.

In this case, A 5G system with satellite access supporting S&F Satellite operation shall support mechanisms for a UE to register with the network when the network is in S&F Satellite operation. Therefore, a S&F registration solution is proposed.

Option 1: User plane enabled S&F								Option 2: 5GC enabled S&F

Option 2: 5GC enabled S&F. Both user plane function and part of the control plane are on-board, i.e., the UPF, AMF, SMF, UDM/AUSF and optionally the EAS could be onboard. 5GC enabled S&F has requirements for onboard resources, but it does not has dependence on (R)AN capabilities, and control signaling exchange on the satellite can save feeder link resources.

In this case, most 5GC service could be provided on-board with minor enhancement. The SMF are supposed to have the S&F policy about the S&F operation. Detailed enhancement is needed.

2 Proposal

It is proposed to capture the following changes in TR 23.700-29.

FIRST CHANGE

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  | Key Issues#2 |  |  |
| Solution #X: Enhancement of registration and PDU session establishment to support on-board S&F |  | X |  |  |

NEXT CHANGE(all new texts)

6.X	Solution #X: Enhancement of registration and PDU session establishment to support on-board S&F

6.X.1	Description

The Store and Forward (S&F)  Satellite operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service.

To support the S&F operation, there could be two deployment options.

Option 1: User plane enabled S&F. Only user plane function is on-board, i.e., the UPF and optionally the EAS could be onboard. User plane on board enabled S&F can support the rapid implementation of delay-insensitive services (CIoT, etc.), and has low requirements for onboard resources. In addition, with the on-board EAS, some service could be provided on the satellite, which could save the feeder link resource, and reduce the transmission delay.

In this case, A 5G system with satellite access supporting S&F Satellite operation shall support mechanisms for a UE to register with the network when the network is in S&F Satellite operation. A S&F registration solution is proposed and will be elaborated in the following clause.

UE

5GC Control Plane

RAN1

UPF1

EAS(s)

DN

PSA-UPF

Figure 6.X.1-1: Architecture assumption for User plane enabled S&F

Option 2: 5GC enabled S&F. Both user plane function and part of the control plane are on-board, i.e., the UPF, AMF, SMF, UDM/AUSF and optionally the EAS could be onboard. 5GC enabled S&F has requirements for onboard resources, and control signaling exchange on the satellite can save feeder link resources.

In this case, most 5GC service could be provided on-board with minor enhancement. The SMF is supposed to have the S&F policy about the S&F operation. Detailed enhancement will be elaborated in the following clause.

UE

SMF

PSA UPF/DN

AMF

RAN1

UPF

EAS(s)

DN

UDM/AUSF

Figure 6.X.1-2: Architecture assumption for 5GC enabled S&F

6.X.2	Procedures

6.X.2.1	Procedures of User plane enabled S&F

To  support mechanisms for a UE to register with the network when the network is in S&F Satellite operation, this solution proposes the UE request the registration and PDU session establishment into single procedure. Following enhancement are supposed to be supported by (R)AN.

- When the S&F operations are supported and indicated in the UE request, (R)AN shall store the received UE registration and PDU session establishment request.

- Trigger the the feeder link/service link establishment when they are available. Upon the feeder link/service link is available, send out the stored UE registration and PDU session establishment request.

6.X.2.1.1	Procedures of registration in S&F mode

Figure 6.X.2.1.1-1: Procedures of registration in S&F mode

1.	UE sends the registration and PDU session establishment request to (R)AN.

In addition to what has been defined in TS 23.502 [xxx] that can be sent by the UE in the Registration Request and PDU Session Establishment request, the following parameters should also be provided by the UE, including the indicator of the support of the S&F registration.

The indicator of the S&F registration indicates the UE wants to perform a S&F registration, and the PDU session establishment would be initiated in the same time.

Optionally, the uplink data could be provided in the NAS message with re-utilizing the CIoT CP optimization.

2.	When the UE provides the S&F registration request, and the (R)AN could cover this area when the service link is available again based on the ephemeris information or based on the pre-configuration,  the (R)AN stores the UE request received. The uplink data would be stored as well in (R)AN as well, if CIoT CP optimization is requested.

3.	The (R)AN sends a S&F registration notification to the UE. The notification would inform the UE the registration is in the S&F mode, indicating the registration response will be delayed. If CIoT CP optimization is requested, the response also indicates the result of the data storage.

4.	Based on the ephemeris information, or the pre-configuration, (R)AN sends out the stored request upon the feeder 	link is available. If CIoT CP optimization is requested, the data is sent upon the feeder link is available.

5.	Complete the registration procedures and PDU session establishment procedures on the ground as defined in the TS 23.502 clause 4.2.2.2 step 2-19, and clause 4.3.2.2 step 2-12.

With the indication of the S&F registration, the AMF is aware that the UE is in S&F mode, then the AMF may, based on configuration, send the satellite ID to the SMF. The SMF selects on-board UPFs based on satellite ID, ephemeris information. If CIoT CP optimization is requested, the procedures follow what been defined in TS 23.502 clause 4.24. The SMF activates the on-board UPF buffering in the N4 session signaling.

Editor’ s note: How the SMF selects on-board UPFs based on satellite ID, ephemeris information is FFS.

- The AMF sends the registration accept and PDU Session Establishment Accept to the (R)AN. The (R)AN stores the information received from AMF that will send to the UE. The (R)AN stores the UE context and acknowledges to the AMF.

- When the service link is available, the (R)AN send out the stored Registration Accept information and PDU Session Establishment Accept information to UE. And (R)AN forwards the NAS message (PDU Session ID, N1 SM container (PDU Session Establishment Accept)) provided in step 12 to the UE.

- Optionally, the N2 PDU session response will be stored in (R)AN and the Uplink Data will be stored in the on-board UPF, wait for the next service link available time.

Editor’s note: how to update the timer to support the above procedure is FFS.

6.X.2.2	Procedures of 5GC enabled S&F

6.X.2.2.1	Enhancement of the registration procedures

- When the AMF, SMF, UDM/AUSF are all on-boarded, the registration procedures follow Registration procedures defined in TS 23.502, clause 4.2.2.2. The differences are:

-	The step 16 could be skipped, since the AM policy could be predefined.

-	After the on-board registration completion, the on-board UDM may send the stored UE subscribe data to the ground UDM for backup.

-	The capability of  NF(s) supporting the S&F mode need to be indicated；

Editor’s note: how the subscription data synchronization among the UDMs in the satellite and ground is achieved is FFS.

6.X.2.2.1	Enhancement of the session management procedures

- When the AMF, SMF, UDM/AUSF are all on-boarded, the registration procedures follow Registration procedures defined in TS 23.502, clause 4.2.2.2. The differences are:

-	The step 7 and/or step 9 could be skipped, since the SM policy could be predefined.

-	The capability of NF supporting the S&F need to be indicated；

6.X.3	Impacts to Services, Entities and Interfaces

- User plane on-board:

SMF: selection of UPF based on the satellite ID and ephemeris information.

- AN: Store the request from the UE in S&F mode.

The timers related to registration procedure and PDU session establishment procedure may need to be updated.

- Control plane and user plane on-board:

- UDM: subscription data synchronization

The timers related to registration procedure and PDU session establishment procedure may need to be updated.

End of CHANGES