SA WG2 Meeting #160-AH-e	S2-2400129

# January 22 – 29, 2024, Elbonia

Source: 	NTT DOCOMO

Title: 	KI#3: New Sol: 5G Core Energy Saving

Document for: 	Approval

Agenda Item: 	19.4 (FS_EnergySys)

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes a solution for KI#3 where user plane traffic paths in the 5GC are adjusted based on the operator’s energy policy.

# 1	Discussion

Energy consumption of the 5G core (and consequently, the related metrics such as carbon emission) is influenced not only by the control plane signallings but also the user plan traffic handling. Energy consumption of the user plane of the 5G core is significant due to the huge amount of the traffic.

In the real-world deployments, the UP of the 5GC is not just a single UPF. For several reason, as depicted in the Figure 1, more than UPF may be deployed in the operators’ network such as

- Handling the peak traffic (which is beyond the capacity of a single UPF)

- Deploying multiple Intermediate UPFs (I-UPF) for reliability objectives

- Deploying multiple Up Link Classifier UPFs (UL CL UPF) for traffic steering to local networks

- Deploying UPF instances for a specific functionality, e.g., a dedicated UPF for Lawful Interception

Figure 1. Scenarios of multiple UPF deployments

Therefore, in real deployments, user plane traffic may be handled by a chain of UPFs interconnected via N9 interface, i.e., it is possible to have a path for each PDU session in the 5GC.

On the other hands, different UPFs may have different characteristics from energy perspective; e.g., some of the UPFs maybe powered on (partially) by renewable energy sources, some of them could be ultra energy efficient; and the operator may have various energy related policies e.g., maximizing energy efficiency or minimize carbon emission.

Putting these observations together, it is possible to adjust UP paths in the 5GC to implement the operator’s energy policy. This paper proposes a solution for this purpose where a new Energy Manger Function drives UP paths based on the operator’s policy, configures them in the SMF and activates them by requesting from the PCF.

# 2 Proposal

It is proposed to update TR 23.700-66 according to the following text.

********** First Change (All new text) **********

## 6.x	Solution #X: Energy Aware User Plane Path Adjustment

### 6.x.1	Key Issue mapping

The proposed solution maps to the following aspect of KI#3:

‐	Whether and how to enhance the existing operations and procedures to satisfy the energy saving and energy efficiency requirements.

### 6.x.2	Functional Description

SMF is responsible for managing UPFs in the 5G via N4 interface considering the PPC rules provided by the PCF. PCC rules composed of various fields that not only determined by the operator policy and charging but also can be influenced by AF such as “N6-LAN Traffic Steering Enforcement Control”. Following a similar concept, this paper proposes a solution to adjust the UP path of PDU sessions based on operator’s policy on energy. The overall procedure is as follows.

It is assumed that the operator configures the energy policies in the OAM which is accessible to other NFs as well. Moreover, it is assumed that OAM can provide energy related information (e.g., energy consumption, carbon emission, etc.) of NFs.

A new network function named Energy Manager Function (EMF) in collaboration with SMF and PCF implements the operator’s policy on energy management with the following steps.

At the first step, EMF collects three categories of information. It gets the energy related information of UPFs deployed in the network from the OAM. It obtains the service UPFs from the SMF and accordingly constructs the path of PDU sessions on the N9 interface. Moreover, it gets the operator’s policy on energy from the OAM.

In the second step, using the information obtained in the first step, EMF derive optimized UP paths for the PDU sessions. These UP paths are computed in such a way that steering N9 traffic through the paths optimizes the operator’s energy goal.

NOTE 1: Whether all or subset of PDU sessions are considered for the UP path adjustment and the solution to compute paths to optimize the operator’s policy are the internal logic of the EMF.

Currently it is possible to configure N6 traffic steering policies in the SMF, following a similar concept, in the third step, EMF configure the N9 traffic steering paths in SMF.

In the fourth step, similar to the AF influence on traffic routing, EMF asks PCF to update PCC rules according to the optimized N9 traffic steering policies. Then PCF, based on this request and considering other configurations, derives PCC rules including the N9 traffic steering information and shares them with the SMF. Then SMF implements these policies via proper configuration of the deployed UPFs.

### 6.x.3	Procedures

The procedure depicted in Figure 6.x.3-1 describes how the UP paths are adjusted based on the operator’s policy.

Figure 6.x.3-1: Energy-aware user plane path adjustment

- The EMF subscribes to the operator’s energy policy configured in the OAM.

- The EMF subscribes to the energy-related information of the UPFs collected by OAM

Editor's note: Whether and how to collect information related to carbon emissions and renewable energy of NFs are FFS.

3a. The EMF subscribes to the NF load information stored in NRF.

3b. The EMF may optionally subscribe to some NWDAF analytics (e.g., NF load analytics and energy analytics)

- The EMF receives a trigger (e.g., expiration of a periodic timer or a notify of the subscribed information in steps 1-3) to adjust UP paths.

- The EMF gets the information of PDU sessions and their corresponding UPFs. Based on these information, EMF construct the path of (a subset of) PDU sessions, i.e., a chain of UPFs interconnected via N9 interfaces.

- Based on the information that the EMF collected in steps 1-5, it adjusts the UP paths in 5GC in such a way that the steering N9 traffic through the paths optimizes the operator objective on energy. In this procedure, it may optionally consider the future condition of the network via the analytics provided by NWDAF. And then performs steps 7-11 for each UP path.

- The EMF configures the UP paths via Nsmf_N9TrafficSteeringPolicy service operation where a unique ID is assigned to each path as N9 Traffic Steering policy ID.

- The EMF determines a set of PDU sessions needs to be steered through this path. Then via Npcf_TrafficPathInfluence asks from PCF to derive PCC rules for this set of PDU sessions (specified by the traffic description input parameter of this service operation) to use the specified UP path (specified by the N9 Traffic Steering Policy reference input parameter of this service operation).

- PCF takes the EMF’s request into account besides other parameters influence N9 traffic steering (e.g., delay) and drives the corresponding PCC rules.

- PCF sends the PCC rules via Npcf_SMPolicyControl_UpdateNotify to the SMF.

- SMF configures the UPFs through N4 interface to implement the N9 traffic steering policy specified in the PCC rule. In the case of changing the PSA UPF of a PDU session, procedures described in clause 4.3.5 of TS 23.502 are used to update the PDU session.

### 6.x.4	Impacts on existing services, entities and interfaces

The solution has the following impacts:

EMF:

-	New network functionalities to collect energy related information and policies.

-	New network functionalities to adjust UP paths according to the operator’s policy.

SMF:

-	Extending service to configure UP paths.

PCF:

-	Extending service to influence N9 traffic steering.

********** End of Changes **********