SA WG2 Meeting #S2-160-Ad Hoc-e	S2-2400148

22-29 January, 2024

# Title: Key issue on enhancement of LMF direct AI/ML positioning

# Source: ZTE

# Document for: Approval

Agenda Item: 19.15

Abstract:	This is a key issue proposal for WT#1.4 2b and 3b in R19 AIML_CN

1 Discussion

The key issue is related to WT#1.4 in R19 AIML_CN:

WT1.4: Study whether and how to consider enhancements to LCS to support AI/ML based Positioning considering the conclusions in 3GPP TR 38.843.

There are different cases to solve the problem, as defined by RAN:

- Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning:

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

START OF CHANGES

- x Key Issue #x: Enhancement of LMF direct AI/ML positioning

This is the key issue corresponds to case 2b and 3b in WID for NR_AIML_Air of RAN WGs, LMF direct positioning with LMF-side model assisted by UE or NG-RAN. In this key issue, the LMF-side model will output UE Location or a group of UE location(s), by using the measurement data or assistance information from UE or NG-RAN. To be specific, the following may include in the scope of this key issue:

-What data(e.g. training data, location measurement data and ground truth data)need to be collected for model training and whether enhancements of data collection for model is needed.

-Whether a new analytic ID of NWDAF is needed.

-How to enhance procedures of data exchange between LMF and NWDAF. For example, LMF provide the collected measurement data from UE and/or RAN and send them to NWDAF for model training. The NWDAF send the location information of UE/group of UE to LMF or other service consumer(s).

-Whether more model output performance indicators related to positioning are needed.

-How to compare the positioning accuracy with AI/ML and without AI/ML.

-When the system uses AI/ML positioning and when the system uses traditional positioning method.

- NOTE X: UE data collection for UE-side model training, model delivery/transfer to the UE and model identification/management between SA2 and RAN are not within the scope of is not in scope of this key issue.

- NOTE Y:  For the core network side, the model training,validation and testing or generation of statistical result, shall be done by NWDAF.

END OF CHANGES