SA WG2 Meeting #160E	S2-2400157

# January 22 – 30, 2024, Electronic Meeting            	       (revision of S2-240xxxx)

Source: 	ETRI

Title: 	FS_MASSS New KI for WT#1.2: Registration aspects for DualSteer

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this paper proposes a new key issue for WT#1.2 of FS_MASSS.

1	Discussion

The Work Task#1 of the FS_MASSS SID is:

“WT#1: Study the overall architecture and function enhancements to 5GS to support a DualSteer Device (see TS 22.261 for definition of DualSteer Device). A DualSteer Device supports traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be (a) a single UE, in case of non-simultaneous data transmission over the two networks, or (b) two separate UEs in case of simultaneous data transmission over the two networks. The subscriber of the DualSteer Device has two subscriptions/SUPIs, sharing one subscription profile from the same operator. For any particular service, at any given time, the DualSteer Device shall transmit all traffic of that service using only a single 3GPP access network.

The following scenarios are considered:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN or integrated with the VPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

NOTE 2: 	The study assumes there is no coordination in RAN between the two 3GPP access networks where the DualSteer Device is accessing simultaneously.

NOTE 3:	For the PNI-NPN scenario, the subscriber is assumed to be a subscriber of the PNI-NPN.

NOTE 4:	The 5GC-EPC scenarios will be studied once the baseline 5GC-5GC scenarios are stable.

NOTE 5:	Each subscription / SUPI of the DualSteer Device is used to connect to only one of the 3GPP access at any given point in time.

WT#1.1: Study whether and how to enhance subscription aspects for each of the above scenarios.

WT#1.2: Study whether there are any impacts to the registration procedure for DualSteer Devices.

NOTE 6:	This WT requires coordination with SA3 for handling security aspects.

NOTE 7:	No impacts on network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

NOTE 8:	If any PLMN selection enhancement is identified, SA2’s role could be limited to study system level impacts and/or trigger SA1 and CT1 to handle the work.

WT#1.3: Session management enhancements and policies:

-	Study whether and how to enhance network policies provided by the HPLMN to the DualSteer Device and within the network to support DualSteer Devices.

-	Study whether and how to enhance session management procedures for initial steering and potential subsequent switching. Traffic policies are in full control of the HPLMN. For sessions subject to potential switching, assuming data anchoring in the HPLMN or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN, study how to select the PSA UPF(s) in the HPLMN to allow routing the traffic across 3GPP accesses towards the same PSA UPF during the switching.

NOTE 9:	Impact to existing functionality related to the change of a service-related data between a 3GPP access network and a non-3GPP access network will be considered as part of WT#1.3.

”

Registration procedures from SUPIs of the DualSteer Device may be independent from each other. However, registration status of SUPIs is required to be managed together to support traffic steering and traffic switching.

2 Proposal

It is proposed to update TR 23.700-54 on FS_MASSS as follows:

* * * * Start of Change (All new text) * * * *

5	Key Issues

5.1	Key Issue for DualSteer

5.1.x	Key Issue #X: Registration aspects for DualSteer

5.1.X.1	Description

Registration procedures from SUPIs of the DualSteer Device may be independent from each other. However, registration status of SUPIs is required to be managed together to support traffic steering and switching. The key issue will study:

- Whether and what information is required to manage registrations of SUPIs of the DualSteer Device

- How to manage correspondence of registration of SUPIs of the DualSteer Device in a single PLMN.

- How to manage correspondence of registration of SUPIs of the DualSteer Device in different PLMNs.

* * * * End of Change * * * *