SA WG2 Meeting #160-Ad Hoc-e	S2-2400229

# January 22 – 29, 2024, Electronic Meeting                  (revision of S2-24xxxxx)

Source: 	ETRI

Title: 	Key Issue on WT#2: Direct subscription to UPF event exposure service and expose information extension

Document for: 	Approval

Agenda Item: 	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract of the contribution: this paper proposes a new key issue for FS_UPEAS_Ph2.

1	Discussion

WT#3: Study whether there are any potential enhancements on the UPF event exposure service that would optimize the procedures related to UPF data collection, e.g. direct/indirect subscription of the UPF via control plane from application.

NOTE Y: Solutions need to analyse benefits compared to existing mechanisms.

2 Proposal

It is proposed to adopt the following Key Issue in TR 23.700-63.

* * * * Start of Change(all new text) * * * *

5	Key Issues

5.x	Key Issue #x: Direct subscription to UPF event exposure service and expose information extension

5.X.1	Description

In Release 18, UPF enhancement for Exposure and SBA were studied, including UPF event exposure service(s) registration/deregistration and discovery via the NRF, UPF event exposure service enhancement etc. However, optimization of the procedure related to UPF data collection and additional exposure information from applications were not fully resolved.

The key issue is to identify the optimal procedure of the existing UPF exposure service for UPF data collection and to identify additional UPF expose information required from other NFs.

The following aspects should be studied to support UPF event expose service enhancement.

- How to optimize the existing UPF event exposure service procedure included indirect subscription to collect UPF data using direct subscription of the UPF from NFs.

- What additional specific information the UPF can expose to NFs , e.g. the information which can be exposed in Rel-19 SA WG2 studies such as FS_AIML_CN and FS_XRM_Ph2 , or information which has been justified for exposure.

* * * * End of Change * * * *