SA WG2 Meeting #S2-160AH-e	S2-2400289

22 - 29 January, 2024, Electronic meeting

Source:	Wiliot

Title:	FS_AmbientIoT architectural assumptions and requirements

Document for:	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT/Rel-19

Abstract of the contribution: This paper proposes architectural assumptions and requirements for FS_ AmbientIoT TR 23.700-13.

# 1	Discussion

This paper proposes FS_AmbientIoT architectural assumptions and requirements based on TR 22.840 and TR 38.848 content.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-13 v0.0.0.

* * * * Start of Changes * * * *

## 4.1	Architectural Assumptions

The architecture in this study should be based on the following assumptions:

- Ambient IoT devices are ultra-low complexity as well as ultra-low power, batteryless with harvesting energy capabilities (e.g. RF, Solar, etc.)

- An ambient IoT device’s memory can retain state for a limited time, due to memory retention energy cost, see S2-2400081 for technical analysis.

- 5GS Ambient architecture should assume the Ambient IoT is a SIM-less device due to power consumption and cost.

- Ambient IoT device communication availability depends on the device’s energy state: energized or non-energized.

- Ambient IoT devices shall have a single communication connection with 5G system

- 5G system and 3rd party entity have a trusted relationship.

- Ambient IoT device credentials (some or all) might be shared or exposed between 5GS and 3rd party

* * * * Next Changes * * * *

## 4.2	Architectural Requirements

The following architectural requirements can be applied to this study:

- The 3GPP shall support a single connection between Ambient IoT devices and 5G system, preference it will be an asymmetric stateless connection.

  - DL Control/Commands/Triggers

  - UL Data

- The 3GPP shall enable stateless or single-state communication between Ambient IoT devices and 5G systems due to Ambient IoT energy limitations.

- The 3GPP shall enable communication between a trusted 3rd party entity and an Ambient IoT device or group of devices

- The 3GPP shall enable the provision of Ambient IoT device credentials by the 5G System to a trusted 3rd party or by a trusted 3rd party to the 5G System

* * * * End of Changes * * * *