SA WG2 Meeting #160-Ad Hoc-e	S2-2400301

E-meeting, January 22 – 29, 2024	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1378 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network initiated IMS DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to LS from GSMA UPG (S2-2400034), in some cases, UE may not be pre-provisioned to initiate IMS Data Channel so the network-initiated Bootstrap DC setup capability is necessary for supporting those UEs. In some cases, e.g customer calsto retailer’s Sales Support or to the Customer Care number and service representative or automatic IVR wants to initiate interactive menu during the call. In currest spec, the Bootstrap DC setup procedure is initiated by the UE only. Therefore, the several additional steps to the IMS network to trigger the UE to initiate the Bootstrap DC setup procedure needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The new procedure is described with additional steps in Bootstrap Data Channel set up Signalling Procedure specified in AC.7.1 in TS 23.288. - The additional steps are the delivery of AF request of Bootstrap DC establishment to IMS AS, and the transport of SIP re-INVITE message which including SDP offer for Bootstrap DC from IMS AS to UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Bootstrap DC setup procedure cannot be initiated when the UE is not pre-provisioned when to initiate IMS Data Channel. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.1x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AC.7.1x	Network initiated Bootstrap Data Channel Setup Signalling procedure

Figure AC.7.1-x depicts a signalling flow diagram for network initiated bootstrap data channel procedure.

Figure AC.7.1-x: Network initiated Bootstrap Data Channel set up Signalling Procedure

- IMS session (audio call) has been established between UE#1 and UE#2.

- AF (UE#2 side) creates a new request. The request includes information needed for generation of Bootstrap DC SDP offer (e.g. Public User Identities of UE#1 and UE#2), and indication of DC Session request.

- AF sends its request to the NEF (UE#1 side).

  - NEF may need to authorize the AF request of DC Session with the HSS/UDM.

- NEF transfers the AF request to IMS AS.

- IMS AS validates user subscription data to determine whether the UE#1 supports IMS data channel. If IMS AS determined that the UE#1 supports IMS data channel, the IMS AS generates a SDP offer for Bootstrap DC based on the received information in step 3.

- IMS AS sends SIP re-INVITE which includes the SDP offer for Bootstrap DC between UE#1 and UE#2.

- UE#1 determines that an IMS Data Channel session needs to be established between UE#1 and UE2, when it receives the SDP offer for Bootstrap DC from the network. The UE#1 initiates Bootstrap DC setup procedure as specified in Figure AC.7.1-1 of TS 23.228, by using re-INVITE message.

Editor's note:	The AF/NEF service operations to signal the DC Session Request in step 1 to 3 is TBD.

Editor's note:	When and how the AF can trigger the DC Session Request in step 1 (e.g. based on request from the UE#2) will be clarified based on GSMA reply.

# * * * * End of changes * * * *