3GPP TSG SA2 Meeting #160AHE	S2-2400406

Elbonia, 22 – 29 January, 2024 	(revision of S2-240xxxx)

# Source:	Lenovo

# Title:	New WID on DNN insertion for Alternative Network Slice

# Document for:	Information

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	DNN insertion for Alternative Network Slice

{Free text. It has to be the same as in the "Title:" section above. Studies have to start by "Study on"}

Acronym:	TEI19_DNN2AltNS

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 980017 | Network Slicing Phase 3 (eNS_Ph3) | The eNS_Ph3 specifies the Network Slice Replacement feature. The current proposed TEI19 is an enhancement to the Network Slice Replacement feature. |

Dependency on non-3GPP (draft) specification:

# 3	Justification

In Rel-18 the Network Slice Replacement feature was introduced. This feature specifies that the AMF may determine (e.g. based on triggers by NSSF, PCF or OAM) that an S-NSSAI has to be replaced with Alternative S-NSSAI. The AMF either 1) uses the Alternative S-NSSAI received in the notification from the NSSF, PCF or OAM; or 2) if not provided in the notification, the AMF uses an Alternative S-NSSAI based on local configuration. The AMF sends the Alternative S-NSSAI, the associated replaced S-NSSAI and the DNN requested by the UE to the SMF during the PDU Session establishment or modification procedure.

The DNN requested by the UE is associated with the S-NSSAI to be replaced (e.g. according to the URSP rules in the UE), but the requested DNN may not be valid or supported in the Alternative S-NSSAI. In other words, the UE requested DNN applies for the replaced S-NSSAI, but the requested DNN may not apply for the Alternative S-NSSAI. Using the UE requested DNN together with the Alternative S-NSSAI as per Rel-18 specification may lead to the following mis-operations:

- the AMF uses the UE requested DNN to select an SMF in the Alternative S-NSSAI, however, if the Alternative S-NSSAI is not associated with such DNN, the inappropriate or wrong SMF may be selected.

For example, the UE requests DNN#A on the S-NSSAI to be replaced, but the Alternative S-NSSAI is configured to operate with DNN#1 and DNN#2, whereby one set of SMFs are configured for a DNN#1 and another set of SMFs are configured for a DNN#2. In such case, the AMF should first determine the appropriate DNN applicable for the PDU Session on the Alternative S-NSSAI and then the AMF can select the serving SMF for the PDU Session.

- the SMF uses the UE requested DNN to establish the PDU session and request SM policies from the SM-PCF, however, if the Alternative S-NSSAI is not associated with such DNN, the PCF may provide inappropriate or wrong SM policies to the SMF.

It is proposed to introduce a functionality for DNN selection for the Alternative S-NSSAI. Such functionality can be similar to the existing functionality of DNN replacement as described in TS23.501 clause 5.6.1 where the AMF may interact with the PCF to perform a DNN replacement. In case of Network Slice Replacement, the AMF may a) interact with the PCF to perform DNN selection for the Alternative S-NSSAI, or b) the AMF may select a DNN based on local policies. The AMF does not replace the UE requested DNN on the S-NSSAI to be replaced, but the AMF determines a selected DNN for the Alternative S-NSSAI.

# 4	Objective

The objective is to enhance the 5GS to enable the AMF to select a DNN, if needed, for the Alternative S-NSSAI. When selecting a DNN for the Alternative S-NSSAI, the AMF considers a) the UE requested DNN and b) the applicability to the Alternative S-NSSAI when to select DNN.

This work item will require 0.5 TU(s) to discuss and agree on the corresponding CRs.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Enhance existing clauses by adding AMF and PCF functionality to select DNN for Alternative S-NSSAI. | SA#106 (September 2024) | Target completion to be determined at SA2#161 or whenever approval takes place |
| 23.502 | Enhance existing clauses for by adding AMF and PCF functionality to select DNN for Alternative S-NSSAI. | SA#106 (September 2024) | Target completion to be determined at SA2#161 or whenever approval takes place |
| 23.503 | Enhance existing clauses by adding AM policy enhancements for DNN selection for Alternative S-NSSAI. | SA#106 (September 2024) | Target completion to be determined at SA2#161 or whenever approval takes place |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Impact to service-based interfaces to be handled by CT4 and/or CT3.

# 9	Supporting Individual Members

Supporting IM name

Lenovo