3GPP TSG-WG SA2 Meeting #160AHE	S2-2400407

Online, Jan 22 – 29, 2024

Source:	Huawei, HiSilicon

Title:	Terms, Architecture Requirements and Assumptions of Multi-hop U2N/U2U Relay for 5G_ProSe_Ph3

Document for:	Approval

Agenda Item:	19.7

Work Item / Release:	FS_5G_ProSe_Ph3 / Rel-19

Abstract: Terms, architecture requirements and assumptions of multi-hop U2N/U2U Relay support is proposed in this contribution.

# 1. Introduction

In the agreed WT#1 of SP-231798, as shown below, the objective is to study how to support multi-hop U2N/U2U Relay.

WT#1: Enhance ProSe to support multi-hop over NR PC5 reference point

WT-1.1: for UE-to-Network Relay

WT-1.2: for UE-UE Relay

NOTE 1:	The UE-to-Network Relay include both Layer-3 and Layer-2 Relays. UE-to-UE Relay include L3 Relay only.

NOTE 2:  This study requires coordination with RAN WGs.

This paper intends to provide architecture requirements and assumptions of multi-hop U2N/U2U Relay support.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-03.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.752: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)".

[3]	3GPP TR 23.700-33: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2".

[4]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS); Stage 1".

[7]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

# * * * * Second change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and TS 23.304 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and TS 23.304 [4].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and TS 23.304 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and TS 23.304 [4].

# * * * * Third change * * * *

# 4	Architecture Requirements and Assumptions

## 4.1	Architecture Requirements

Solutions shall be built on the 5G ProSe architecture principles as defined in TS 23.304 [4] and 5G System architectural principles as defined in TS 23.501 [5], including flexibility and modularity for newly introduced functionalities.

In order to satisfy the normative stage-1 multi-hop-related requirements in TS 22.278 [6], TS 22.261 [7] and TS 22.115 [8]:

-	The system shall support multiple NR PC5 hops UE-to-Network/UE-to-UE Relay for unicast.

-  The network operator shall be able to define the maximum number of hops supported in their networks when using relay UEs.

## 4.2 	Architecture Assumptions

-	Architecture reference models of single-hop UE-to-Network/UE-to-UE Relay defined in TS 23.304 [4] are used as basis architecture for supporting multi-hop UE-to-Network/UE-to-UE Relay.

-	The UE-to-Network Relays on the multi-hop path are the same type, either Layer-2 UE-to-Network Relay or Layer-3 UE-to-Network Relay.

-	To align with RAN WG2, only one indirect UE-to-Network path via Relays is supported at a time.

-	NG-RAN is considered, non-3GPP AN is not considered.

-	NR based PC5 and NR Uu are considered.

# * * * * End of changes * * * *