3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400419

Electronic Meeting, 22-29 January 2024

# Title:	[Draft] Reply LS on UE Location Information for NB-IoT NTN

# Response to:	LS on UE Location Information for NB-IoT NTN (R2-2311326/S2-2400020)

# Release:	Release 18

# Work Item:	IoT_NTN_enh

Source:	SA2

To:	SA1, SA3-LI

Cc:	RAN2, CT1, RAN3

Contact Person:

Name:	Haris Zisimopoulos

Tel. Number:

E-mail Address:	harisz AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 received an LS from RAN2 on UE Location Information for NB-IoT NTN. The incoming LS stated:

“RAN2 rediscussed whether and how location reporting for an NB-IoT UE in NTN can be supported in case LPP mechanism is not supported.

No consensus could be reached in RAN2 as to whether an AS-based solution of reporting UE location, even coarse-grained, for NB-IoT UEs using control plane optimization could be supported, due to privacy and security concerns raised by some companies.

RAN2 would thus like to ask SA2 and CT1 if UE location reporting via NAS layer can be considered in Rel-18, to satisfy this operator requirement.”

During the discussion in SA2#160AH-e meeting, it was questioned by certain companies whether the current regulatory requirements for location verification can be met when the UE and/or the network do not support LPP and therefore when the related LCS procedures that allow the network to perform a network initiated location request to the UE are not supported. In this case, a UE could report its location in association with MO events as described in the RAN2 LS. However, a UE could still move from an allowed region or country to a non-allowed region or country without detecting this which would lead to the network not becoming aware of this if the UE was not sending MO events to the network at this time with its coarse location.

Thus, SA2 question whether relying on location reporting associated only with some existing Mobile Originated (MO) signalling or Data by the UE alone will be enough to meet regulatory requirements, or whether instead it is also required to have an ability for the network to perform a Mobile Terminated (MT) query to obtain a UE’s coarse location even at times when there is no other Mobile Originated (MO) signalling or data that needs to be send by the UE.

2. Actions:

To SA1, SA3-LI:

ACTION: 	SA2 asks SA1 and SA3-LI to clarify the regulatory requirements for location verification for NTN NB-IoT and specifically answer whether relying on location reporting associated only with some existing Mobile Originated (MO) signalling or Data by the UE alone will be enough to meet the regulatory requirements or whether instead it is also required to have the ability for the network to perform a Mobile Terminated (MT) query to obtain UE’s location even at times when there is no other Mobile Originated (MO) signalling or data that needs to be send by the UE .

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #161		26 February-1 March 2024	Athens, Greece

TSG-SA2 Meeting #162		15-19 April 2024	China (TBD)