SA WG2 Meeting #160-Ad Hoc-e	S2-2400435

# E-meeting, January 22 – 29, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | xxx | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS Roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_MBS_Roam |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is currently documented in accordance with TS 23.247 Rel-18 that MBS roaming is not supported. This is a severe restriction for V2X use cases, as it prevents cars in a foreign country to receive V2X related information provided in that country using MBS. The restriction that MBS services of a VPLMN are not supported to be received by inbound roaming UEs is lifted by updates against TS 23.247 in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Document for V2X communication that roaming UEs are enabled to receive MBS multicast and broadcast services provided in the VPLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cars in a foreign country are not able to receive V2X related information provided in that country using MBS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2, 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.2.2	V2X message reception via MBS

The mechanisms defined in TS 23.247 [28] can be used to establish the suitable MBS sessions, and V2X messages are routed from the V2X Application Server towards UEs via broadcast MBS sessions or multicast MBS sessions. As defined in TS 23.247 [28], the reception of a multicast/broadcast service provided by a VPLMN is supported for inbound roaming UEs in this release.

For MBS reception of V2X messages, announcement for V2X services is required for the UE. To provide the UE with the announcement, the following ways may be used:

-	Existing announcement mechanisms specified in TS 23.247 [28], TS 26.502 [29] and TS 26.517 [30].

-	MBS session announcement provisioned as described in clauses 5.1.1 and 5.1.3.1.

The V2X Application Server providing the V2X service identified by V2X service type sends the V2X message via UDP/IP transport using information provided in the announcement.

V2X message transfer via MBS may be achieved by locating the MBS CN NFs (e.g. MB-UPF) closer to the gNB to meet the latency requirement as defined for 5QI 75 specified in TS 23.501 [6].

### 5.3.1	General

A UE needs to discover the V2X Application Server(s), when V2X communication over Uu operation mode is used. The V2X Application Server address information may be configured on the UE or provisioned over N1 reference point, as specified in clause 5.1.3.1.

When the configuration contains the FQDN(s), the UE shall perform DNS to resolve the address(es) of the V2X Application Server. The UE may use the configured V2X Application Server information only in the designated geographical area. When the UE changes serving PLMN or crosses configured geographic areas, it should perform address resolution again.

NOTE:	When the V2X Application Server is notified by SMF indicating the DNAI change, as specified in clause 5.6.7 of TS 23.501 [6], the application layer can trigger the UE to perform DNS to resolve the address(es) of the V2X Application Server.

For a network that has deployed MBS, additional information to assist V2X Application Server discovery can be provided via broadcast MBS session. When a UE has the configuration for receiving V2X Application Server information via broadcast MBS session as specified in clause 5.1.3.1, it can perform the procedure specified in clause 6.4.2 to obtain additional local V2X Application Server information. The local V2X Application Server information obtained by MBS as described in clause 6.4.2 takes precedence over the V2X Application Server information in the UE. As defined in TS 23.247 [28] , the reception of a broadcast service provided by a VPLMN is supported for inbound roaming UEs  in this Release.