3GPP TSG-SA2 Meeting #160AHE	S2-2400526

# January 22 – 29, 2024, Electronic

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5242 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE MSISDN exposure to trusted AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To use the localized services, the UE need to consider availability of time and location of localized services. In order to inform the UE of this information, the validity information is introduced. Therefore, introducing separated two parameters, validity information and location assistance information, is not reasonable. Even, we explicitly mentioned “the location assistance information and is not used for any area restriction enforcement.” in the text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification on AF specific UE ID retrieval procedures to support providng the MSISDN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution to providing the extern entities with the UE MSISDN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.17, 5.20, 7.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.8.2.17	Data exposure via Service Based interface

The UPF may expose information by means of UPF Event Exposure service as described in TS 23.502 [3] clause 5.2.26.2, via a service-based interface directly. The NF consumers, which may receive UPF event notifications, are AF/NEF, TSNAF/TSCTSF and NWDAF/DCCF/MFAF.

When the UPF supports the data exposure via the service based interface, it may register its NF profile to the NRF including the UPF Event Exposure services and the related Event ID(s).

For data collection from UPF (see clause 4.15.4.5 of TS 23.502 [3]), NF consumers do the subscription to the UPF directly or indirectly via SMF. A n NF consumer may subscribe to the UPF Event Exposure service directly only for data collected for "any UE" e.g. to collect user data usage information for NWDAF NF Load analytic (see clause 6.5 of TS 23.288 [86]) and if the subscription is not including any of the following parameters: AoI, traffic filtering, BSSID/SSID and Application ID.

To alleviate the load of UPF due to frequent event notification, the event subscription may include Reporting suggestion information. The Reporting suggestion information includes Report urgency and Reporting window information. Reporting urgency information represents whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting window is also provided, which defines the last valid reporting time, and UPF shall report the detected event before the last valid time. Per Reporting suggestion information UPF can concatenate several notification messages to the same notification endpoint in one notification message.

The UPF may also expose UE information by means of the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in TS 23.502 [3] clause 5.2.26.3. An UPF which is deployed with NAPT (Network Address Port Translation) functionality may support to provide the 5GC UE IP address to NEF based on NEF request containing public IP address and port number using the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in clause 4.15.10 of TS 23.502 [3] for GPSI retrieval which is either an AF specific UE ID or a MSISDN.

* * * Second Changes * * *

5.20	External Exposure of Network Capability

The Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability, Analytics reporting capability and Member UE selection capability. The Monitoring capability is for monitoring of specific event for UE in 5G System and making such monitoring events information available for external exposure via the NEF. The Provisioning capability is for allowing external party to provision of information which can be used for the UE in 5G System. The Policy/Charging capability is for handling access and mobility management, QoS and charging policies for the UE based on the request from external party. The Analytics reporting capability is for allowing an external party to fetch or subscribe/unsubscribe to analytics information generated by 5G System (this is further defined in TS 23.288 [86]). The Member UE selection capability is for allowing an external party to acquire one or more list(s) of candidate UE(s) (among the list of target member UE(s) provided by the AF) and additional information that is based on the assistance information generated by 5G System based on some defined filtering criteria, the details are explained in clause 4.15.13 in TS 23.502 [3].

Monitoring capability is comprised of means that allow the identification of the 5G network function suitable for configuring the specific monitoring events, detect the monitoring event, and report the monitoring event to the authorised external party. Monitoring capability can be used for exposing UE's mobility management context such as UE location, reachability, roaming status, and loss of connectivity. Monitoring capability can also be used for exposing QoS monitoring result. AMF stores URRP-AMF information in the MM context to determine the NFs that are authorised to receive direct notifications from the AMF. UDM stores URRP-AMF information locally to determine authorised monitoring requests when forwarding indirect notifications. The Monitoring capability also allows AF to subscribe to the group status changes for a group, either a 5G VN group as described in clause 5.29.2, as well as a group configured by OA&M. In this case the AF is notified if the group member list is updated or a group member is no longer subscribed to the group.

Provisioning capability allows an external party to provision the Expected UE Behaviour or the 5G-VN group information or DNN and S-NSSAI specific Group Parameters or ECS Address Configuration Information or service specific information to 5G NF via the NEF. The provisioning comprises of the authorisation of the provisioning external third party, receiving the provisioned external information via the NEF, storing the information, and distributing that information among those NFs that use it. The externally provisioned data can be consumed by different NFs, depending on the data. In the case of provisioning the Expected UE Behaviour, the externally provisioned information which is defined as the Expected UE Behaviour parameters in clause 4.15.6.3 of TS 23.502 [3] or Network Control parameter in clause 4.15.6.3a of TS 23.502 [3] consists of information on expected UE movement, Expected UE Behaviour parameters or expected Network Configuration parameter. The provisioned Expected UE Behaviour parameters may be used for the setting of mobility management or session management parameters of the UE. In the case of provisioning the 5G-VN group information the externally provisioned information is defined as the 5G-VN group parameters in clause 4.15.6.7 of TS 23.502 [3] and it consists of some information on the 5G-VN group. In the case of the provisioning the DNN and S-NSSAI specific Group Parameters, the externally provisioned information is defined in clause 4.15.6.14 of TS 23.502 [3] and clause 5.20b. In the case of provisioning ECS address, the externally provisioned information is defined as the ECS Address Configuration Information in clause 4.15.6.3d of TS 23.502 [3]. The affected NFs are informed via the subscriber data update as specified in clause 4.15.6.2 of TS 23.502 [3]. The externally provisioned information which is defined as the Service Parameters in clause 4.15.6.7 of TS 23.502 [3] consists of service specific information used for supporting the specific service in 5G system. The provisioned Service Parameters may be delivered to the UEs. The affected NFs are informed of the data update.

Policy/Charging capability is comprised of means that allow the request for session and charging policy, enforce QoS policy, apply accounting functionality and requests to influence access and mobility management policies. It can be used for specific QoS/priority handling for the session of the UE, and for setting applicable charging party or charging rate.

Analytics reporting capability is comprised of means that allow discovery of type of analytics that can be consumed by external party, the request for consumption of analytics information generated by NWDAF.

Member UE selection capability is comprised of means that allows filtering and providing one or more list(s) of candidate UE(s) (among the list of target member UE(s) provided by the AF) and additional information that can be consumed by external party, the request for consumption of UE list generated by external party.

An NEF may support CAPIF functions for external exposure as specified in clause 6.2.5.1.

An NEF may support exposure of NWDAF analytics as specified in TS 23.288 [86].

The NEF may support exposure of 5GS and/or UE availability and capabilities for time synchronization service as specified in clause 5.27.1.8.

An NEF may support exposure of event based notifications and reports for NSACF as specified in clause 5.15.11.

An AF may only be able to identify the target UE of an AF request for external exposure of 5GC capabilities (e.g. Data Provisioning or for Event Exposure for a specific UE) by providing the UE's address information. In this case the NEF first needs to retrieve the Permanent identifier of the UE before trying to fulfil the AF request. The NEF may determine the Permanent identifier of the UE, as described in clause 4.15.3.2.13 of TS 23.502 [3], based on:

-	the address of the UE as provided by the AF; this may be an IP address or a MAC address;

-	the corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or determined by the NEF based on the requesting AF; this is needed if the UE address is an IP address.

The NEF may provide an AF specific UE Identifier or a MSISDN to the AF:

-	that has explicitly requested a translation from the address of the UE to a unique UE identifier (via Nnef_UEId service); or

-	that has implicitly requested a translation from the address of the UE to a AF specific UE Identifier or a MSISDN by requesting external exposure about an individual UE identified by its address.

The AF may have its own means to maintain the AF specific UE Identifier through, e.g. an AF session. After the retrieval of an AF specific UE Identifier the AF shall not keep maintaining a mapping between this identifier and the UE IP address as this mapping may change.

The AF specific UE Identifier shall not correspond to a MSISDN; it is represented as a GPSI in the form of an External Identifier. When used as an AF specific UE identifier, the External Identifier provided by the 5GCN shall be different for different AF.

NOTE 1:	This is to protect user privacy.

NOTE 2:	The AF specific UE identifier is ensured to be unique across different AFs as defined in TS 23.003 [19] by configuration. Such configuration is assumed to be coordinated between the different involved entities (e.g. NEF(s) and UDM/UDR).

NOTE 3:	Based on policies, the NEF can be configured to enforce restriction on the usage of AF specific UE identifier (e.g. rejection of a service request from AF not authorized to use the UE identifier).

* * * Third Changes * * *

7.2.8	NEF Services

The following NF services are specified for NEF:

Table 7.2.8-1: NF Services provided by NEF

| Service Name | Description | Reference in TS 23.502 [3] or other TS |
| --- | --- | --- |
| Nnef_EventExposure | Provides support for event exposure. | 5.2.6.2 |
| Nnef_PFDManagement | Provides support for PFDs management. | 5.2.6.3 |
| Nnef_ParameterProvision | Provides support to provision information which can be used for the UE in 5GS. | 5.2.6.4 |
| Nnef_Trigger | Provides support for device triggering. | 5.2.6.5 |
| Nnef_BDTPNegotiation | Provides support for background data transfer policy negotiation and optionally notification for the renegotiation. | 5.2.6.6 |
| Nnef_TrafficInfluence | Provide the ability to influence traffic routing. | 5.2.6.7 |
| Nnef_ChargeableParty | Requests to become the chargeable party for a data session for a UE. | 5.2.6.8 |
| Nnef_AFsessionWithQoS | Requests the network to provide a specific QoS for an AF session. | 5.2.6.9 |
| Nnef_MSISDN-less_MO_SMS | Used by the NEF to send MSISDN-less MO SM to the AF. | 5.2.6.10 |
| Nnef_ServiceParameter | Provides support to provision service specific information. | 5.2.6.11 |
| Nnef_APISupportCapability | Provides support for awareness on availability or expected level of a service API. | 5.2.6.12 |
| Nnef_NIDDConfiguration | Used for configuring necessary information for data delivery via the NIDD API. | 5.2.6.13 |
| Nnef_NIDD | Used for NEF anchored MO and MT unstructured data transport. | 5.2.6.14 |
| Nnef_SMContext | Provides the capability to create, update or release the SMF-NEF Connection. | 5.2.6.15 |
| Nnef_AnalyticsExposure | Provides support for exposure of network analytics. | 5.2.6.16 |
| Nnef_UCMFProvisioning | Provides the ability to configure the UCMF with dictionary entries consisting of UE manufacturer-assigned UE Radio Capability IDs, the corresponding UE radio capabilities, the corresponding UE Radio Capability for Paging and the (list of) associated IMEI/TAC value(s) via the NEF. The UE radio capabilities the NEF provides for a UE radio Capability ID can be in TS 36.331 [51] format, TS 38.331 [28] format or both formats. Also used for deletion (e.g. as no longer used) or update (e.g. to add or remove a (list of) IMEI/TAC value(s) associated to an entry) of dictionary entries in the UCMF. | 5.2.6.17 |
| Nnef_ECRestriction | Provides support for queuing status of enhanced coverage restriction, or enable/disable enhanced coverage restriction per individual UEs. | 5.2.6.18 |
| Nnef_ApplyPolicy | Provides the capability to apply a previously negotiated Background Data Transfer Policy to a UE or a group of UEs. | 5.2.6.19 |
| Nnef_Location | Provides the capability to deliver UE location to AF. | 5.2.6.21 |
| Nnef_AMInfluence | Provides the ability to influence access and mobility management related policies for one or multiple UEs. | 5.2.6.22 |
| Nnef_AMPolicyAuthorization | Provides the ability to provide inputs that can be used by the PCF for deciding access and mobility management related policies. | 5.2.6.23 |
| Nnef_AKMA | AKMA Application Key derivation service. | TS 33.535 [124] |
| Nnef_Authentication | This service enables the consumer to authenticate and authorize the Service Level Device Identity as described in TS 23.256 [136]. | TS 23.256 [136] |
| Nnef_TimeSynchronization | Provides the ability to support for (g)PTP or 5G access stratum based time synchronization service. | 5.2.6.25 |
| Nnef_EASDeployment | EAS deployment service. | 5.2.6.26 |
| Nnef_UEId | UE Identifier service, which supports to retrieve AF specific UE Identifier or MSISDN based on UE address. | 5.2.6.27 |
| Nnef_MBSTMGI | Allows AF to request allocation/deallocation of TMGI(s) for MBS Session. | TS 23.247 [129] |
| Nnef_MBSSession | Allows AF to create, update and delete MBS Session. | TS 23.247 [129] |
| Nnef_MBSGroupMsgDelivery | Allows AF to request to create, update and delete resource for group message delivery via MBS Session. | TS 23.247 [129] |
| Nnef_ASTI | Provides the ability to influence 5G access stratum based time distribution configuration. | 5.2.6.28 |
| Nnef_SMService | Used for SBI-based MO SM transmit through NEF for MSISDN-less MO SMS. | 5.2.6.29 |
| Nnef_PDTQPolicyNegotiation | Provides support for negotiation for Planned Data Transfer with QoS requirements policy and optionally notification for the renegotiation. | 5.2.6.30 |
| Nnef_MemberUESelectionAssistance | Provides one or more list(s) of candidate UE(s) (among the list of target member UE(s) provided by the AF) and additional information based on the parameters contained in the request from the AF. | 5.2.6.31 |
| Nnef_DNAIMapping | Allows AF to obtain DNAI. | 5.2.6.34 |
| Nnef_TrafficInfluenceData | Used in HR SBO as defined in TS 23.548 [130] to get AF Traffic Influence configuration from the V-NEF. | 5.2.6.35 |
| Nnef_ECSAddress | This service is defined only for the support of HR-SBO. It allows AF to provide ECS Address Configuration Information for a group of UE or any UE to V-NEF. It allows V-SMF to subscribe and retrieve ECS Address Configuration Information. | 5.2.6.37 |

* * * End of Changes * * *