SA WG2 Meeting SA2#160-Ad Hoc-e	S2-2400565

# 22-29 January, 2024, Electronic

Source: 	OPPO

Title: 	KI#1, New Sol: Support of feeder link switchover for regenerative eNB payload

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a solution for KI#1 of FS_5GSAT_ARCH_Ph3.

# Introduction

KI#1 focus on the deployment of regenerative payload on LEO/MEO satellite access, and whether there is any impact on 5GS and EPS.

Regenerative payloads, when there is an eNB embedded on the satellite, results in the termination of the S1-AP interface at the satellite. This configuration encounters challenge due to eNB moving, which might lead to the feeder link switchover from a source NTN GW to a target NTN GW when a LEO/MEO satellite exits the coverage area of the source NTN GW. In such instances, the eNB could be served, before and after the feeder link switchover, by different MMEs. Therefore, a TAU procedure should be initiated when the UE is informed of the occurrence of feeder link switchover. This procedure applies to UEs in either idle mode or connected mode.

# 2	Proposal

It is proposed to agree the following changes to TR 23.700-29.

Start of Changes

## 6.X	Solution #X: Support of Feeder Link Switchover for regenerative eNB payload

### 6.X.1	Description

This is a candidate solution to address Key Issue #1. Specifically, the solution proposes to enhance TAU procedure with feeder link switchover as one of the triggering conditions.

Regenerative payloads, when there is an eNB embedded on the satellite, results in the termination of the S1-AP interface at the satellite. This configuration encounters challenge due to eNB moving, which might lead to the feeder link switchover from a source NTN GW to a target NTN GW when a LEO/MEO satellite exits the coverage area of the source NTN GW. In such instances, the eNB could be served, before and after the feeder link switchover by different MMEs. Therefore, the UE should initiate a TAU procedure when it is informed that a feeder link switchover has occurred. This procedure applies to UEs in either idle mode or connected mode.

If the embedded eNB onboard predicts the feeder link switchover, it might broadcast a “feeder link switchover information” message across the cell.

NOTE 1: Whether and how to include proper feeder link switchover information in SIBs requires further discussion and coordination with RAN WG2.

Alternatively, before the feeder link switchover, if any ground-based entity, such as the source NTN Gateway or MME, predicts an imminent feeder link switchover based on the satellite's control data, e.g., satellite ephemeris information, MME might notify UE via a proper NAS message the feeder link switchover information including the type of switchover and timing of the feeder link switchover.

During feeder link switchover, the TNL node on the satellite will be triggered to configure the transport association for the target NTN GW feeder link, e.g., by adding an IP address for the existing SCTP association or establishing a new SCTP association upon eNB request. The eNB might notify the UE when the new feeder link TNL association has become operational.

NOTE 2: Whether and how to include proper feeder link switchover information in SIBs requires further discussion and coordination with RAN WG2.

To avoid large number of UEs causing excessive signalling load on the network, Overload control mechanism, described in Clause 4.13.8.6 of TS 23.401 might be used.

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.X.2-1 illustrates the call flow and system changes for feeder link switchover-triggered Tracking Area Update (TAU) procedure.

Figure 6.X.2-1: High-level procedure for TAU triggered by Feeder Link Switchover

- As described in clause 6.X.1, the feeder link switchover triggers UE to start TAU procedure, after being notified that new feeder link TNL association has become operational.

- The UE initiates the TAU procedure by sending, to the eNB, a TAU Request with cause, e.g., feeder link switchover, in addition to regular parameters described in clause 5.3.3.1 of TS 23.401.

- The eNB derives the MME address from the RRC parameters carrying the old GUMMEI, the indicated Network and the RAT. If that MME is not associated with that eNB, the eNB selects an MME as described in clause 4.3.8.3 of TS 23.401.

- In steps 4-19, regular TAU procedures will be followed, as specified in clause 5.3.3.1 for TAU with SGW change and in clause 5.3.3.2 for TAU without SGW change in TS 23.401.

Following the steps 4-19, the MME sends a TAU Accept in step 20. And in step 21, UE might acknowledge the received TAU Accept message by returning a TAU Complete message to the MME.

After the TAU procedures, for scenarios involving a hard feeder link switchover, where the onboard eNB can connect to only one NTN Gateway at a time, and when the MMEs associated with the source and target NTN Gateways are different, the S1 release procedure may be performed locally by the eNB and by the MME, with cause, e.g., feeder link switchover, without using any of the signalling directly between eNB and MME.

In the case of soft feeder link switchover where the onboard eNB can connect to multiple NTN Gateways simultaneously during a given period, and when the MME associated with the source and target NTN Gateways are different, the process for releasing S1 connections may be initiated by either the eNB or the MME associated with the source NTN Gateway, with cause, e.g., feeder link switchover.

### 6.X.3	Impacts to Services, Entities and Interfaces

This solution impacts the following entities.

RAN:

- Release S1-AP based on feeder link switchover information.

- Broadcast feeder link switchover information

MME:

- Release S1-AP based on feeder link switchover information.

- New triggering condition, feeder link switchover, for TAU

- New IE in NAS message to include feeder link switchover information.

UE:

- Initiate the Tracking Area Update procedure when being informed of the occurrence of feeder link switchover.

End of Changes