3GPP TSG-WG SA2 Meeting #160AH-e	S2-2400591

E-meeting, Jan 22 – 29, 2024

Title:	Reply LS on Support of interworking between SA4 RTC and IMS

Response to:	LS on Support of interworking between SA4 RTC and IMS (S2-2400070/S4-232055)

Release:	Rel-18

Work Item:	FS_eiRTCW

Source:	SA2

To:	SA4

Cc:	SA1, CT3

Contact person:			Yoshihiro Inoue

yoshihiro.inoue AT ntt-at DOT co DOT jp

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

- Overall description

SA2 thanks SA WG4 for the LS on Support of interworking between SA4 RTC and IMS. SA2 has the following answers to the question from SA4.

Question:

Are there any potential impacts on existing IMS specifications from the above NNI-based solution (connecting the WebRTC-based RTC communication network to IMS network, by applying the interworking model specified in 3GPP TS 29.162), considering that the solution uses the interface of existing 3GPP IMS specification without any modifications?

Answer:

From the perspective of IMS stage 2 specification, IMS supports interconnection with IM CN subsystem network and other SIP based multimedia network as specified in 3GPP TS 23.228 clause 4.0 and Annex I. Therefore, there are no potential impacts on existing IMS specifications from the proposed NNI-based solution, if the RTC communication network is connected via the reference point complies with existing Ici/Izi reference point or Mm/Mb reference point.

If the interworking function of the RTC communication network complies with Inter-IMS Network-to-Network specification, the RTC communication network is connected to IMS network via the reference point complies with Ici/Izi reference point. Otherwise, the RTC communication network is connected to IMS network as an SIP based IP multimedia network via the reference point complies with Mm/Mb reference point.

- Actions

To SA4:

ACTION: 	SA2 respectfully asks SA4 to take the above into account.

3	Dates of next TSG SA WG 2 meetings

TSG-SA2 Meeting #161		26th February - 1st March, 2024		Athens, GR

TSG-SA2 Meeting #162		15th April - 19th April, 2024			China (TBD), CN