3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400641

Online, Jan 22 – 29, 2024

Source:	Huawei, HiSilicon

Title:	New term of MWAB

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2/ Rel-19

Abstract: A new term of MWAB of FS_VMR_Ph2 is proposed.

1. Introduction

In order to distinguish the existing term of MBSR (Mobile Base Station Relay) defined in TS 23.501 since Rel-18, it is proposed to introduce a new term of MWAB (Mobile Wireless Access Backhaul) into TR 23.700-06.

1. Discussion

During R18 FS_VMR, the term of MBSR is introduced in the TR 23.700-05.

mobile base station relay: A mobile base station acts as a relay between a UE and the 5G network, i.e. providing a NR access link to UEs and connected wirelessly (using NR) through a IAB-donor to the 5G Core. Such mobile base station relay is assumed to be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle).

According to the agreed RP-234041, it is assumed that Wireless Access Backhaul (WAB) is aligned with VMR use cases and with the SA2-endorsed SID on architectural enhancements for Rel-19 VMR.

The architectures with full gNB onboard the WAB is described in RP-231973(see Figure 2-1).

Figure 2-1: Wireless Access and Backhauling (WAB)

The WAB node holds full gNB function, which is co-located with UE function (WAB-UE) and provides a NR access link to UEs. The gNB onboard the WAB node (i.e. WAB-gNB) establishes N2/N3 interface to an AMF/UPF residing in the 5GC over a PDU session of WAB-UE.

In order to distinguish the term of MBSR (Mobile Base Station Relay) defined in TS 23.501 since Rel-18, it is proposed to introduce a new term of MWAB (Mobile Wireless Access Backhaul) into TR 23.700-06. The definition of MWAB is proposed as the following:

Mobile Wireless Access Backhaul: The Mobile Wireless Access Backhaul, which consists of a full gNB function and UE function, provides a NR access link to UEs and establishes N2/N3 interface to an AMF/UPF residing in the 5GC over a PDU session of the UE onboard the Mobile Wireless Access Backhaul node. Such Mobile Wireless Access Backhaul is assumed to be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle).

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-06.

# * * * * First change * * * *

3	Definitions of terms and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Mobile Wireless Access Backhaul: The Mobile Wireless Access Backhaul, which consists of a full gNB function and UE function, provides a NR access link to UEs and establishes N2/N3 interface to an AMF/UPF residing in the 5GC over a PDU session of the UE onboard the Mobile Wireless Access Backhaul node. Such Mobile Wireless Access Backhaul is assumed to be mounted on a moving vehicle and serve UEs that can be located inside or outside the vehicle (or entering/leaving the vehicle).

# * * * * Second change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

WAB          Wireless Access Backhaul

MWAB        Mobile Wireless Access Backhaul

# * * * * End of changes * * * *