SA WG2 Meeting #160AH-e	S2-2400671

# E-meeting, January 22nd – 29th, 2024

Source: 	China Telecom

Title: 	New Key Issue on WT #1.2

Document for: 	Approval

Agenda Item: 	19.7

Work Item / Release:	FS_5G_ProSe_Ph3 / Rel-19

Abstract of the contribution:

- Discussion

This contribution proposes a new Key Issue to address work task #1.2 of the FS_5G_ProSe_Ph3 SID (SP-231798) approved at the SA #102 meeting.

- WT-1: Enhance ProSe to support multi-hop over NR PC5 reference point - WT-1.1: for UE-to-Network Relay - WT-1.2: for UE-UE Relay NOTE 1: The UE-to-Network Relay include both Layer-3 and Layer-2 Relays. UE-to-UE Relay include L3 Relay only. NOTE 2: This study requires coordination with RAN WGs.

2. Proposal

It is proposed to adopt the following text in TR 23.700-03.

*** Start of the 1st change (all new text) ***

5.X	Key Issue #X: Support of Multi-hop Layer-3 UE-to-UE Relay

5.X.1	General description

This key issue intends to support multi-hop UE-to-UE Relay for unicast as illustrated in figure 5.x.1-1, including support for in coverage and out of coverage operation of Source UE, Target UE as well as the Multi-hop Layer-3 UE-to-UE Relay.

Figure 5.x.1-1: Example scenario of support of Multi-hop UE-to-UE Relay

When studying the above aspect, the following needs to be considered:

- How to discover multi-hop UE-to-UE Relay(s) and create a route, including how to prevent routing Loops.

- Whether and how the network can control multi-hop UE-to-UE Relay operation, at least including:

-	Whether and how to authorize the multi-hop UE-to-UE Relay, e.g. authorize a UE as multi-hop UE-to-UE Relay.

-	Whether and how to authorize Source/Target UEs to use a multi-hop UE-to-UE Relay.

-	How to establish the connection between the source UE and the target UE via multi-hop UE-to-UE Relays.

-	How to provide end-to-end QoS framework to satisfy the QoS requirements (such as data rate, reliability, latency).

-	How to enhance the system architecture to provide security/privacy protection for a relayed connection.

-  Whether and how to provide a mechanism for a path changing in the case of e.g. UE-to-UE Relay changes, or re-create a route.

-  Whether and how to enhance the existing mechanisms for IP address/prefix allocation.

*** End of the 1st change ***