SA WG2 Meeting #160-AH-e	S2-2400677

# January 22 – 29, 2024; Elbonia

Source: 	NTT DOCOMO

Title: 	KI#1: New solution for UE configuration with regards to the CAG-CSG mapping

Document for: 	Information

Agenda Item: 	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract: This contribution proposes a new solution for KI#1.

1	Introduction

Editor's note: This part is subject to change depending on the approved KI#1 description.

This contribution proposes a solution for an aspect of UE configuration with regards to the CAG-CSG mapping. This solution is intended to be used together with a solution in pCR "KI#1: New solution for how to enable interworking between CAG and CSG cells".

2	Proposal

It is proposed to add the following text to TR 23.700-45.

## ---Start of the 1st Change---

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #2 |  |  |
| #x | X |  |

## ---Start of the 2nd Change---

6.X	Solution #X: UE configuration with regards to the CAG-CSG mapping

6.X.1	Description

6.X.1.1	General

This solution is intended to be used together with "Solution of Handover restriction assisted by UE and AMF."

NOTE:	A separated solution is proposed here, since another solution, e.g., configuration by OMA DM, can work together with "Solution of Handover restriction assisted by UE and AMF."

6.X.1.2	Handover from EPS to 5GS

UE is prepared as follows for this direction of the handover.

OAM pre-configures the mapping of CAG ID and CSG ID in SMF+PGW-C. It is enhanced in such a way that SMF+PGW-C provides the mapping of CAG ID and CSG ID to UE in PCO during the Attach procedure. UE is provisioned Allowed CSG information via OMA DM, as the existing specification suggests.

6.X.1.3	Handover from 5GS to EPS

UE is prepared as follows for this direction of the handover.

OAM pre-configures the mapping of CAG ID and CSG ID in AMF. It is enhanced in such a way that AMF provides the mapping of CAG ID and CSG ID to UE in Registration Accept. AMF provides Allowed CAG information in Registration Accept, as the existing specification suggests.

6.X.2	Procedures

Editor's Note:	This clause describes high-level procedures and information flows for the solution.

6.X.3	Impacts on services, entities and interfaces

UE: to treat the mapping of CAG ID and CSG ID.

SMF+PGW-C: to treat the mapping of CAG ID and CSG ID.

AMF: to treat the mapping of CAG ID and CSG ID.

## ---End of the Changes---