3GPP TSG|WG-2 Meeting #160 Ad hoc-e 	S2-2400697

22-29 Jan 2024, Emeeting	(revision of S2-2312210)

# Source:	China Telecom, Vivo, CATT, Spreadtrum, Xiaomi, ZTE, Hughes/EchoStar, Inmarsat, Viasat, Oppo, TCL

# Title:	New WID on 5GC support for IoT NTN

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WI on 5GC support for IoT NTN

Acronym:	TEI19_ 5GC_ IoT NTN

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 80048 | Stage 1 of 5GSAT | Service requirements of satellite access in 5G |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks (LTE_NBIOT_eMTC_NTN) | E-UTRAN support for IoT NTN |
| 860005 | Integration of satellite systems in the 5G architecture (5GSAT_ARCH) | 5GS NTN functionality to be used as baseline for IoT NTN in EPS |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS (IoT_SAT_ARCH_EPS) | WID of E-UTRAN support for IoT NTN |
| 800026 | Study on architecture aspects for using satellite access in 5G | Unresolved key issue leftover from R17 |
| 940074 | 5GC enhancement for satellite access Phase 2 | SA2 Rel-18 SID for 5G System enhancement for satellite access |
| 980014 | 5GC/EPC enhancement for satellite access Phase 2 | SA2 Rel-18 WID for 5G System enhancement for satellite access |

3	Justification

The Internet of Things (IoT) Non-Terrestrial Network (NTN) plays a pivotal role in enhancing the IoT market, offering efficient, cost-effective, and scalable connectivity solutions. In addition, IoT NTN can also offer services for personal safety and messaging where TN coverage is out of reach. This is especially significant when considering interoperability with conventional terrestrial networks. Recent market developments and business strategies from a range of operators, satellite companies, and chipmakers underscore the significance of IoT NTN as the first commercially deployed 3GPP NTN technology.

The initial IoT NTN specification, known as IoT_SAT_ARCH_EPS, has been standardized in 3GPP Release-17, with a focus on supporting Cellular IoT over satellite access within the Evolved Packet Core (EPC). Traditional mobile network operators, equipped with legacy EPC infrastructure, tend to prefer upgrading their existing EPC to accommodate IoT NTN access seamlessly.

However, for operators, particularly emerging satellite providers lacking an established EPC, a forward-looking perspective favours the use of 5G Core (5GC) for future commercial IoT NTN deployment. Furthermore, in cases where an operator needs to simultaneously deploy New Radio NTN (NR NTN) and IoT NTN to meet diverse market demands, opting for a single-core solution for both NR NTN and IoT NTN offers distinct advantages. It helps in reducing both capital expenditures (CAPEX) and operational expenditures (OPEX) from a cost-efficiency standpoint.

Based on the aforementioned considerations, the inclusion of 5GC support for IoT Non-Terrestrial Network (NTN) becomes an imperative and time-sensitive requirement for the successful launch of IoT NTN in the commercial arena. Additionally, the 5GC is part of an evolving roadmap with continuous development of new features, some of which are highly relevant to IoT NTN access. In contrast, 3GPP has largely ceased the development of new features for the Evolved Packet Core (EPC), e.g. Slicing, Edge Computing. Therefore, standardizing 5GC support for IoT NTN in Release 19 (R19) is crucial to address both the needs of commercial deployment and the evolving requirements of IoT NTN.

It's worth noting that 3GPP has already standardized 5GC support for Cellular IoT (CIoT), including Narrowband IoT (NB-IoT), since Release 16, involving enhancements to both 5GC and E-UTRA, although not deployed mainly due to the existence/availability of legacy EPC. Consequently, the standardization effort for 5GC support for IoT NTN can primarily concentrate on specific aspects related to CIoT over satellite access. In fact, similar specific mechanisms that were originally designed for 5GC support of NR NTN are already being repurposed for EPC support of IoT NTN, streamlining the standardization process.

4	Objective

Based on the above justifications, the observations and proposals in DP S2-2400791, it is proposed to proceed a TEI to enable IoT NTN access 5GC with the following enhancements:

- AMF needs to recognize new IoT NTN RAT types (as defined in clause 4.13.2 in TS23.401) transmitted by ng-eNB for mobility restrictions (RAT restrictions) and policy control (QoS control, charging control)

- Network-initiated location verification is not supported if RAT type is IoT NTN RAT type.

NOTE: 	If network verified UE location for IoT NTN is enhanced in Rel 18, this part will be aligned with SA&RAN results.

And the following alignments:

- Support of network selection for cellular IoT can apply to IoT NTN in 5GC.

- Support of tracking area handling, mobility registration update, mobility forbidden area, service area restrictions, discontinuous coverage for NR NTN can apply to IoT NTN in 5GC.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | A new clause to state the general guidence on support for Cellular IoT over satellite access into 5GS. New RAT Types values relevant to Cellular IoT over satellite access are introduced in 5GC, for identification and access restriction of the different RATs. | SA#105 | This will be handled as “TEI19”. |
| 23.502 | Extension in relevant clauses to support E-UTRA satellite access, on the place of existing support of NR satellite access. | SA#105 |  |
| 23.503 | 5GC policy control for IoT NTN | SA#105 |  |

This TEI19_WID requires 1 TU

6	Work item Rapporteur(s)

Nie Heng, China Telecom, nieheng@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

NGAP enhancement in 38.413 by RAN3, e.g. identification, restriction for RATs of IoT NTN, location reporting for IoT NTN.

Uu interface enhancement by RAN2, e.g. Access control on SIB, New UE capability for IoT NTN.

Enhancement on ng-RAN architecture to support satellite ng-eNB in 38.300 and 36.300 by RAN2.

9	Supporting Individual Members

Supporting IM name

China Telecom

Vivo

CATT

Spreadtrum

Xiaomi

ZTE

Hughes/EchoStar

Inmarsat

Viasat

Oppo

TCL