3GPP TSG-WG SA2 Meeting #160-AH-E	S2-2400811

# Electronic Meeting, Jan 22th – 26th		(revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4713 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Corrections on AF traffic influence and related information transmission in HR-SBO case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Edge_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. It was agreed that when AF request sent via VPLMN for HR-SBO PDU session, HPLMN DNN/S-NSSAI will be provided rather than VPLMN DNN/S-NSSAI, and HPLMN ID of the UE should be also provided to differ the same DNN/S-NSSAI value used in different PLMNs. 2. As described in clause 4.3.6.5 and the SMF service operation, V-SMF subscribes AF request targeting Internal Group Identifier(s) for HR-SBO case is supported. But some descriptions are missed. This contribution propose to add the missing IEs and descriptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add “HPLMN ID” in Data Keys used by V-SMF for AF request subscription, and add clarifications on the HPLMN DNN/S-NSSAI. 2. Add descriptions on how V-SMF receives Internal Group Identifier(s) and subscribes for AF request using Internal Group Identifier(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How AF traffic influence procedure for HR-SBO is performed is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.548 CR 0213 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.3.6.5	Processing AF requests to influence traffic routing for HR-SBO session

Processing an AF requests to influence traffic routing for HR-SBO session via VPLMN are based on procedure described in clause 4.3.6.2 and clause 4.3.6.4 with the following differences:

In step 0 of figure 4.3.6.2-1, V-SMF supporting HR-SBO subscribes to notification of AF request by invoking Nnef_TrafficInfluenceData_Subscribe service from V-NEF (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = HPLMN S-NSSAI and/or DNN and HPLMN ID and UE IP address) and (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = HPLMN S-NSSAI and/or DNN and HPLMN ID and  “any UE” indication) and (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = HPLMN S-NSSAI and/or DNN and HPLMN ID and SUPI) and (Data Set = Application Data; Data Subset = AF traffic influence request information; Data Key = HPLMN S-NSSAI and/or DNN and HPLMN ID and Internal Group Identifier(s)).

NOTE:	The V-SMF receives Internal Group Identifier(s) corresponding to the UE from H-SMF during HR-SBO PDU session establishment procedure.

-	For any UE, the V-SMF supporting HR-SBO performs the above subscription using HPLMN S-NSSAI and/or DNN and HPLMN ID as Data Key.

-	For individual UE or group of UE(s), when a PDU Session is authorized for HR-SBO as described in clause 6.7 of TS 23.548 [74], the V-SMF serving the PDU Session performs the above subscription using HPLMN S-NSSAI and/or DNN and HPLMN ID and/or Internal Group Identifier(s) or SUPI as Data Key. The V-NEF subscribes to notification of AF request from V-UDR.

Steps 4 and 5 of figure 4.3.6.2-1 are replaced with the following steps:

-	Step 4: The V-UDR notifies the subscribed V-NEF of the AF traffic influence request information.

-	Step 5: The V-NEF notifies the subscribed V-SMF of the AF traffic influence request information.

# * * * * End of changes * * * *