3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400821

Online, Jan 22 – 29, 2024	(revision of S2-2312960)

Source:	Huawei, HiSilicon

Title:	KI#4: New Sol: Support of traffic detection and QoS mapping for RTP multiplexing

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRMPh2 / Rel-19

Abstract: Solution for support of traffic detection and QoS mapping for RTP multiplexing is proposed in this contribution.

1. Introduction

In the agreed Key Issue #4 as shown below, the objective is to study how to perform the traffic detection and QoS Flow mapping for different m edia types multiplexed data flows within a single end-to-end transport connection.

- How to identify multiplexed traffic flows with different QoS requirements within a single transport connection.

- How to do QoS Flow mapping for traffic flows with different QoS requirements.

- Whether and what information needs to be provided from AF for traffic detection.

- Whether and how AF provides QoS requirements of different traffic flows to the 5GS.

RTP multiplexing is widely used in the industry. This paper intends to provide support of multiplexed media traffic handling.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[X]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications", July 2003.

[Y]	IETF RFC 8872: " Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams ", Jan 2021.

[Z]	IETF RFC 5761: " Multiplexing RTP Data and Control Packets on a Single Port", April 2010.

Editor’s note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

# * * * * Second change, all new texts * * * *

6.X	Solution #X: Support of traffic detection and QoS mapping for RTP multiplexing

6.X.1	Key Issue mapping

This solution is for Key Issue #4, which address the traffic detection and QoS flow mapping for multiplexed data flows, including:

- How to identify multiplexed traffic flows with different QoS requirements within a single transport connection.

- How to do QoS Flow mapping for traffic flows with different QoS requirements.

- Whether and what information needs to be provided from AF for traffic detection.

- Whether and how AF provides QoS requirements of different traffic flows to the 5GS.

6.X.2	Description

In case of RTP multiplexing, there are different scenarios where multiplexed media flows with different QoS requirements can be detected via different multiplexing fields as:

-	For RTP streams multiplexed in an RTP session [X], the Synchronization Source (SSRC) is used to identify the source of a stream of RTP packets, when multiplexing the RTP streams in the RTP sender and de-multiplex the RTP streams in the RTP receiver side. In this case, the SSRC in the RTP header can be used as the multiplexing field to differentiate the RTP stream(s) with different QoS requirements and map them into different QoS Flows for different QoS handling. For Example, multiple RTP streams are used to carry different layers of a layered encoding [Y].

Figure 6.X.2-1 SSRC format in the RTP header[X]

-	The Payload Type in the RTP header can be used to differentiate the RTP streams carrying different media payload type (e.g. audio vs. video), while media streams of different payload types may require different QoS handlings. The payload type field in the RTP header can be used as the multiplexing field to differentiate the media streams with different QoS requirements and map them into different QoS Flows for different QoS handling.

Figure 6.X.2-2 Payload Type in the RTP header[X]

-	In case of multiplexing RTP and RTCP in a single port [Z], the combination of RTP M bit and the RTP payload type occupy the same position in the RTCP packet as the RTCP packet type field. This field can be used to distinguish the RTP and RTCP packets, e.g. the values or value range of this field is different for RTP and RTCP packets. Therefore, this field can be used as the multiplexing field to differentiate the RTP packets and RTCP packets with different QoS requirements and map them into different QoS Flows for different QoS handling.

Figure 6.X.2-3 M & Payload Type in RTP header[X]

Figure 6.X.2-4 Packet Type in RTCP header[X]

In order to support the mixed traffic handling, the multiplexing field is needed to assist UPF/UE for traffic detection and QoS Flow mapping, e.g., the SSRC, payload type, M bit in the RTP header and also the packet type in the RTCP header as discussed in the above. The existing PCC and QoS framework can be reused for the traffic detection and QoS Flow mapping as following.

Editor’s Note: Additional multiplexing fields for other multiplexing traffic handling for RTP or other protocols are FFS.

- The Multiplexing Fields can be provisioned by the third-party AF together with flow description and the QoS requirements corresponding to the different Multiplexing Fields.

- The PCF generates the PCC rules with the multiplexing fields and corresponding QoS parameters based on the AF input and/or operator’s local configuration.

2.	The SMF generates and provides the corresponding QoS Rules, QoS Profiles and N4 rules to the UE, NG-RAN and the PSA UPF separately during the PDU Session Modification procedure.

3.	For Uplink traffic, the UE, based on the Service Data Flow template and Multiplexing Fields in the QoS Rules, maps the multiplexed data flows in the service data flow into the corresponding QoS Flow.

4.	For Downlink traffic, the UPF, based on the Service Data Flow template and Multiplexing Fields in the PDRs in N4 Rules, maps the multiplexed media flows in the service data flow into the corresponding QoS Flows.

6.X.3	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

PCF:

-	Receive the flow description, multiplexing fields and corresponding QoS requirements from AF and generate PCC rules accordingly.

UPF&UE:

-	traffic detection and QoS Flow mapping based on the service data flow templates and the multiplexing fields.

# * * * * End of changes * * * *