3GPP TSG-SA WG2 Meeting #160AHE	S2-2400826

# 22 - 29 January, 2024, Electronic										 (revision of S2-2312682)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0087 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ranging capability and subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-1-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The Capability of supporting Ranging/SL Positioning is not clear so far, as some of them indicate the role of the capability (e.g., the SL positioning server UE), while some of them just are using a general capability description, they are not common for understanding and have no a common granularity. Actually, for the Target UE and reference UE, it can be understood that the role determination is defined in a specific ranging procedure, it actually can be reflected by using the Ranging/SL Positioning operations over PC5. While for the Located UE, SL server UE, they have additional functionalities for the Ranging/SL Positioning besides the common capability for Ranging/SL Positioning operations over PC5. As such, it is proposed to additionally describe them. 2.For the UE subsrcription parts, they also have the similar issue to above. It is proposed to use a general Ranging/SL Positioning operations over PC5 to reflect the Target UE and reference UE role. While for the server UE, Located UE and client UE, they have additional operations that need to be specifed. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Correct the UE capability with following features: - - Ranging/SL Positioning operations over PC5 (i.e., for Target UE and SL Reference UE); - Located UE; - SL Positioning Server UE; - SL Positioning Client UE. 2. correct the UE subsrcription. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Capabilities/subscriptions of supporting Ranging/SL Positioning are not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1, 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 4.3.1	UE

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UE may support the following functions:

-	Reporting the following Ranging/SL Positioning Capabilities to 5GC over the N1 reference point:

-	Capability of supporting Ranging/SL Positioning over PC5 indicating that the UE supports:

-	Ranging/SL Positioning operations over PC5 (i.e., for Target UE and SL Reference UE);

-	Located UE;

-	SL Positioning Server UE;

-	SL Positioning Client UE.

-	Procedures for Ranging/SL Positioning over PC5.

-	Procedures to Network based SL Positioning, and Network assisted SL Positioning.

-	Procedures to UE-only SL Positioning.

-	Procedures to Ranging/SL Positioning service exposure.

-	Indicating UE Policy Provisioning Request in UE Policy Container for UE triggered Ranging/SL Positioning Policy provisioning, which requests one or multiple types of policies/parameters as listed below:

-	Policy/parameters for Ranging/SL Positioning operations over PC5 (i.e., for Target UE and SL Reference UE);

-	Policy/parameters for Located UE;

-	Policy/parameters for SL Positioning Client UE;

-	Policy/parameters for SL Positioning Server UE.

-	Receiving the Ranging/SL Positioning Policy from 5GC over N1 reference point.

-	Configuration of parameters for Ranging/SL Positioning over PC5. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over SR1 reference point from the Ranging/SL Positioning Application Server.

* * * * Start of 2ed Change * * * *

## 5.8	Subscription to Ranging/SL Positioning

The subscription information in the UDM contains information to give the user permission to use 5G Ranging/SL Positioning.

At any time, the operator can amend or remove the UE subscription rights from subscription information in the UDM, or to revoke the user's permission to use Ranging/SL Positioning.

The following subscription information is defined for Ranging/SL Positioning:

-	subscription for Ranging/SL Positioning operation over PC5.

-	subscription for UE acting as Located UE.

-	subscription for UE acting as SL positioning server UE.

-	subscription for UE acting as SL positioning client UE.

-	Ranging/SL positioning QoS parameters.

-	the list of the PLMNs authorized for Ranging/SL Positioning services, including:

-	the list of the PLMNs where the UE is authorised for Ranging/SL Positioning operation over PC5 reference point.

-	the list of the PLMNs where the UE is authorised for Ranging/SL positioning service with Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for Ranging/SL positioning service with Direct Discovery Model B, i.e. to perform Discoverer operation or Discoveree operation or both.

-	the list of the PLMNs where the UE is authorised to act as a Located UE.

-	the list of the PLMNs where the UE is authorised to act as a SL positioning server UE.

-	the list of the PLMNs where the UE is authorised to act as a SL positioning client UE.

.

* * * * End of Change * * * *