SA WG2 Meeting #160-Ad Hoc-e	S2-2400865

E-meeting, January 22 – 29, 2024

Source: 	IPLOOK

Title: 	KI #2, New Sol: Support of Store and Forward Satellite operation

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for KI#2

- Introduction

A new solution is proposed in this contribution for inclusion in TR 23.700-29. This solution outlines the specifics of the minimum necessary set of core network elements/network functions that should be placed on board and the procedure of S&F Satellite operation.

2	Proposal

It is proposed to capture the following solution in TR 23.700-29.

Start of Changes

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| x |  | x |  |  |

6.1	Solution #x: Support of Store and Forward Satellite operation

6.x.1	Description

This solution attempts to resolve Key Issue #2 about to support of store and forward satellite operation.

Core network on board contributes to reduce communication latency. However, in order to enhance the mobility management, this solution defines a lightweight core network on board that integrates AMF and UPF, with UPF responsible for user data traffic forwarding.

UE

RAN

UPF

DN

AMF

UDM

AF

PCF

SMF

User Equipment

Core Network functions on board

Core Network functions on the ground

Figure 6.X.1-1: The architecture of the minimum necessary set of core network elements/network functions on board

6.X.2	Procedures

Earth Station

UE

1.signalling/data exchanges

2.feeder link disconnection

3.trigger S&F Satellite operation

4.UL information storage/DL information forwarding

5.Restore connection of feeding link

6.DL information storage/UL information forwarding

Figure 6.X.2-1  The procedure of S&F Satellite operation

In the case of normal feeder link, signalling/data exchanges between UE and Earth Station;

The feeder link disconnection due to earth station handover or feeder link failure;

Trigger S&F Satellite operation when feeder link disconnection;

Core network elements/network functions on-board storage of UL information from UE or forwarding of stored DL information to the UE;

Feeder link recovery after completing the handover;

Core network elements/network functions on-board storage of DL information from the ground network or forwarding of stored UL information to the ground network.

There is no need to inform the UE when the S&F Satellite operation is applied because of S&F operations will not affect UE operations. In addition, not to inform UE may void unnecessary power consumption and save signalling overhead.

6.X.3	Impacts to Services, Entities and Interfaces

UPF:

The UPF provides the capacity of information exchange between two emergency users under the same satellite.

End of Changes