3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400951

# Online, Jan 22 – 29, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Integration with Ethernet LAN via enhancements of 5G VN group data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support 5G VN integration with Ethernet LAN network, there are still gaps that need to be investigated: - N6 traffic routing information provisioning and notification is not per 5G VN group basis. To address this, when AF requests to manage a specific 5G VN group, the AF needs to be able to provide the N6 traffic routing information per DNAI and subscription to UP path events as part of the 5G VN group data, this is used by the SMF to configure the N6 tunnels at UPF within the 5G VN in order to connect to the correct Ethernet LAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance 5G VN group management procedure to support provisioning of N6 traffic routing information per local LAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of 5G VN integration with Ethernet LAN network in a large scale |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

4.15.6.3b	5G VN group data

The 5G VN group data is described in Table 4.15.6.3b-1.

Table 4.15.6.3b-1: Description of 5G VN group data

| Parameters | Description |
| --- | --- |
| DNN | DNN for the 5G VN group. |
| S-NSSAI | S-NSSAI for the 5G VN group. |
| PDU Session Type | PDU Session Types allowed for 5G VN group. |
| Application descriptor | There may be multiple instances of this information; this information may be used to build URSP sent to 5G VN group members (NOTE 1). |
| Information related with secondary authentication / authorization | This may indicate: - the need for secondary authentication/authorization (as defined in clause 5.6 of TS 23.501 [2]); - the need for SMF to request the UE IP address from the DN-AAA Server. If at least one of secondary authentication/authorization or DN-AAA UE IP address allocation is needed, the AF may provide DN-AAA Server addressing information. |
| 5G VN group communication indication | Indicates that the 5G VN group is associated with 5G VN group communication. |
| Maximum Group Data Rate | This limits the total bit rate that is expected to be provided across all sessions of a 5G VN group. [optional]. |
| N6 traffic routing information and subscription to UP path events | The N6 traffic routing information is per DNAI and indicates any tunnelling that may be used over N6 that connects 5G VN and the LAN corresponding to the DNAI. The subscription to UP path events indicates the subscription to change of UPF for the 5G VN. |
| NOTE 1: As described in TS 23.503 [20], the PCF may be configured with a mapping from Application Descriptor to other information required to construct the URSP rules, e.g. IP filters and SSC mode. |  |

The information described in Table 4.15.6.3b-1 corresponds to 5G VN group data that an AF may provide together with External Group ID.

**** End of Changes ****