SA WG2 Meeting #160-Ad Hoc-e	S2-2400983

E-meeting, January 22 – 29, 2024

Source: 	Samsung

Title: 	23.700-77: KI#8, New Sol: Support of capability negotiation between UE and IMS network for Avatar communication

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This paper proposes solution for KI#8.

1 Introduction

This paper proposes new solution for KI#8 in TR 23.700-77, how to support capability negotiation between UE and IMS network for Avatar communications.

2 Proposal

It is proposed to approve following content in TR 23.700-77.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| X |  |  |  |  |  |  |  | X |

# * * * * Second change * * * *

6.X	Solution #X: Support of capability negotiation between UE and IMS network for Avatar communication

6.X.1	Description

To enable avatar communications in IMS network, the several capabilities of avatar media processing should be considered whether it is supported in the UE and the IMS network.

By referring the recent SA4 study on Avatar Reference Architecture (S4-232051: TR 26.813 v0.2.1), the capabilities of avatar media processing can be captured as data capture, base avatar generation, animation data generation, and avatar animation.

If the UE supports some part of capabilities, the rest of capabilities should be supported by the IMS network or the remote side (the remote IMS network, and/or the remote UE).

This solution proposes how to negotiate the capabilities of avatar media processing of the UE and the IMS network.

6.X.2	Procedures

The avatar media processing capability negotiation between UE and the IMS network is described in Figure 6.X.2.1.

Figure 6.X.2.1: Avatar media processing capability negotiation between UE and the IMS network

- The IMS session and the Bootstrap DC have been established between UE-A and UE-B.

- The UE-A and the originating IMS network negotiate with the UE capabilities of UE-A and the Network capabilities of the originating IMS network. The UE-A and the IMS network provide the supporting capabilities among data capture, base avatar generation, animation data generation, and avatar animation of each side and determine the total capabilities that can be supported by the originating side.

- The UE-A sends to UE-B the determined capabilities of avatar media processing that can be supported by the originating side.

- The UE-B and the terminating IMS network negotiate with the UE capabilities of UE-B and the Network capabilities of the terminating IMS network, in the same manner described in step 2.

- The UE-B sends to UE-A, the determined capabilities of avatar media processing that can be supported by the terminating side, and informs which capabilities are accepted by the terminating side.

- The UE-A and the originating IMS network setup the media resources based on the accepted capabilities from the terminating side.

- The UE-A responses to UE-B to complete the capability negotiation.

- The UE-B and the terminating IMS network setup the media resources based on the accepted capabilities from the originating side.

- The UE-B responses to UE-A to complete the capability negotiation.

- The Application Data Channel can be established for UE-A and UE-B.

6.X.3	Impacts on Existing Nodes and Functionality

This solution may have the following impacts to exiting entities and interfaces:

UE:

- UE provides and negotiates the capabilities of avatar media processing with the IMS network and the remote UE.

- UE sets up the media resources based on the negotiation result.

IMS network (IMS AS, DCSF, MF/MRF, DC AS):

- IMS network provides and negotiates the capabilities of avatar media processing with the UE.

- IMS network sets up the media resources based on the negotiation result.

# * * * * End of changes * * * *