3GPP TSG-WG SA2 Meeting #160AHE	S2-2401070

Emeeting, 22- 26 January, 2024	(revision of S2-240xxxx)

Source:	Lenovo

Title:	KI#1, Collection of Energy Consumption related information

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This paper proposes a new solution to KI#1 and KI#2 describing the collection of EC information.

# 1. Introduction/Discussion

Key Issue #1 includes the following aspect to be studied:

How and what network energy related information from the Network entities (i.e. RAN nodes, 5GC NFs) can be obtained in order to support network energy related information exposure.

Similarly, KI#2 includes the following aspect to be studied:

What network energy related information is required for subscription and policy control and how it is obtained”.

In both requirements the EC information may have the granularity per UE, or finer granularity like per PDU Session or QoS flow per UE.

While the EC information for a Network Slice or for a “service” can be determined by statistical models and collected from the OAM, the EC information on per UE level (or any finer granularity associated with the UE) strongly depends on the EC spent in the access network for this UE and can be individual for each UE. If one UE is located as cell edge for most of the time, the EC for this UE would be considerably higher than the EC for a UE located close to a base station. For the EC associated with a UE it is therefore beneficial to collect the EC data from the (R)AN serving the UE.

The proposed solution is based on the principle that the EC data for a UE (or any finer granularity associated with the UE) can be split in the EC consumed in the (R)AN serving the UE and the EC consumed in the CN. The EC is collected and calculated in a Control Plane NF called “EC collecting NF” (ECF). The ECF performs the following functionality:

- Collecting EC consumed in the (R)AN on per UE basis;

- Determining the EC consumed in the CN;

- Calculating the total EC for the UE (or any finer granularity associated with the UE like per PDU Session or QoS flow).

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-66.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 |  |
| 1 | X | X |  |  |
| 2 |  |  |  |  |
| … |  |  |  |  |

# * * * * Second change * * * *

## 6.x	Solution #X: Collecting of Energy Consumption information on per UE level

### 6.x.1	Key Issue mapping

This solution addresses the following requirements:

-	from Key Issue #1 the requirement: “How and what network energy related information from the Network entities (i.e. RAN nodes, 5GC NFs) can be obtained in order to support network energy related information exposure”; and

-	from Key Issue #2 the requirement “What network energy related information is required for subscription and policy control and how it is obtained”.

### 6.x.2	Functional Description

In order to determine the Energy Consumption (EC) information associated with a UE (or any finer granularity associated with the UE like per PDU Session or QoS flow), it is proposed to collect the EC information from the (R)AN entity serving the UE. It is expected that the (R)AN is able to measure EC information on per UE level granularity.

The present solution is based on the following principles:

-	the EC information may be collected in control plane NF called “EC collecting NF” (ECF), which may be a stand-alone NF collocated in an existing NF e.g. CHF, or NWDAF.

-	The EC information, which is collected specifically for a UE or for any finer granularity associated with the UE (e.g. per PDU Session or per QoS flow of the UE), can be split into:

-	Access Network (AN) part of the EC information: this information is measured in the (R)AN. The ECF collects the AN part of EC information via signalling to/from the (R)AN node and the ECF processes this information.

-	Core Network (CN) part of the EC information: this is the EC in the CN and backhaul network in order to serve the UE. The ECF collects and calculates the CN part of EC information based on information configured in the ECF by the OAM or obtained by the NWDAF.

-	The ECF calculates the EC information for the UE and/or for any sub-category of a UE granularity (e.g. per PDU Session or per QoS flow) by adding the corresponding AN part and the CN part of the EC information.

### 6.x.3	Procedures

### The Figure 6.X.3-1 describes the details about the

Figure 6.X.3-1:  Signalling flow for EC information collection on per UE level (or finer granularity)

The detailed description of the steps is provided as follows:

1.	The AF sends a request message to subscribe for notifications for EC information. The AF may use either services offered by the NEF, or can directly send the request to the ECF. The AF may include an External ID or external group ID which identifies the UE(s) for which EC information collection is required

The AF/NEF may send Subscribe Request message for event notifications including at least one of: event ID, event filter information (one or more SUPIs, [per UE, per PDU Session, per QoS flow]), event reporting information, notification target address (e.g. NEF or AF), event threshold.

NOTE:	The details for event ID, event filter information, event reporting information and notification target address are not in the scope of this solution and can be described in different solutions.

2.	The PCF sends a request message to the ECF to indicate that EC information collection is required for the UE or corresponding granularity of EC information of the UE (e.g. for a PDU Session or QoS flow). The PCF first selects an appropriate ECF. The PCF may provide to the ECF the UE’s SUPI and some of the following indications: the current serving AMF ID for the UE, a EC collection information type, a granularity of EC information and/or reporting conditions (e.g. how frequent, based on which reporting threshold, etc. the ECF needs to send reports to the PCF).

Any of the steps 1 or 2 are used as triggers in the ECF to determine that EC collection for a specific UE is required.

3.	If the ECF doesn’t know the serving AMF for the UE yet, the AMF resolves the serving AMF of the UE (e.g. by requesting the UDM). The ECF may also register with the UDM as a serving ECF for the UE. The ECF sends a request message to the (R)AN via the AMF to indicate that the EC information collection in the (R)AN is required. The ECF may send the AN EC collection request as a container transparently from the ECF to the (R)AN node.

If the UE is in CM-Connected state, the AMF forwards the request to the (R)AN node. If the UE is SM-Idle state, the AMF stores the request in the UE context and when the UE transfers to CM-Connected state, the AMF forwards the request to the (R)AN node.

4.	If the AN entity supports the EC collection and/or the identified AN EC assistance information, the AN entity stores the information in the UE context. If the AN entity does not support the EC collection and/or the identified AN EC assistance information, the AN entity sends a reply message to the AMF indicating an appropriate failure code.

The (R)AN starts to measure the transmission power and the internal processing power for the DL data, the UL data and/or signalling message to/from the UE. The (R)AN calculate the AN part of the EC information based in the gathered information (e.g. transmission and reception power) and based on local statistical algorithm for EC calculation.

5.	When one or more of the reporting conditions for the EC report are fulfilled, or when the UE transfers from Connected state (e.g. RRC-Connected) to Idle state (e.g. RRC-Idle), the (R)AN sends a report message to the ECF containing the AN EC limitation and transmit it to the ECF.

The (R)AN EC report can contain the following information: AN EC value and optionally the measurement unit (e.g. 20 Watt per data volume., or 20 Watt for the last 60 minutes), the granularity (e.g. per PDU Session ID, or for a 5QI; or for non-GBR or GBR traffic, or per access type).

6.	The ECF may collect input information from the NWDAF or from the OAM system. The input information is used in the ECF to calculate the CN part of the EC information.

For example, the input information can be: the average EC per data unit used in the backhaul and CN (e.g. 2 Watt per Mbyte), or a factor by which the AN part of EC info can be multiplied to calculate the CN part of EC info.

7.	The ECF receives the AN part of the EC data and the associated assistance information. The ECF determines the CN part of the EC information and calculates the total EC data value for the UE or for the required granularity of EC information. If the notifications from the ECF are requested (as per step 1 or 2) for a reporting threshold, the ECF determines whether the reporting threshold is reached.

8.	The ECF sends a notification to the NF which requested the collection of EC information, e.g. NEF/AF or PCF.

### 6.x.4	Impacts on existing services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# * * * * End of changes * * * *