3GPP TSG SA2#160AH	S2-2401099

Emeeting, 22- 26 Jan, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Slice mapping information notification from NSSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TBD |  |  |  |  |  |  |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The network slice mapping information between VPLMN and HPLMN is only provided from NSSF to AMF in the Nnssf_NSSelection_Get service operation. When the mapping information is updated in the NSSF(e.g. the VPLMN network slice is not available), it is not possible to notify the AMF in time about the change. A mechanism to support slice mapping information notification from NSSF needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support slice mapping information notification from NSSF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The slice mapping information in AMF may not be up-to-date |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16.3.3,5.2.16.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2.16.3.3	Nnssf_NSSAIAvailability_Notify service operation

Service operation name: Nnssf_NSSAIAvailability_Notify

Description: This service operation enables the NSSF to update a NF Service Consumer (e.g. AMF) with any S-NSSAIs restricted per TA and if needed, subsequently lift any restriction per TA. This service operation also enables the NSSF to notify to the NF Service Consumer (e.g. AMF or NSSF in the VPLMN) that an S-NSSAI has to be replaced by an Alternative S-NSSAI or a Network Slice instance should be replaced. This service operation also enables the NSSF to notify to the NF Service Consumer (e.g. AMF or NSSF in the VPLMN) that the mapping between the HPLMN S-NSSAI and VPLMN S-NSSAI is updated.

Inputs, Required: Subscription Correlation ID.

Inputs, Conditional Required:

If this service operation is invoked by subscription for changes in the status of the NSSAI availability information, then the following inputs are required:

-	a list of TAIs and the S-NSSAIs for which the status is changed (restricted/unrestricted) per each TAI.

If this service operation is invoked by subscription for event notification for Network Slice Replacement, then the following inputs are required:

-	For replacement of an S-NSSAI of the serving PLMN: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAI to be replaced.

-	For replacement of an HPLMN S-NSSAI: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAIs of the HPLMN, PLMN ID.

-	For termination of the Network Slice replacement:

1)	indication to stop the Network Slice replacement for new UEs; or

2)	indication to terminate the Network Slice replacement and move back the UEs and PDU Sessions from the Alternative S-NSSAI to the S-NSSAI.

If this service operation is invoked by subscription for event notification for Network Slice Instance Replacement, then the following inputs are required:

-	The S-NSSAI(s) and NSI ID(s) for which the status is changed (e.g. congested or no longer available).

If this service operation is invoked by subscription for event notification for change in mapping between the HPLMN S-NSSAI and VPLMN S-NSSAI, then the following inputs are required:

-	The mapping between HPLMN S-NSSAI and the V-PLMN S-NSSAI.

Inputs, Optional:

-	NSI IDs in the S-NSSAI for which the status is changed (restricted/unrestricted) per each TAI.

If this service operation is invoked by subscription for event notification for Network Slice Replacement or Network Slice Instance Replacement, then the following inputs are optional:

-	For replacement due to congestion: congestion mitigation information.

Outputs, Required: None.

Outputs, Optional: None.

5.2.16.3.4	Nnssf_NSSAIAvailability_Subscribe service operation

Service operation name: Nnssf_NSSAIAvailability_Subscribe

Description: This service operation enables a NF Service Consumer (e.g. AMF) to subscribe to a notification of any changes in status of the NSSAI availability information (e.g. S-NSSAIs and optionally NSI IDs available per TA and the restricted S-NSSAI(s) and optionally NSI IDs per PLMN in that TA in the serving PLMN of the UE) upon this is updated by another AMF. This service also enables the NF Service Consumer (e.g. AMF or NSSF in the VPLMN) to subscribe to a notification of Network Slice Replacement and Network Slice Instance Replacement.

Inputs, Required: Callback URI of the NF Service Consumer:

Inputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following inputs are required:

-	list of TAIs supported by the NF service consumer, event to be subscribed.

If this service operation is invoked to subscribe to a notification for Network Slice Replacement, then the following inputs are required:

-	for the S-NSSAI of the serving PLMN: set of S-NSSAIs served by the NF Service Consumer that may be replaced, NF type of the NF Service Consumer (e.g. AMF), Requester ID, event to be subscribed.

-	for HPLMN S-NSSAI: S-NSSAIs for the HPLMN, NF type of the NF Service Consumer (e.g. AMF or NSSF), Requester ID and PLMN ID, event to be subscribed.

If this service operation is invoked to subscribe to a notification when the Network Slice instance is congested or no longer available, then the following inputs are required:

-	The S-NSSAIs, NSI IDs, event to be subscribed.

If this service operation is invoked by subscription for event notification for change of mapping between the HPLMN S-NSSAI and VPLMN S-NSSAI, then the following inputs are required:

-	The V-PLMN S-NSSAIs, event to be subscribed.

Inputs, Optional: Expiry time.

Outputs, Required: Subscription Correlation ID.

Outputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are required:

-	Expiry time (if present in the request, may be included in the response based on operator's policy and taking into account the expiry time present in the request (i.e. should be less than or equal to that value); if not present in the request, may be included in the response based on operator's policy. Whatever the case, if not included in the response, this means that the subscription is valid without an expiry time).

Outputs, Optional:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are optional:

-	A list of TAIs and for each TAI, the S-NSSAIs and optionally NSI IDs supported by the AMF and 5G-AN and authorized by the NSSF for the TAI and a list of S-NSSAIs and optionally NSI IDs restricted per PLMN for the TAI. The NSSS indicates to the AMF that the S-NSSAI(s) are not available in the corresponding TAIs.

If this service operation is invoked to subscribe event notification for change in mapping between the HPLMN S-NSSAI and VPLMN S-NSSAI, then the following outputs are optional::

-	The mapping between HPLMN S-NSSAI and the V-PLMN S-NSSAI.

# * * * * End of change * * * *