SA WG2 Meeting #160-Ad Hoc-e	S2-2401139

# E-meeting, January 22 – 29, 2024			         (revision of S2-234xxxx)

Source: 	Nokia, Nokia Shanghai Bell, Ericsson

Title: 	FS_MASSS_Key Issue for policy impacts of WT#1.3

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes a KI for WT#1.3 of FS_MASSS for policy control impacts.

# 1	Discussion

FS_MASSS SID objectives have 2 sets of work tasks. WT#1 is for supporting DualSteer device. WT#1 has been further divided into sub-tasks as below:

WT#1.1: Study whether and how to enhance subscription aspects for each of the above scenarios.

WT#1.2: Study whether there are any impacts to the registration procedure for DualSteer Devices.

WT#1.3: Session management enhancements and policies:

-	Study whether and how to enhance network policies provided by the HPLMN to the DualSteer Device and within the network to support DualSteer Devices.

-	Study whether and how to enhance session management procedures for initial steering and potential subsequent switching. Traffic policies are in full control of the HPLMN. For sessions subject to potential switching, assuming data anchoring in the HPLMN or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN, study how to select the PSA UPF(s) in the HPLMN to allow routing the traffic across 3GPP accesses towards the same PSA UPF during the switching.

NOTE 9:	Impact to existing functionality related to the change of a service-related data between a 3GPP access network and a non-3GPP access network will be considered as part of WT-D-1.4.

This CR focuses policy aspects of WT#1.3 above.

WT#1.3 has two aspects, i.e. session management and policies. The enhancements needed in the policies provided by the HPLMN to a DualSteer device and the network to support the DualSteer device, is in itself a quite big study topic and deserves an independent investigation, evaluation and conclusion. Therefor it would be appropriate to have an independent key issue for the enhancements needed in network provided policies.

Observation 1: It is appropriate to study the policies provided by the HPLMN to the DualSteer device and the Network in an independent Key Issue.

As per the SA1 requirements, a DualSteer device has two subscriptions/SUPIs, sharing one subscription profile from the same operator; The two subscriptions/SUPIs of the DualSteer device are registered to two 3GPP access networks simultaneously and the DualSteer device shall perform steering or switching of application traffic across those two access connections from the two SUPIs/UEs. Which means that there is a steering and/or switching layer above the UEs that manages the traffic between the two SUPIs/UEs and corresponding steering/switching policies shall be provided to this DualSteer layer.

Observation 2: DualSteer device is a new type of device with a Steering and Switching layer above the UE. Steering and Switching policies have to be provided to the DualSteer device.

From the network pov, the network shall be to identify the two SUPIs/UEs that are associated with a DualSteer device, identify and correlate the PDU sessions from those two SUPIs/UEs for performing Steering and Switching of traffic between the two SUPIs/UEs. The network (e.g. UPF) shall be provided with rules to identify the two access paths of the DualSteer device and Steer and/or Switch traffic between them.

Observation 3: The network (e.g. UPF) shall be provided with rules to identify the two access paths of the DualSteer device and Steer and/or Switch traffic between them.

Based on the above observations, it is proposed to agree on a Key Issue to study the policies provided by the HPLMN to the DualSteer device and the Network.

# 2 Proposal

It is proposed to update TR 23.700-54 v0.0.0 on FS_MASSS as follows.

* * * * First Change * * * *

### 5.1.X	Key Issue #X: Providing policies/rules for DualSteer devices and UPF

#### 5.1.x.1 	Description

This Key Issue will study policy control impacts to support steering and/or switching of application traffic between the two PDU Sessions of the two SUPIs/UEs associated with a DualSteer device, including the following:

-	What policies/rules are used for controlling steering and switching, e.g. whether they are based on ATSSS rules, MAR rules, PCC rules or not.

-	Whether and what enhancements are needed in UE Route Selection Policies (URSP) to allow the UE of a DualSteer device to establish a PDU Session for DualSteer.

-	Whether and what enhancements are required in the policy subscription data to support generation of rules/policies for DualSteer.

* * * * End of Changes * * * *