SA WG2 Meeting #160AH	S2-2401242

# E-meeting, January 22 – 29, 2024      		       		          (revision of S2-240)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New WID on immediate EIR reporting of PEI status change

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	WID on immediate EIR reporting of PEI status change

Acronym:	TEI19_IMMEIR_REPORT

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

In the 5GC the Equipment Identity Register is defined

- If a ME is reported as stolen, a record for this ME may be added to the EIR that marks this device as restricted

- The records in the EIR may be checked by the AMF each time a UE to register to the network. If the EIR record exists for the device and the status is set to restricted, the AMF may reject the registration request.

When a device is to be marked as restricted, the device’s record can be added to, or updated in, the EIR, and any subsequent attempts to register by that device can be blocked, assuming the AMF checks the EIR. There may be a time lag between the device being marked as restricted and the device attempting to register again. During this period, the device is still able to obtain network services. Ideally the device would be deregistered as soon as the record is updated to be restricted in EIR.

This TEI19 aims at solving this issue by EIR triggering deregistration at the AMF as soon as the device’s record is set to restricted.

# 4	Objective

Following aspects will be specified:

- Define a new service operation for the EIR service, allowing AMF to receive notifications when an  equipment identity (PEI) record in the EIR has changed of status..

- when the EIR is updated with a record that marks a device (PEI) as blocked, or changes the status of a device in any other way, the EIR can immediately send a notification directly to the AMF that has subscribed to notifications on that device (PEI).

- AMF behaviour is extended to handle notifications of equipment status change and apply operator policies such as immediately de-registering the corresponding UE.

This work considers only 5GC.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 0 | .75 |  |  |

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| None |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.502 | immediate EIR reporting of PEI status change | SA#104 June 2024 |  |

# 6	Work item Rapporteur(s)

, Nokia, at nokia dot com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

none.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell