SA WG2 Meeting #160-AH-e	S2-2401329

January 22 - 29, 2024, Electronic Meeting

Source:	Apple

Title:	FS_MASSS DualSteer Key Issue for Session management aspects

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 for DualSteer for the FS_MASSS TR 23.700-54.

# 1	Discussion

This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 of the FS_MASSS SID (SP-231802).

# 2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes * * * *

## 5.1	Key Issue for DualSteer

### 5.1.x	Key Issue #X: DualSteer – Session management aspects

#### 5.1.x.1 	Description

This key issue will study whether and how to enhance session management procedures for initial steering and potential subsequent switching:

- 	for the single UE case, how to support traffic steering and/or switching with non-simultaneous transmission of a DualSteer device’s user data for different services across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN) or two different PLMNs, assuming data anchoring in the HPLMN or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN;

- 	for the case of two separate UEs, how to support traffic steering and/or switching with simultaneous transmission of a DualSteer device’s user data for different services across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN) or two different PLMNs, assuming data anchoring in the HPLMN, or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN;

- 	for sessions subject to potential switching, how to select the PSA UPF(s) in the HPLMN (or in the PNI-NPN) to allow routing the traffic across 3GPP accesses towards the same PSA UPF during the switching; and

- 	whether existing functionality is impacted related to the change of a service-related data between a 3GPP access network and a non-3GPP access network.

* * * * End of Changes * * * *