3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401337

Electronic Meeting, 22-29 January 2024

# Title:	[Draft] Reply LS on RAN feedback during handover

# Response to:	RAN feedback during handover (R3-238038/S2-2400064)

# Release:	Release 18

# Work Item:	TRS_URLLC-NR-Core

Source:	SA2

To:	RAN3

Cc:	-

Contact Person:

Name:	Sebastian Speicher

E-mail Address:	speicher AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.502 CR#4737, CR#4738, CR#4739

1. Overall Description:

SA2 would like to thank RAN3 for the LS on RAN feedback during handover. With respect to reactive RAN feedback, SA2 would like to provide the following feedback:

From SA2's perspective, the CR in R3-238135 agreed by RAN3 does not solve all scenarios. The case that a UE is handed over via Xn from an NG-RAN node that does not support reactive RAN feedback to an NG-RAN node that supports reactive feedback is not supported. What is missing in SA2's view is that the AMF needs to provide the Capability for BAT Adaptation to the target NG-RAN node in the N2 Path Switch Request Ack (together with other N2 SM information).

SA2 understand that RAN3 agreed to include the Capability for BAT Adaptation in the TSC Assistance Information IE according to the CR in R3-238135.

As part of this discussion, SA2 discovered that also for other TSC Assistance Information such as periodicity (from Rel-16) and Survival Time (from Rel-17), etc., there is no support for scenarios that a UE is handed over via Xn from an NG-RAN node that does not support TSC Assistance Information to an NG-RAN node that supports TSC Assistance Information. The target NG-RAN will not receive the TSC Assistance Information because the TSC Assistance Information is not necessarily provided to the target NG-RAN node in the N2 Path Switch Request Ack.

SA2 would like to point out to RAN3 that the SMF may already provide TSC Assistance Information to NG-RAN as part of an Xn handover according to TS 23.502 v16.18.0 clause 4.9.1.2.2 step 6. It appears that this

has not been reflected in RAN3 specifications.

SA2 agreed correction CRs from Rel-16 to clarify that the SMF shall provide TSC Assistance Information to NG-RAN in the N2 Path Switch Request Ack as part of Xn handovers if TSC Assistance Information is available for a QoS flow at the SMF.

2. Actions:

To RAN3:

ACTION: 	SA2 kindly asks RAN3 to take the above into account and to apply the related changes to RAN3 specifications.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #161		26 February-1 March 2024	Athens, Greece

TSG-SA2 Meeting #162		15-19 April 2024	China (TBD)